c     =======================================================

c     Program for accomplishing by means of The Boundary

c     Element Method (BEM) three-dimensional (3D) stationary

c     analyses of problems governed by the Poisson's equation

c     and time domain analyses of linear acoustic problems

c     =======================================================

c     version 1.0

c     date: 30 December 98

c     developed by:

c     ==================================

c     Dr.-Ing. Francisco Célio de Araújo

c     Department of Civil Engineering

c     Federal University Ouro Preto

c     35400-000 Ouro Preto-MG-Brazil

c     tel: +55 31 559 1547

c     fax: +55 31 551 1988

c     e-mail: fcelio@em.ufop.br

c     ==================================

c     Explanations concerning the numerical BEM model used in this computer

c     code can be found in the chapter  "Time-Domain Three-Dimensional Analysis" of

c     the book "Boundary Element Acoustics - Fundamentals and Compute Codes"

c     edited by T. Wu, CMP, Southampton, (1999), written by  F. C. Araújo (fcelio@em.ufop.br),

c     W. J. Mansur (webe@coc.ufrj.br) and  J. A. M. Carrer (carrer@coc.ufrj.br).

c     This computer code is a modulus of the computer system BEWPA (Boundary

c     Elements for Time-domain Wave Propagation Analysis), which can model:

c     - 2D and 3D scalar wave propagation for homogeneous and non-homogeneous

c       media;

c     - 2D and 3D elastic wave propagation for homogeneous and non-homogeneous

c       media.

c     BEWPA also contains a FEM modulus which permits users to use

c     coupled BE/FE models

implicit double precision(a-h,o-z)

parameter(maxpoi=50000,maxpo1=3*1024*1024)

common/work1/  feld1(maxpo1)

common/worki/  ifeld(maxpoi)

common/rw/     ire,iwr

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/temp/   delt,niter

common/cicpt/  icorr,icpt,nmsdi,nmnos

character*4    title(20)

dimension      npg(15),ndiin(5)

c     definition of the input channel(ire) and output channel(iwr)

ire=5

iwr=6

open(ire,file= 'input.dat', status=    'old')

open(iwr,file='output.dat', status='unknown')

write(iwr,5)

5 format(5x,

*    '===================================================',

*/5x,'Modulus of BEWPA Program for accomplishing',

*/5x,'3D time domain analyses of linear acoustic problems',

*/5x,'with The Boundary Element Method;'

*/5x,'version 1.0, 1998',

*/5x,'=================================================')

c     input of analysis type (nupro)

c     [nupro=1 --> stationary analysis,nupro=2 --> transient analysis]

read(ire,*)nupro

if(nupro.eq.1)then

write(iwr,*)'    stationary analysis is being considered'

endif

if(nupro.eq.2)then

write(iwr,*)'    transient analysis is being considered'

endif

c     input of the analysis title

read(ire,10)title

10 format(20a4)

write(iwr,15)title

15 format(/5x,'title :',1x,20a4)

c     input/output of control data for BEM analyses

read(ire,*)infb,nno,ne,npoi,nnoel,npropb,ndime,

*           nnopl,ivols

if(infb.eq.0)write(iwr,*)'    finite domain problem'

if(infb.eq.1)write(iwr,*)'    infinite domain problem'

if(infb.eq.2)write(iwr,*)'    half-infinite domain problem'

write(iwr,20)infb,nno,ne,npoi,nnoel,npropb,ndime,

*            nnopl,ivols

20 format(/5x,'infb =',i6,5x,'nno =',i6,5x,'ne =',i6,5x,'npoi =',i6,

*        5x,'nnoel =',i6,/5x,

*        5x,'npropb =',i4,5x,'ndime =',i5,5x,'nnopl =',i5,

*       /5x,'ivols =',i5)

if(infb.ne.2)then

nnoe=0

nee=0

endif

if (infb.eq.2) then

read (ire,*) nnoe,nee

write (iwr,*) nnoe,nee

endif

c     input of volume sources data

if(ivols .ne. 1)nspo=0

if(ivols .eq. 1)then

c      concentrated sources are to be considered

c      nspo=number of points with concentrated sources

read(ire,*)nspo

endif

c     input of:

c     - integration dimension (if one- ,bi- or three-dimensional) (ndiin);

c     - number of integration points per integration direction (npg);

c     - order of the subelement mesh (nordm)

read(ire,*)ndiin(5),(npg(i),i=13,13+ndiin(5)-1),nordm

write(iwr,30)ndiin(5),(npg(i),i=13,13+ndiin(5)-1),nordm

30 format(/5x,'ndiin(5) =',i5,5x,'npg(1) =',i5,5x,'npg(2) =',i5,

*        5x,'nordm  =',i5)

c     number of integration points per subelement (npgse)

npgse=1

do idiin=13,13+ndiin(5)-1

npgse = npgse*npg(idiin)

enddo

c     number of subelements(nse)

nse=nordm*nordm

nnose=4

deltal=2./dfloat(nordm)

djacse=(deltal**2)/4.

write(iwr,40)nnose,nse,deltal,djacse

40 format(5x,'nnose =',i6,5x,

*       'nse =',i6,5x,'deltal =',d15.8,5x,'djacse =',d15.8)

c     variable for determining the integration process to be applied

c     icpt=1 ==> triangular polar co-ordinate transformation

c     icpt=0 ==> standard gauss quadrature

read(ire,*)icpt

write(iwr,45) icpt

45 format (5x,'icpt = ',i5)

if(icpt.eq.1)then

nmsdi=3

nmnos=9

if(nupro.eq.2)then

icorr=5

write(iwr,50)icorr

50     format(/5x,'integration on singular elements by using',

*           /5x,'triang. polar co-ord. for i < or =  ',2x,i5)

endif

else

nmsdi=1

nmnos=1

write(iwr,60)

60   format(/5x,'num. integ. by means of standard gauss',

*             'quadrature')

endif

c     allocation of the variables in the integer work vector "ifeld"

ijdiag=1

iincse= ijdiag + nno

iinc= iincse + nse*nnose

if (infb.ne.2) net= ne

if (infb.eq.2) net= ne+nee

ilelno= iinc + net*nnoel

iiencl= ilelno + ne*nnoel

litvar= iiencl + net

lnmult= litvar + nno

limult= lnmult + nno+nnoe

lifip = limult + (nno+nnoe)*5

indnpl= lifip + nno

iitvarb = indnpl + nnopl

if (nupro.eq.1)ibspi = iitvarb

if (nupro.eq.2)ibspi = iitvarb + nspo

if(ibspi .gt. maxpoi)then

write(iwr,*)'there is not enough space in the array "ifeld"'

write(iwr,*)'please set "maxpoi" greater or equal',ibspi

stop

else

ibspib=ibspi*2

write(iwr,70)ibspi,ibspib

70   format(/5x,'minimum necessary ram memory for "ifeld":',i10,

*          1x,'(=',i10,1x,'bytes)')

maxbyi=maxpoi*2

write(iwr,80)maxpoi,maxbyi

80   format(5x,'available ram memory for "ifeld":',i10,

*         1x,'(=',i10,1x,'bytes)')

endif

c     allocation of real variables in the work vector "feld1":

ihsg=1

if(nupro.eq.1)ib=1

if(nupro.eq.2)ib=ihsg+nno*nno

ipinf=ib + nno*nno

ipsup =ipinf + (nno*(nno+1))/2

icrnoe = ipsup+(nno*(nno+1))/2

icsnoe = icrnoe + nnoel

ishapse = icsnoe + nnoel

icoorse = ishapse + nnose*npgse

ieta1 = icoorse + (nordm+1)*(nordm+1)*ndiin(5)

ieta2 = ieta1 + nse*npgse

izeta1 = ieta2 + nse*npgse

izeta2 = izeta1 + nse*npgse

irpg = izeta2 + nse*npgse

iwrpg  =irpg  + npg(13)

ispg   =iwrpg + npg(13)

iwspg  =ispg  + npg(14)

itpg  =iwspg  + npg(14)

iwtpg  =itpg  + npg(15)

ircoor=iwtpg  + npg(15)

iscoor=ircoor + nse*npgse*nmsdi*nmnos

ishape=iscoor + nse*npgse*nmsdi*nmnos

ideriv=ishape + nse*npgse*nmsdi*nnoel*nmnos

icoord=ideriv + nse*npgse*nmsdi*2*nnoel*nmnos

if (infb.ne.2) lp= icoord + nno*ndime

if (infb.eq.2) lp= icoord + (nno + nnoe)*ndime

lp0=lp + nno

lomeg=lp0 + nno

ltemp0= lomeg + nno

lencl = ltemp0 + nno

iunode = lencl + nno

ipnode = iunode + nno

icoori= ipnode + nno

isp = icoori + npoi*ndime

icoorp = isp + npoi

ivs    = icoorp + nspo*ndime

ibsp1  = ivs + nspo

if (nupro.eq.1)then

ihsl = ibsp1

igsl = ihsl + nnoel

ihip = igsl + nnoel

igip = ihip + npoi*nno

ibsp1 = igip + npoi*nno

endif

if(nupro.eq.2)then

ifcs    = ibsp1

itp0b   = ifcs +  nspo

iomeb   = itp0b + nspo

iunode1 = iomeb + nspo

ifc     = iunode1 + nno

ihsl    = ifc + nno

igsl    = ihsl + nnoel

ih1l    = igsl + nnoel

ih2l    = ih1l + nnoel

ig1l    = ih2l + nnoel

ig2l    = ig1l + nnoel

ih2g    = ig2l + nnoel

ig2g    = ih2g + nno

ibsp1   = ig2g + nno

endif

c     initializing work vector

do i=1,ibsp1

feld1(i)=0.d+00

enddo

c     input of problem data

call inrem(feld1(icoord),feld1(icoori),feld1(ivs),feld1(icoorp),

*           ifeld(iinc),ifeld(ilelno),ifeld(iiencl),

*           feld1(lp),feld1(lomeg),feld1(ltemp0),

*           ifeld(litvar),ifeld(lnmult),ifeld(limult),ifeld(lifip),

*           nupro,ifeld(indnpl),

*           ifeld(iitvarb),feld1(itp0b),feld1(iomeb))

if(nupro.eq.1)then

c       ===================

c       stationary analyses

c       ===================

c       decision if the available random access memory is enough

iramsp=ibsp1

if(iramsp .gt. maxpo1)then

write(iwr,*)'there is not enough available space in "feld1"'

write(iwr,*)'please set "maxpo1" greater or equal ',iramsp

stop

else

iramspb=iramsp*8

write(iwr,90)iramsp,iramspb

90    format(/5x,'minimum used ram memory for "feld1":',i10,

*          1x,'(=',i10,1x,'bytes)')

maxby1=maxpo1*8

write(iwr,100)maxpo1,maxby1

100    format(5x,'available ram memory for "feld1"    :',i10,

*          1x,'(=',i10,1x,'bytes)')

endif

c       preparation of the integration domain

call domint(feld1(ircoor),feld1(iscoor),feld1(ishape),

*              feld1(ideriv),

*              feld1(irpg),feld1(ispg),feld1(itpg),

*              feld1(iwrpg),feld1(iwspg),feld1(iwtpg),

*              ndiin(5),npg(13),npg(14),npg(15),

*              feld1(icrnoe),feld1(icsnoe),feld1(ishapse),

*              feld1(icoorse),feld1(ieta1),feld1(ieta2),

*              feld1(izeta1),feld1(izeta2),

*              ifeld(iincse))

c       assemblage of the global H- and G-matrices for stationary problems

call matrxs(feld1(ib),feld1(ipinf),feld1(ipsup),

*              feld1(ihsl),feld1(igsl),

*              feld1(ishape),feld1(ideriv),ifeld(lnmult),

*              ifeld(limult),ifeld(ijdiag),npg(13),

*              feld1(icoord),ifeld(iinc),ifeld(ilelno),

*              ifeld(iiencl),feld1(lencl),

*              feld1(iwrpg),feld1(iwspg),

*              feld1(ieta1),ndiin(5))

c       introduction of the boundary conditions (interchange of columns)

c       and LU-decomposition of the A-matrix

call fators(feld1(ib),feld1(ipinf),feld1(ipsup),

*              ifeld(lnmult),ifeld(limult),ifeld(lifip),

*              ifeld(ijdiag),feld1(lencl))

c       evaluation of the contribution of volume sources

if(ivols .eq. 1)then

call vsourc(feld1(icoord),feld1(ivs),feld1(icoorp),

*               feld1(iunode),ifeld(lnmult),ifeld(limult))

endif

c       determination of the solution vector

call vectos(feld1(ib),feld1(ipinf),feld1(ipsup),

*              feld1(lp),ifeld(lifip),ifeld(ijdiag),

*              feld1(iunode),feld1(ipnode))

c       determination of potentials at internal points

if(npoi .ne. 0) then

call innerps(feld1(ihip),feld1(igip),

*                feld1(ihsl),feld1(igsl),

*                feld1(iunode),feld1(ipnode),feld1(isp),

*                feld1(ivs),feld1(icoori),feld1(icoorp),

*                feld1(ishape),feld1(ideriv),

*                npg(13),feld1(icoord),ifeld(iinc),ifeld(ilelno),

*                ifeld(iiencl),

*                feld1(iwrpg),feld1(iwspg),feld1(ieta1),ndiin(5))

endif

c       data output

call outputs(feld1(iunode),feld1(ipnode),feld1(isp))

c     end of the stationary analysis

endif

if(nupro.eq.2)then

c      ==================

c      transient analyses

c      ==================

nmaxl=max0(nno,npoi)

if(ivols.eq.1)then

c       determination of the limit time-step for truncating

c       the contribution of internal sources/forces

c       in the time-marching process

call dmaxd(dmaxbs,feld1(icoord),feld1(icoorp),nno,nspo)

call dmaxd(dmaxis,feld1(icoori),feld1(icoorp),nno,nspo)

dmax=dmax1(dmaxbs,dmaxis)

itli1m=idint(dmax/(prop(1)*delt)) + 2

c       number of time steps in which matrices GBG for considering

c       volume sources/forces must be calculated(nste5)

nste5=min0(niter,itli1m)

c       allocation of the matrix for considering volume sources

igbg=ibsp1

ibsp1=igbg + nmaxl*nspo*nste5

c       initializing work vector "feld1"

do i=igbg,ibsp1-1

feld1(i)=0.d+00

enddo

endif

c      decision if the available ram memory is enough

iramsp=ibsp1+nmaxl*nno*4

if(iramsp .gt. maxpo1)then

write(iwr,*)'there is not enough available space in "feld1"'

write(iwr,*)'please set "maxpo1" greater or equal ',iramsp

stop

endif

c      use of the free ram memory (free space in work vector "feld1")

ifsp=maxpo1-ibsp1

c      number of time steps possible to be calculated in main memory

nste1=ifsp/(nno*nno*2)

c      determination of the limit time step for truncating

c      the time-marching process

call dmaxe(dmax,feld1(icoord),nno)

itlim=idint(dmax/(prop(1)*delt)) + 2

c      allocation of the H- and G-matrices in the work vector "feld1"

nste3=min0(nste1,niter,itlim)

ih1g=ibsp1

ig1g=ih1g + nno*nno*nste3

write(iwr,*)'nste1 =',nste1,'  nste3 =',nste3,'  itlim =',itlim

c      really used ram memory at determination of the

c      boundary values (irbsp, bhsp)

irbsp=ibsp1 + nno*nno*nste3*2

bhsp=dble(irbsp*8)*1.d-06

write(iwr,110)bhsp

110  format(/5x,'used random access memory at determination of',

*         1x,'the bound. values:',

*        /5x,'(space for "feld1") ==> ',d12.5,1x,'Mbytes')

c      initializing work vector "feld1"

do i=ibsp1,irbsp

feld1(i)=0.d+00

enddo

c      variables for controlling the use of random access and disk

c      memory at the determination of the vector on the r.h.s. of

c      the system of equations

v1=dfloat(itlim)/dfloat(nste3)

v2=dint(v1)

v3=v1-v2

if(v3 .eq. 0.)then

nblim=idint(v1)

else

nblim=idint(v2)+1

endif

itblim=itlim-(nblim-1)*nste3

nmplim=(nblim-2)*nste3+itblim

c      number of time steps in which matrices must be calculated(nste4)

nste4=min0(niter,itlim)

c      determination of the necessary memory on disk(bhisp)

if(nste4 .gt. nste1)then

nste2=nste4-nste1

bhisp=dble(nno*nno*nste2*2*8)*1.d-6

write(iwr,115)bhisp

115    format(5x,'necessary memory on the disk ',

*          '==> ',d12.5,1x,'Mbytes',/)

c        opening direct access files for storing BE matrices

ipar1=nno*8

open(unit=1,file='h1g.bin',

*        form='unformatted',access='direct',recl=ipar1)

open(unit=3,file='g1g.bin',

*        form='unformatted',access='direct',recl=ipar1)

endif

ipar5=nno*8

open(unit=8,status='scratch',form='unformatted',

*           access='direct',recl=ipar5)

open(unit=9,status='scratch',form='unformatted',

*           access='direct',recl=ipar5)

c       preparation of the integration domain

call domint(feld1(ircoor),feld1(iscoor),feld1(ishape),

*              feld1(ideriv),

*              feld1(irpg),feld1(ispg),feld1(itpg),

*              feld1(iwrpg),feld1(iwspg),feld1(iwtpg),

*              ndiin(5),npg(13),npg(14),npg(15),

*              feld1(icrnoe),feld1(icsnoe),feld1(ishapse),

*              feld1(icoorse),feld1(ieta1),feld1(ieta2),

*              feld1(izeta1),feld1(izeta2),

*              ifeld(iincse))

c      time-marching process for determining the boundary values

do iter=1,niter

if(iter .eq. 1) then

call matrx0(feld1(ihsg),feld1(ihsl),

*             feld1(ib),feld1(ipinf),feld1(ipsup),

*             feld1(ishape),feld1(ideriv),feld1(ih1g),feld1(ig1g),

*             feld1(ih1l),feld1(ih2l),feld1(ig1l),feld1(ig2l),

*             ifeld(lnmult),ifeld(limult),

*             ifeld(ijdiag),nste3,npg(13),iter,

*             feld1(icoord),ifeld(iinc),ifeld(ilelno),

*             ifeld(iiencl),feld1(lencl),

*             feld1(iwrpg),feld1(iwspg),feld1(ieta1),ndiin(5))

call fatora(feld1(ihsg),feld1(ib),feld1(ipinf),feld1(ipsup),

*              ifeld(lnmult),ifeld(limult),ifeld(lifip),

*              ifeld(ijdiag),feld1(lencl))

endif

if((iter .ge. 2) .and. (iter.le.itlim))then

call matrx(feld1(ishape),feld1(ideriv),

*              feld1(ih1g),feld1(ig1g),

*              feld1(ih1l),feld1(ih2l),feld1(ig1l),feld1(ig2l),

*              ifeld(lnmult),

*              ifeld(limult),nste3,npg(13),iter,

*              feld1(icoord),ifeld(iinc),ifeld(ilelno),

*              feld1(iwrpg),feld1(iwspg),feld1(ieta1),ndiin(5))

endif

c       evaluation of the contribution of volume sources

if(ivols .eq. 1)then

c        real limit time step for truncating the time-marching process

c        when by evaluating the contribution of internal sources/forces

itli1=idint(dmaxbs/(prop(1)*delt)) + 2

call vsti(feld1(icoord),feld1(icoorp),feld1(igbg),

*             feld1(ivs),feld1(iomeb),feld1(itp0b),

*             ifeld(iitvarb),feld1(iunode),feld1(ifcs),

*             ifeld(lnmult),ifeld(limult),iter,itli1,nste5)

endif

call vector(feld1(ib),feld1(ipinf),feld1(ipsup),

*             feld1(ih1g),feld1(ig1g),feld1(ih2g),feld1(ig2g),

*             feld1(lp),feld1(lp0),feld1(lomeg),feld1(ltemp0),

*             ifeld(litvar),ifeld(lifip),

*             ifeld(ijdiag),nste3,

*             iter,itlim,nmplim,itblim,

*             feld1(iunode),feld1(iunode1),

*             feld1(ipnode),feld1(ifc))

enddo

c      deletion of files for storing the BE matrices along the

c      time-marching process

if(nste4 .gt. nste1)then

close(unit=1,status='delete')

close(unit=3,status='delete')

endif

c      evaluation of potentials at internal points

c      use of the free random access memory

c     (free space in work vector "feld1")

if(npoi .ne. 0)then

ifsp=maxpo1-ibsp1

c       number of blocks of two matrices possible to be allocated

c       in main memory

nste1=ifsp/(npoi*nno*2)

c       determination of the limit time step for truncating

c       the time-marching process

call dmaxd(dmax,feld1(icoori),feld1(icoord),npoi,nno)

itlim=idint(dmax/(prop(1)*delt)) + 2

c       nste3 = number of blocks of two matrices effectively

c               allocated in array "feld1"

nste3=min0(nste1,niter+1,itlim+1)

c       allocation of the H- and G-matrices in the work vector "feld1"

ihipt=ibsp1

igipt=ihipt + npoi*nno*(nste3)

write(iwr,*)'number of time steps treated in the ram memory'

write(iwr,*)'when by determining the sol. at internal points'

write(iwr,*)'nste1 =',nste1-1,' nste3 =',nste3-1,

*              ' itlim =',itlim

c       really used random access memory (irbsp)

irbsp=ibsp1 + (npoi)*nno*2*(nste3)

c       really used random access memory in Mbytes (bhsp)

bhsp=dble(irbsp*8)*1.d-06

write(iwr,120)bhsp

120   format(/5x,'really used random access memory',1x,

*             'at the evaluation of internal potentials:',

*         /5x,'(space for "feld1") ==> ',d12.5,1x,'Mbytes')

c       initializing work vector "feld1"

do i=ibsp1,irbsp

feld1(i)=0.d+00

enddo

c       variables for controlling the matrix-vector multiplications

c       for obtaining the internal potential vectors

v1=dfloat(itlim+1)/dfloat(nste3)

v2=dint(v1)

v3=v1-v2

if(v3 .eq. 0.)then

nblim=idint(v1)

else

nblim=idint(v2)+1

endif

itblim=(itlim+1)-(nblim-1)*nste3

nmplim=(nblim-2)*nste3+itblim

c       number of blocks of two matrices that must be calculated(nste4)

nste4=min0(niter+1,itlim+1)

c       determination of the necessary memory on disk(bhisp)

if(nste4.gt.nste1)then

nste2=nste4-nste1

bhisp=dble(npoi*nno*nste2*2*8)*1.d-6

write(iwr,130)bhisp

130     format(5x,'necessary memory on disk for evaluating',1x,

*              'internal potentials:',

*           /5x,'==> ',d12.5,1x,'Mbytes',/)

c         opening direct access files for storing matrices

ipar1=nno*8

open(unit=20,file='hig.bin',

*         form='unformatted',access='direct',recl=ipar1)

open(unit=21,file='gig.bin',

*         form='unformatted',access='direct',recl=ipar1)

endif

c       opening direct access files for storing the internal potentials

ipar2=npoi*8

open(unit=22,status='scratch',form='unformatted',

*           access='direct',recl=ipar2)

c       real limit time step for truncating the time-marching process

c       when by evaluating the contribution of internal sources/forces

itli1=idint(dmaxis/(prop(1)*delt)) + 2

do iter=1,niter

if(iter.le.itlim)then

c         necessary matrices

call innerpt(feld1(ihipt),feld1(igipt),

*                 feld1(ih1l),feld1(ih2l),feld1(ig1l),feld1(ig2l),

*                 feld1(icoori),feld1(ishape),feld1(ideriv),

*                 nste3,npg(13),iter,

*                 feld1(icoord),ifeld(iinc),ifeld(ilelno),

*                 feld1(iwrpg),feld1(iwspg),

*                 feld1(ieta1),ndiin(5))

endif

c        evaluation of the contribution of volume sources

if(ivols .eq. 1)then

c         initializing work vector "feld1"

do i=igbg,ibsp1-1

feld1(i)=0.d+00

enddo

call vsti2(feld1(icoori),feld1(icoorp),feld1(igbg),

*               feld1(ivs),feld1(iomeb),feld1(itp0b),

*               ifeld(iitvarb),feld1(iunode),feld1(ifcs),

*               iter,itli1,nste5)

endif

c        values at internal points

call intp(feld1(ihipt),feld1(igipt),feld1(ih2g),feld1(ig2g),

*             feld1(lp0),nste3,iter,itlim,nmplim,itblim,

*             feld1(isp),feld1(iunode),feld1(ipnode))

enddo

close(unit=20,status='delete')

close(unit=21,status='delete')

c      end of determination of quantities at internal points

endif

c      data output

call output(ifeld(lifip),feld1(iunode),feld1(ipnode),

*             ifeld(indnpl),feld1(isp))

if(nnopl.ne.0)close(unit=11,status='keep')

close(unit=8,status='delete')

close(unit=9,status='delete')

if(npoi.ne.0)close(unit=22,status='delete')

c     end of the transient analysis

endif

stop

c     end of the main program

end

c     =================

c     employed routines

c     =================

c     ________________

subroutine inrem(coord,coori,vs,coorp,inc,lelno,iencl,

*                 p,omega,temp0,

*                 itvar,nmult,imult,ifip,nupro,ndnopl,

*                 itvarb,tp0b,omeb)

c     subroutine for data in- and output

implicit double precision (a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/num/    pi

common/propf/  prop(5),npropb

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/vafv/   ivafv

common/temp/   delt,niter

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/rw/     ire,iwr

dimension      coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),

*               p(nno),omega(nno),temp0(nno),itvar(nno),

*               nmult(nno),imult(nno,5),ifip(nno),ndnopl(nnopl),

*               coori(npoi,ndime),

*               vs(nspo),coorp(nspo,ndime),

*               itvarb(nspo),tp0b(nspo),omeb(nspo)

c     definition of "pi" number

pi=4.*datan(1.d0)

c     In-/output of the BE mesh

c     in-/output of the element connectivities

write(iwr,10)

10 format(/5x,'element',5x,'connectivity')

do 20 ie=1,ne

read(ire,*)k,(inc(k,inoel),inoel=1,nnoel),iencl(k)

20 write(iwr,30)k,(inc(k,inoel),inoel=1,nnoel),iencl(k)

30 format(5x,i7,5x,9i6)

c     in-/output of the boundary node numbers and correspondent

c     cartesian co-ordinates

c     (boundary nodes numbers are less or equal nno)

write(iwr,50)

50 format(/5x,'nodal co-ordinates',

*       /5x,'node  ',11x,'koor x',11x,'koor y',11x,'koor z',6x,

*        'imult')

do 80 ino=1,nno

read(ire,*)k,nmult(k)

if(nmult(k) .eq. 0) then

read(ire,*)(coord(k,idime),idime=1,ndime)

go to 80

else

read(ire,*)(imult(k,kk),kk=1,nmult(k))

do 460 ii=1,nmult(k)

j=imult(k,ii)

nmult(j)=nmult(j) + 1

460   imult(j,nmult(j))=k

do 85 idime=1,ndime

85   coord(k,idime)=coord(j,idime)

endif

80 continue

do 95 ino=1,nno

if(nmult(ino) .eq. 0) then

write(iwr,105) ino,(coord(ino,idime),idime=1,ndime)

105   format(5x,i6,3d17.8)

else

write(iwr,125) ino,(coord(ino,idime),idime=1,ndime),

*                 (imult(ino,ii),ii=1,nmult(ino))

125   format(5x,i6,3d17.8,i10,i6)

endif

95 continue

c     input data of the enclosing elements mesh

if (infb.eq.2) then

c     input of connectivities of the enclosing elements

write(iwr,12)

12 format(/5x,'element',5x,'enclosing elements connectivities')

do  ie=1,nee

read(ire,*)k,(inc(k+ne,inoel),inoel=1,nnoel),iencl(k+ne)

do inoel=1,nnoel

inc(k+ne,inoel)=inc(k+ne,inoel)+nno

enddo

enddo

c     input of nodal point coordinates of enclosing elements mesh

do 81 inoe=1,nnoe

read(ire,*)k,nmult(k+nno)

if(nmult(k+nno) .eq. 0) then

read(ire,*)(coord(k+nno,idime),idime=1,ndime)

go to 81

else

read(ire,*)(imult(k+nno,kk),kk=1,nmult(k+nno))

do 461 ii=1,nmult(k+nno)

j=imult(k+nno,ii)

nmult(j)=nmult(j) + 1

461   imult(j,nmult(j))=k+nno

do 86 idime=1,ndime

86   coord(k+nno,idime)=coord(j,idime)

endif

81 continue

do ino=1,nno

do ie=ne+1,ne+nee

do inoel=1,nnoel

do idime=1,ndime

if (coord(ino,idime).ne.coord(inc(ie,inoel),idime))goto 1200

if (idime.eq.ndime) then

inc(ie,inoel)=ino

endif

enddo

1200      continue

enddo

enddo

enddo

c     output of the updated connectivities

do iee=1,nee

write(iwr,33)iee+ne,(inc(iee+ne,inoel),inoel=1,nnoel),

*              iencl(iee+ne)

33  format(5x,i7,5x,9i6)

enddo

c     output of the nodal point co-ordinates of the enc. el. mesh

write(iwr,51)

51 format(/5x,'nodal point co-ordinates of the enclosing elements',

*       /5x,'nodes ',11x,'koor x',11x,'koor y',11x,'koor z',6x,

*        'imult')

do inoe=1,nnoe

if(nmult(inoe) .eq. 0) then

write(iwr,105) inoe+nno,(coord(inoe+nno,idime),idime=1,ndime)

else

write(iwr,125) inoe+nno,(coord(inoe+nno,idime),idime=1,ndime),

*                 (imult(inoe+nno,ii),ii=1,nmult(inoe+nno))

endif

enddo

endif

c     in-/output of the internal node numbers and correspondent

c     cartesian co-ordinates.

if(npoi .ne. 0) then

ntp=nno+npoi

write(iwr,135)

135  format(/5x,'internal point co-ordinates',

*        /5x,'nodes ',11x,'coor x',11x,'coor y',11x,'coor z')

do ipoi=1,npoi

read(ire,*)j,(coori(j,idime),idime=1,ndime)

write(iwr,105)j,(coori(j,idime),idime=1,ndime)

enddo

endif

c     physical properties

c     (npropb=number of physical properties to be given)

c     for instance: prop(1)= sound speed (c), prop(2)=density (ro)

read(ire,*)(prop(iprop),iprop=1,npropb)

c     output of the physical constants

write(iwr,130)(prop(iprop),iprop=1,npropb)

130 format(/5x,'physical constants',/5x,'c =',d15.8,3x,

*       'density =',d15.8)

c     input of the boundary conditions

do ino=1,nno

p(ino)=0.

ifip(ino)=0

enddo

c     variable for input of loading data of the element nodes

c     (lelno(ie,inoel*idofn).

c     lelno(ie,inoel)=1 => the inoel-th node of the ie-th

c                          element is loaded

c                    =0 => the inoel-th node of the ie-th

c                          element is not loaded

do 157 ie=1,ne

do 157 inoel=1,nnoel

kk=inc(ie,inoel)

if(nmult(kk) .eq. 0)then

lelno(ie,inoel)=0

else

lelno(ie,inoel)=1

endif

157 continue

write(iwr,165)

165 format(/5x,'node  ',10x,'prescribed boundary values')

c     input of the variable 'kel' and 'kno' for input control of the

c     boundary values

c     kel=0  ==> bound. values are not to be furnished elementwise

c        =1  ==> bound. values are to be furnished elementwise

c     kno=0  ==> bound. values are not to be furnished nodewise

c        =1  ==> bound. values are to be furnished nodewise

read(5,*)kel,kno

if(kel .ne. 0)then

c     input of:

c     - number of elements with constant boundary values(nevcc)

c     - correspondent number, boundary value and type of boundary condition

read(5,*)nevcc

do ievcc=1,nevcc

read(ire,*)k,px,ifipx

if((px.ne.0) .or. (ifipx.eq.1))then

do inoel=1,nnoel

lelno(k,inoel)=1

kk=inc(k,inoel)

p(kk)=px

ifip(kk)=ifipx

enddo

endif

enddo

end if

if(kno .ne. 0)then

c     input of the number of elements with variable boundary values(nevcv),

c     the correspondent boundary values and type of boundary conditions at

c     each node.

c     "nnvcc" specifies the number of nodes of the ke-th element with

c     special values.

read(ire,*)nevcv

do 180 i=1,nevcv

read(ire,*)ke,nnvcc

do 180 inoel=1,nnvcc

read(ire,*)k,p(k),ifip(k)

do jnoel=1,nnoel

jj=inc(ke,jnoel)

if(k .eq. jj)then

c.........absolut value of the boundary flux(dsap)

dsap=dabs(p(k))

c.........ifip-value (is)

is=ifip(k)

if((dsap.ne.0.) .or. (is.ne.0))lelno(ke,jnoel)=1

goto 180

endif

enddo

write(iwr,*)'*** error in the input of the bound. conditions'

write(iwr,*)'*** on ,ke= ',ke,' node  =',k

stop

180   continue

endif

do 195 ino=1,nno

itip=ifip(ino)

go to(210,220),itip+1

210  write(iwr,215)ino,p(ino)

215  format(5x,i6,3x,'p =',d15.8)

go to 195

220  write(iwr,225)ino,p(ino)

225  format(5x,i6,3x,'u =',d15.8)

195 continue

if(nupro .eq. 2)then

c      input of the type of time variation of the field variable(ivafv)

c      along the time step:

c      ivafv=0 ==> potentials linear, bound. fluxes linear

c      ivafv=1 ==> potentials linear, bound. fluxes constant

read(ire,*)ivafv

if(ivafv.eq.0)then

write(iwr,*)'     linear time interpolation for'

write(iwr,*)'     potential and boundary fluxes'

endif

if(ivafv.eq.1)then

write(iwr,*)'     linear time interpolation for potentials'

write(iwr,*)'     and constant time interpolation for fluxes'

endif

c      input of the time variation type of the loading function at

c      node "ino" ("itvar") and the correspondent characteristic

c      time ("temp0") and frequency("omega")

c      input of the general loading function type(itger), correspondent

c      frequency(omger) and characteristic loading time(tpger)

read(ire,*)itger,tpger,omger

write(iwr,324)itger,tpger,omger

324  format(/5x,'general loading data',

*        /5x,'itger =',i6,3x,'tpger =',d15.8,3x,'omger =',d15.8)

do ino=1,nno

itvar(ino)=itger

temp0(ino)=tpger

omega(ino)=omger

enddo

c      input of the node number with other loading function types

c      (nnocs)

read(ire,*)nnocs

if(nnocs .ne. 0) then

write(iwr,325)

325   format(/5x,'node',3x,'loading time variation (itvar),',

*              1x,'correspondent characteristic time(temp0)',

*         /12x, 'and frequencies(omega)',/)

do inocs=1,nnocs

read(ire,*)k,itvar(k),temp0(k),omega(k)

enddo

endif

do 330 k=1,nno

itip=ifip(k) + 1

go to(340,350),itip

340  write(iwr,345)k,itvar(k),temp0(k),omega(k)

345  format(3x,i6,3x,'itvar =',i6,

*              3x,'temp0 =',d15.8,3x,'omega =',d15.8)

go to 330

350  write(iwr,355)k

355  format(3x,i6,3x,'itvar =   ***' ,3x,'temp0 =          ***   '

*              3x,'omega =          ***  ')

330  continue

endif             !if(nupro.eq.2)

c     input of internal sources data

if(ivols.eq.1)then

c      concentrated sources/forces are to be considered

do ispo=1,nspo

c       coordinates of the loaded point

read(ire,*)k,(coorp(k,idime),idime=1,ndime)

c       source values

read(ire,*)vs(k)

enddo

if(nupro.eq.2)then

c       input of the time variation type of the source/loading function

c       at node "ino" ("itvarb") and the correspondent time ("tp0b")

c       and frequency("omeb")

c       input of the general values (for all nodes) for "itvarb" (itger),

c       "tp0b" (tpger), "omeb" (omger)

read(ire,*)itger,tpger,omger

write(iwr,326)itger,tpger,omger

326   format(/5x,'general concentrated source data',

*         /5x,'itger =',i6,3x,'tpger =',d15.8,3x,'omger =',d15.8)

do ispo=1,nspo

itvarb(ispo)=itger

tp0b(ispo)=tpger

omeb(ispo)=omger

enddo

c       input of the node number with source function other than the

c       given above (nspos)

read(ire,*)nspos

if(nspos .ne. 0) then

do ispos=1,nspos

read(ire,*)k,itvarb(k),tp0b(k),omeb(k)

enddo

endif

write(iwr,327)

327   format(/5x,'node',3x,'source function data "itvarb",',

*             1x,'"tp0b" and "omeb"',/)

do k=1,nspo

write(iwr,346)k,itvarb(k),tp0b(k),omeb(k)

346    format(3x,i6,3x,'itvarb =',i6,

*                3x,'tp0b =',d15.8,3x,'omeb =',d15.8)

enddo

endif

endif

if(nupro.eq.2)then

c      input of the time step length(delt) and number

c      of time steps(niter)

read(ire,*)delt,niter

c      determination of the total analysis time(tanal)

tanal=dfloat(niter)*delt

write(iwr,140)delt,niter,tanal

140  format(/5x,'delt =',d15.8,5x,'niter =',i6,

*         5x,'tanal =',d15.8)

c      input of the numbers of boundary nodes [ndnopl(inopl)],

c      at which unknowns are to be plotted

if(nnopl .ne. 0)then

write(iwr,500)

500   format(/5x,'unknowns to be plotted at nodes:')

read(ire,*)(ndnopl(inopl),inopl=1,nnopl)

write(iwr,510)(ndnopl(inopl),inopl=1,nnopl)

510   format(5x,5i8)

endif

endif

return

end

c     ________________

subroutine varfc(p,p0,omega,temp0,itvar,ifip,fc,iter)

c     subroutine for determination of the time variation

c     of the nodal boundary values

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/temp/   delt,niter

dimension      fc(nno),p(nno),p0(nno),omega(nno),

*               temp0(nno),itvar(nno),ifip(nno)

te=dfloat(iter)*delt

do ino=1,nno

fc(ino)=0.d+00

if(ifip(ino) .eq. 0)then

c       evaluation of the loading function at time t=iter*delt

if(itvar(ino).eq.1)then

c         time-constant load with homogeneous initial conditions

if(iter .eq. 1) p0(ino)=0.

if(te .le. temp0(ino))then

fc(ino)=(p(ino)/temp0(ino))*te

else

fc(ino)=p(ino)

endif

endif

if(itvar(ino).eq.2)then

c         time-constant load with non-homogeneous initial conditions

if(iter .eq. 1) p0(ino)=p(ino)

fc(ino)=p(ino)

endif

if(itvar(ino).eq.3)then

c         time-sinesoidal load function

if(iter .eq. 1) p0(ino)=0.

fc(ino)=p(ino)*dsin(omega(ino)*te)

endif

if(itvar(ino).eq.4)then

c         retangular impulse

if(iter .eq. 1) p0(ino)=p(ino)

if(te .le. temp0(ino)) then

fc(ino)=p(ino)

else

fc(ino)=0.

endif

endif

if(itvar(ino).eq.5)then

c         triangular impulse

if(iter .eq. 1) p0(ino)=0.

if(te .le. temp0(ino)/2.) then

fc(ino)=(2.*p(ino)/temp0(ino))*te

else

if(te .le. temp0(ino)) then

fc(ino)=(2.*p(ino)/temp0(ino))*

*                     (temp0(ino)-te)

else

fc(ino)=0.

endif

endif

endif

else

c       evaluation of the nodal potentials at time t=iter*delt

if(iter .eq. 1)p0(ino)=p(ino)

fc(ino)=p(ino)

endif

enddo

c     output of the nodal values at the iter-th time step

if(iter .eq. 1)then

write(6,190)(p0(ino),ino=1,nno)

190   format(/5x,'initial boundary values',/(5x,6d17.8))

endif

return

end

c     ________________

subroutine varvs(vs,omeb,tp0b,itvarb,fc,it)

c     subroutine for determination of the time variation

c     of the given volume sources

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/temp/   delt,niter

dimension      vs(nspo),fc(nspo),itvarb(nspo),tp0b(nspo),

*               omeb(nspo)

te=dfloat(it)*delt

do 100 ispo=1,nspo

fc(ispo)=0.d+00

c      evaluation of the loading function at time t=it*delt

go to(10,20,30,40),itvarb(ispo)

c      time-constant source

10  fc(ispo)=vs(ispo)

go to 100

c      time-sinesoidal source/load function

20  fc(ispo)=vs(ispo)*dsin(omeb(ispo)*te)

go to 100

c      retangular impulse

30  if(te .le. tp0b(ispo)) then

fc(ispo)=vs(ispo)

else

fc(ispo)=0.

endif

go to 100

c      time-cosinesoidal source function

40  fc(ispo)=vs(ispo)*dcos(omeb(ispo)*te)

100 continue

return

end

c     _________________

subroutine domint(rcoor,scoor,shape,deriv,

*                  rgaub,sgaub,tgaub,

*                  wg1b,wg2b,wg3b,ndiint,npr,nps,npt,

*                  crnoe,csnoe,shapse,coorse,

*                  eta1,eta2,zeta1,zeta2,incse)

c     Subroutine for determination of the stations and evaluation

c     of the shape functions and correspondent derivatives at all

c     integration points.For singular elements the positions of

c     integration points are calculated after the Li-Han-Mang-process.

c     For non-singular elements the positions of the

c     integration points are determined following the standard Gauss integration

c     process.

implicit double precision (a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      rcoor(nse*npgse,nmsdi,nmnos),

*               scoor(nse*npgse,nmsdi,nmnos),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               crnoe(nnoel),csnoe(nnoel),

*               shapse(nnose,npgse),

*               coorse((nordm+1)*(nordm+1),ndiint),

*               incse(nse,nnose),

*               eta1(nse,npgse),eta2(nse,npgse),

*               zeta1(nse,npgse),zeta2(nse,npgse),

*               rgaub(npr),sgaub(nps),tgaub(npt),

*               wg1b(npr),wg2b(nps),wg3b(npt)

c     generation of the mesh of subelements

call gerase(coorse,incse,ndiint)

c     furnishment of the integration point positions and weights

c     factors for Gauss-quadrature

call gaussb(ndiint,npr,nps,npt,

*            rgaub,sgaub,tgaub,wg1b,wg2b,wg3b)

c     evaluation of the shape function at the integration points of the

c     subelements

call shfse(npr,nps,rgaub,sgaub,shapse)

c     evaluation of the (eta1,eta2)-co-ordinates of the integration

c     points of subelements

call cpgase(shapse,coorse,incse,eta1,eta2,zeta1,zeta2,

*            ndiint)

c     evaluation of the (r,s)-co-ordinates of the element nodes

call clnoel(crnoe,csnoe,nnoel)

c     evaluation of the (r,s)-co-ordinates of all integration points

c     related to all positions of the singular points and

c     under eventual consideration of integration subelements

call coorrs(rcoor,scoor,crnoe,csnoe,eta1,eta2,zeta1,zeta2)

c     evaluation of the shape functions and the correspondent derivatives

c     at all integration points

call shder2(rcoor,scoor,shape,deriv,ndiint)

return

end

c     _________________

subroutine gaussb(ndiint,npr,nps,npt,

*                  rgaub,sgaub,tgaub,wg1b,wg2b,wg3b)

c     subroutine for furnishing the integration point positions and

c     the correspondent weighting factors

implicit double precision(a-h,o-z)

common/rw/     ire,iwr

dimension      psg(12),wg(12),

*               rgaub(npr),sgaub(nps),tgaub(npt),

*               wg1b(npr),wg2b(nps),wg3b(npt)

do i=1,npr

rgaub(i)=0.

wg1b(i)=0.

enddo

if(ndiint .lt. 2) go to 300

do i=1,nps

sgaub(i)=0.

wg2b(i)=0.

enddo

if(ndiint .lt. 3) go to 300

do i=1,npt

tgaub(i)=0.

wg3b(i)=0.

enddo

300 continue

if((npr.gt.12) .or. (nps.gt.12) .or. (npt.gt.12))then

write(iwr,*)'please set number of integration points'

write(iwr,*)'in each integration direction less than 12'

stop

endif

do idiint=1,ndiint

if(idiint.eq.1)npdir=npr

if(idiint.eq.2)npdir=nps

if(idiint.eq.3)npdir=npt

if(npdir.eq.1)then

psg(1)= .0000000000000000d+00

wg(1) = .2000000000000000d+01

endif

if(npdir.eq.2)then

psg(1)=-.5773502691896260d+00

psg(2)=-psg(1)

wg(1) = .1000000000000000d+01

wg(2) = .1000000000000000d+01

endif

if(npdir.eq.3)then

psg(1)=-.7745966692414830d+00

psg(2)= .0000000000000000d+00

psg(3)=-psg(1)

wg(1)=  .5555555555555560d+00

wg(2)=  .8888888888888890d+00

wg(3)= wg(1)

endif

if(npdir.eq.4)then

psg(1)=-.8611363115940530d+00

psg(2)=-.3399810435848560d+00

psg(3)=-psg(2)

psg(4)=-psg(1)

wg(1) =  .3478548451374540d+00

wg(2) =  .6521451548625460d+00

wg(3) =wg(2)

wg(4) =wg(1)

endif

if(npdir.eq.5)then

psg(1)=-.9061798459386640d+00

psg(2)=-.5384693101056830d+00

psg(3)= .0000000000000000d+00

psg(4)=-psg(2)

psg(5)=-psg(1)

wg(1) =  .2369268850561890d+00

wg(2) =  .4786286704993660d+00

wg(3) =  .5688888888888890d+00

wg(4) =wg(2)

wg(5) =wg(1)

endif

if(npdir.eq.6)then

psg(1)=-.9324695142031521d+00

psg(2)=-.6612093864662650d+00

psg(3)=-.2386191860831970d+00

psg(4)=-psg(3)

psg(5)=-psg(2)

psg(6)=-psg(1)

wg(1) = .1713244923791700d+00

wg(2) = .3607615730481390d+00

wg(3) = .4679139345726910d+00

wg(4) =wg(3)

wg(5) =wg(2)

wg(6) =wg(1)

endif

if(npdir.eq.7)then

psg(1)= -.9491079123427590d+00

psg(2)= -.7415311855993940d+00

psg(3)= -.4058451513773970d+00

psg(4)=  .0000000000000000d+00

psg(5)= -psg(3)

psg(6)= -psg(2)

psg(7)= -psg(1)

wg(1) =  .1294849661688700d+00

wg(2) =  .2797053914892770d+00

wg(3) =  .3818300505051190d+00

wg(4) =  .4179591836734690d+00

wg(5) = wg(3)

wg(6) = wg(2)

wg(7) = wg(1)

endif

if(npdir.eq.8)then

psg(1)= -.9602898564975360d+00

psg(2)= -.7966664774136270d+00

psg(3)= -.5255324099163290d+00

psg(4)= -.1834346424956500d+00

psg(5)= -psg(4)

psg(6)= -psg(3)

psg(7)= -psg(2)

psg(8)= -psg(1)

wg(1) =  .1012285362903760d+00

wg(2) =  .2223810344533740d+00

wg(3) =  .3137066458778870d+00

wg(4) =  .3626837833783620d+00

wg(5) = wg(4)

wg(6) = wg(3)

wg(7) = wg(2)

wg(8) = wg(1)

endif

if(npdir.eq.9)then

psg(1)= -.9681602395076260d+00

psg(2)= -.8360311073266360d+00

psg(3)= -.6133714327005900d+00

psg(4)= -.3242534234038090d+00

psg(5)=  .0000000000000000d+00

psg(6)= -psg(4)

psg(7)= -psg(3)

psg(8)= -psg(2)

psg(9)= -psg(1)

wg(1) =  .8127438836157400d-01

wg(2) =  .1806481606948570d+00

wg(3) =  .2606106964029350d+00

wg(4) =  .3123470770400030d+00

wg(5) =  .3302393550012600d+00

wg(6) = wg(4)

wg(7) = wg(3)

wg(8) = wg(2)

wg(9) = wg(1)

endif

if(npdir.eq.10)then

psg(1)= -.9739065285171720d+00

psg(2)= -.8650633666889850d+00

psg(3)= -.6794095682990240d+00

psg(4)= -.4333953941292470d+00

psg(5)= -.1488743389816310d+00

psg(6)= -psg(5)

psg(7)= -psg(4)

psg(8)= -psg(3)

psg(9)= -psg(2)

psg(10)=-psg(1)

wg(1)=   .6667134430868810d-01

wg(2)=   .1494513491505810d+00

wg(3)=   .2190863625159820d+00

wg(4)=   .2692667193099960d+00

wg(5)=   .2955242247147530d+00

wg(6)=  wg(5)

wg(7)=  wg(4)

wg(8)=  wg(3)

wg(9)=  wg(2)

wg(10)= wg(1)

endif

if(npdir.eq.11)then

psg(1)= -.9782286581460570d+00

wg(1) =  .5566856711617370d-01

psg(2)= -.8870625997680950d+00

wg(2) =  .1255803694649050d+00

psg(3)= -.7301520055740490d+00

wg(3) =  .1862902109277340d+00

psg(4)= -.5190961292068120d+00

wg(4) =  .2331937645919900d+00

psg(5)= -.2695431559523450d+00

wg(5)=   .2628045445102470d+00

psg(6)=  .0000000000000000d+00

wg(6) =  .2729250867779010d+00

psg(7)= -psg(5)

psg(8)= -psg(4)

psg(9)= -psg(3)

psg(10)=-psg(2)

psg(11)=-psg(1)

wg(7)=  wg(5)

wg(8)=  wg(4)

wg(9)=  wg(3)

wg(10)= wg(2)

wg(11)= wg(1)

endif

if(npdir.eq.12)then

psg(1)= -.9815606342467190d+00

psg(2)= -.9041172563704790d+00

psg(3)= -.7699026741943050d+00

psg(4)= -.5873179542866170d+00

psg(5)= -.3678314989981800d+00

psg(6)= -.1252334085114690d+00

psg(7)= -psg(6)

psg(8)= -psg(5)

psg(9)= -psg(4)

psg(10)=-psg(3)

psg(11)=-psg(2)

psg(12)=-psg(1)

wg(1)=  .4717533638651180d-01

wg(2)=  .1069393259953180d+00

wg(3)=  .1600783285433460d+00

wg(4)=  .2031674267230660d+00

wg(5)=  .2334925365383550d+00

wg(6)=  .2491470458134030d+00

wg(7)=  wg(6)

wg(8)=  wg(5)

wg(9)=  wg(4)

wg(10)= wg(3)

wg(11)= wg(2)

wg(12)= wg(1)

endif

if(idiint.eq.1)then

do i=1,npr

rgaub(i)=psg(i)

wg1b(i)=wg(i)

enddo

endif

if(idiint.eq.2)then

do i=1,nps

sgaub(i)=psg(i)

wg2b(i)=wg(i)

enddo

endif

if(idiint.eq.3)then

do i=1,npt

tgaub(i)=psg(i)

wg3b(i)=wg(i)

enddo

endif

enddo

return

end

c     _________________

subroutine gerase(coorse,incse,ndiint)

c     subroutine for generation of the subelement mesh of order "nordm"

implicit double precision(a-h,o-z)

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

dimension      coorse((nordm+1)*(nordm+1),ndiint),

*               incse(nse,nnose)

c     determination of the subelement connectivities

ise=0

do 10 iordm=1,nordm

do 10 jordm=1,nordm

ise=ise+1

incse(ise,1)=(iordm-1)*(nordm+1)+jordm

incse(ise,2)=incse(ise,1)+1

incse(ise,3)=incse(ise,2)+nordm+1

10 incse(ise,4)=incse(ise,1)+nordm+1

c     determination of the co-ordinates (eta1,eta2) of

c     the nodal points of the subelements

ipse=0

do 20 iordm=1,nordm+1

do 20 jordm=1,nordm+1

ipse=ipse+1

coorse(ipse,1)=-1.+(dfloat(jordm-1))*deltal

20 coorse(ipse,2)=-1.+(dfloat(iordm-1))*deltal

return

end

c     ________________

subroutine shfse(npr,nps,rgaub,sgaub,shapse)

c     subroutine for evaluation of the two-dimensional shape functions

c     at the Gauss points of the subelements

implicit double precision(a-h,o-z)

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

dimension      rgaub(npr),sgaub(nps),shapse(nnose,npgse)

c     initializing the shape functions at all Gauss points

do 10 inose=1,nnose

do 10 ipgse=1,npgse

10 shapse(inose,ipgse)=0.

ipgse=0.

do 15 i=1,npr

r1=1.+rgaub(i)

r2=1.-rgaub(i)

do 15 j=1,nps

s1=1.+sgaub(j)

s2=1.-sgaub(j)

ipgse=ipgse+1

shapse(1,ipgse)= .25*r2*s2

shapse(2,ipgse)= .25*r1*s2

shapse(3,ipgse)= .25*r1*s1

15 shapse(4,ipgse)= .25*r2*s1

return

end

c     _________________

subroutine cpgase(shapse,coorse,incse,eta1,eta2,zeta1,zeta2,

*                  ndiint)

c     subroutine for evaluation of co-ordinates (eta1,eta2)

c     of the Gauss points of the subelements

implicit double precision(a-h,o-z)

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

dimension      coorse((nordm+1)*(nordm+1),ndiint),

*               incse(nse,nnose),shapse(nnose,npgse),

*               eta1(nse,npgse),eta2(nse,npgse),

*               zeta1(nse,npgse),zeta2(nse,npgse)

c     initializing co-ordinates (eta1,eta2), (zeta1,zeta2)

c     at all Gauss points of the subelements

do ise=1,nse

do ipgse=1,npgse

eta1(ise,ipgse)=0.

eta2(ise,ipgse)=0.

zeta1(ise,ipgse)=0.

zeta2(ise,ipgse)=0.

do inose=1,nnose

eta1(ise,ipgse)=eta1(ise,ipgse)+

*               coorse(incse(ise,inose),1)*shapse(inose,ipgse)

eta2(ise,ipgse)=eta2(ise,ipgse)+

*               coorse(incse(ise,inose),2)*shapse(inose,ipgse)

enddo

zeta1(ise,ipgse)=.5*(1. + eta1(ise,ipgse))

zeta2(ise,ipgse)=.5*(1. + eta2(ise,ipgse))

enddo

enddo

return

end

c     _________________

subroutine clnoel(crnoe,csnoe,nnoel)

c     subroutine for evaluation of the co-ordinates

c     (r,s) of the nodal points of the element

implicit double precision(a-h,o-z)

dimension crnoe(nnoel),csnoe(nnoel)

crnoe(1)=-1.d+00

crnoe(2)= 0.d+00

crnoe(3)= 1.d+00

crnoe(4)= 1.d+00

crnoe(5)= 1.d+00

crnoe(6)= 0.d+00

crnoe(7)=-1.d+00

crnoe(8)=-1.d+00

csnoe(1)=-1.d+00

csnoe(2)=-1.d+00

csnoe(3)=-1.d+00

csnoe(4)= 0.d+00

csnoe(5)= 1.d+00

csnoe(6)= 1.d+00

csnoe(7)= 1.d+00

csnoe(8)= 0.d+00

return

end

c     _________________

subroutine coorrs(rcoor,scoor,crnoe,csnoe,eta1,eta2,zeta1,zeta2)

c     subroutine for determination of the (r,s)- co-ordinates

c     of the integration points(Gauss points)

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      rcoor(nse*npgse,nmsdi,nmnos),

*               scoor(nse*npgse,nmsdi,nmnos),

*               crnoe(nnoel),csnoe(nnoel),

*               eta1(nse,npgse),eta2(nse,npgse),

*               zeta1(nse,npgse),zeta2(nse,npgse),

*               incd1(3),incd2(3),incd3(3)

if(icpt.eq.1)then

do 10 inloi=1,7,2

incd1(1)=inloi

incd2(1)=inloi+2 - ((inloi+2)/8)*8

incd3(1)=inloi+4 - ((inloi+4)/8)*8

incd1(2)=inloi

incd2(2)=incd3(1)

incd3(2)=incd3(1)+2 - ((incd3(1)+2)/8)*8

do 10 isdint=1,2

ipgsdi=0

do 10 ise=1,nse

do 10 ipgse=1,npgse

ipgsdi=ipgsdi+1

rcoor(ipgsdi,isdint,inloi)=

*   (1.-zeta1(ise,ipgse))*crnoe(incd1(isdint)) +

*   zeta1(ise,ipgse)*(1.-zeta2(ise,ipgse))*crnoe(incd2(isdint)) +

*   zeta1(ise,ipgse)*zeta2(ise,ipgse)*crnoe(incd3(isdint))

scoor(ipgsdi,isdint,inloi)=

*   (1.-zeta1(ise,ipgse))*csnoe(incd1(isdint)) +

*   zeta1(ise,ipgse)*(1.-zeta2(ise,ipgse))*csnoe(incd2(isdint)) +

*   zeta1(ise,ipgse)*zeta2(ise,ipgse)*csnoe(incd3(isdint))

10 continue

do 20 inlop=2,8,2

incd1(1)=inlop

incd2(1)=inlop+1 - ((inlop+1)/8)*8

incd3(1)=inlop+3 - ((inlop+3)/8)*8

incd1(2)=inlop

incd2(2)=incd3(1)

incd3(2)=incd3(1)+2 - ((incd3(1)+2)/8)*8

incd1(3)=inlop

incd2(3)=incd3(2)

incd3(3)=incd3(2)+2 - ((incd3(2)+2)/8)*8

do 20 isdint=1,3

ipgsdi=0

do 20 ise=1,nse

do 20 ipgse=1,npgse

ipgsdi=ipgsdi+1

rcoor(ipgsdi,isdint,inlop)=

*   (1.-zeta1(ise,ipgse))*crnoe(incd1(isdint)) +

*   zeta1(ise,ipgse)*(1.-zeta2(ise,ipgse))*crnoe(incd2(isdint)) +

*   zeta1(ise,ipgse)*zeta2(ise,ipgse)*crnoe(incd3(isdint))

scoor(ipgsdi,isdint,inlop)=

*   (1.-zeta1(ise,ipgse))*csnoe(incd1(isdint)) +

*   zeta1(ise,ipgse)*(1.-zeta2(ise,ipgse))*csnoe(incd2(isdint)) +

*   zeta1(ise,ipgse)*zeta2(ise,ipgse)*csnoe(incd3(isdint))

20 continue

c     In this case a local connectivity equal to 9 is defined and

c     there will be naturally only one integration subdomain(isdint=1)

inlo=9

ipgsdi=0

do 30 ise=1,nse

do 30 ipgse=1,npgse

ipgsdi=ipgsdi+1

rcoor(ipgsdi,1,inlo) = eta1(ise,ipgse)

scoor(ipgsdi,1,inlo) = eta2(ise,ipgse)

30 continue

endif

if(icpt.eq.0)then

ipgsdi=0

do 40 ise=1,nse

do 40 ipgse=1,npgse

ipgsdi=ipgsdi+1

rcoor(ipgsdi,1,1) = eta1(ise,ipgse)

scoor(ipgsdi,1,1) = eta2(ise,ipgse)

40   continue

endif

return

end

c     _________________

subroutine shder2(rcoor,scoor,shape,deriv,ndiint)

c     subroutine for evaluation of the two-dimensional shape functions

c     and the correspondent derivatives at all Gauss points

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      rcoor(nse*npgse,nmsdi,nmnos),

*               scoor(nse*npgse,nmsdi,nmnos),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos)

c     initializing the shape functions at all Gauss points

do 5 idime=1,2

do 5 inoel=1,nnoel

do 5 ipsine=1,nmnos

do 5 isdint=1,nmsdi

do 5 ipgsdi=1,nse*npgse

if(idime .eq. 1)shape(ipgsdi,isdint,inoel,ipsine)=0.

5 deriv(ipgsdi,isdint,idime,inoel,ipsine)=0.

do 10 ipsine=1,nmnos

if(icpt.eq.0)nsdint=1

if(icpt.eq.1)then

do 20 itest=1,7,2

if(ipsine .eq. itest) then

nsdint=2

go to 30

end if

20   continue

if(ipsine .lt. 9) then

nsdint=3

else

nsdint=1

endif

endif

30 do 10 isdint=1,nsdint

do 10 ipgsdi=1,nse*npgse

r0=rcoor(ipgsdi,isdint,ipsine)

r1=1.+rcoor(ipgsdi,isdint,ipsine)

r2=1.-rcoor(ipgsdi,isdint,ipsine)

r3=1.-rcoor(ipgsdi,isdint,ipsine)**2

s0=scoor(ipgsdi,isdint,ipsine)

s1=1.+scoor(ipgsdi,isdint,ipsine)

s2=1.-scoor(ipgsdi,isdint,ipsine)

s3=1.-scoor(ipgsdi,isdint,ipsine)**2

c     evaluation of the shape functions

shape(ipgsdi,isdint,2,ipsine)= .5*r3*s2

shape(ipgsdi,isdint,4,ipsine)= .5*r1*s3

shape(ipgsdi,isdint,6,ipsine)= .5*r3*s1

shape(ipgsdi,isdint,8,ipsine)= .5*r2*s3

shape(ipgsdi,isdint,1,ipsine) =.25*r2*s2-

*                       .5*(shape(ipgsdi,isdint,2,ipsine)+

*                           shape(ipgsdi,isdint,8,ipsine))

shape(ipgsdi,isdint,3,ipsine) =.25*r1*s2-

*                       .5*(shape(ipgsdi,isdint,2,ipsine)+

*                           shape(ipgsdi,isdint,4,ipsine))

shape(ipgsdi,isdint,5,ipsine) =.25*r1*s1-

*                       .5*(shape(ipgsdi,isdint,4,ipsine)+

*                           shape(ipgsdi,isdint,6,ipsine))

shape(ipgsdi,isdint,7,ipsine) =.25*r2*s1-

*                       .5*(shape(ipgsdi,isdint,6,ipsine)+

*                           shape(ipgsdi,isdint,8,ipsine))

c     evaluation of the derivatives of the shape functions

deriv(ipgsdi,isdint,1,2,ipsine) =-r0*s2

deriv(ipgsdi,isdint,2,2,ipsine) =-.5*r3

deriv(ipgsdi,isdint,1,4,ipsine) = .5*s3

deriv(ipgsdi,isdint,2,4,ipsine) =-r1*s0

deriv(ipgsdi,isdint,1,6,ipsine) =-r0*s1

deriv(ipgsdi,isdint,2,6,ipsine) = .5*r3

deriv(ipgsdi,isdint,1,8,ipsine) =-.5*s3

deriv(ipgsdi,isdint,2,8,ipsine) =-r2*s0

deriv(ipgsdi,isdint,1,1,ipsine) =-.25*s2-

*               .5*(deriv(ipgsdi,isdint,1,2,ipsine)+

*                   deriv(ipgsdi,isdint,1,8,ipsine))

deriv(ipgsdi,isdint,2,1,ipsine) =-.25*r2-

*               .5*(deriv(ipgsdi,isdint,2,2,ipsine)+

*                   deriv(ipgsdi,isdint,2,8,ipsine))

deriv(ipgsdi,isdint,1,3,ipsine) = .25*s2-

*               .5*(deriv(ipgsdi,isdint,1,2,ipsine)+

*                   deriv(ipgsdi,isdint,1,4,ipsine))

deriv(ipgsdi,isdint,2,3,ipsine) =-.25*r1-

*               .5*(deriv(ipgsdi,isdint,2,2,ipsine)+

*                   deriv(ipgsdi,isdint,2,4,ipsine))

deriv(ipgsdi,isdint,1,5,ipsine) = .25*s1-

*               .5*(deriv(ipgsdi,isdint,1,4,ipsine)+

*                   deriv(ipgsdi,isdint,1,6,ipsine))

deriv(ipgsdi,isdint,2,5,ipsine) = .25*r1-

*               .5*(deriv(ipgsdi,isdint,2,4,ipsine)+

*                   deriv(ipgsdi,isdint,2,6,ipsine))

deriv(ipgsdi,isdint,1,7,ipsine) =-.25*s1-

*               .5*(deriv(ipgsdi,isdint,1,6,ipsine)+

*                   deriv(ipgsdi,isdint,1,8,ipsine))

deriv(ipgsdi,isdint,2,7,ipsine) = .25*r2-

*               .5*(deriv(ipgsdi,isdint,2,6,ipsine)+

*                   deriv(ipgsdi,isdint,2,8,ipsine))

10 continue

return

end

c     _________________

subroutine vectra(ielem,iorps,ipgsdi,isdint,

*                  shape,qsi,r,rmod,derr,coord,inc)

c     subroutine for evaluation of the radius vector, the respective

c     modulus and its derivative at integration point "ipgsdi" of

c     "isdint"

implicit double precision (a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      r(3),derr(3),qsi(3),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel)

r(1)=-qsi(1)

r(2)=-qsi(2)

r(3)=-qsi(3)

do 20 inoel=1,nnoel

r(1)=r(1)+coord(inc(ielem,inoel),1)*

*                   shape(ipgsdi,isdint,inoel,iorps)

r(2)=r(2)+coord(inc(ielem,inoel),2)*

*                   shape(ipgsdi,isdint,inoel,iorps)

20 r(3)=r(3)+coord(inc(ielem,inoel),3)*

*                   shape(ipgsdi,isdint,inoel,iorps)

c     determination of the modulus of the radius vector

rmod=dsqrt(r(1)**2 + r(2)**2 + r(3)**2)

c     calculation of dr/dx1, dr/dx2 und dr/dx3

derr(1)=r(1)/rmod

derr(2)=r(2)/rmod

derr(3)=r(3)/rmod

return

end

c     _________________

subroutine jacre8(ielem,iorps,ipgsdi,isdint,deriv,djacb,vnu,

*                  coord,inc,ndiint)

c     subroutine for determination of the jacobi-determinant

c     of the element surface and the outward normal unit vector

c     at integration point "ipgsdi" of

c     the "isdint"-th integration subdomain

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      xjacm(2,3),vnu(3),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel)

c     evaluation of the Jacobi-matrix

do 10 idime=1,2

do 10 jdime=1,3

xjacm(idime,jdime)=0.

do 10 inoel=1,nnoel

10 xjacm(idime,jdime)=xjacm(idime,jdime)+

*                   deriv(ipgsdi,isdint,idime,inoel,iorps)*

*                   coord(inc(ielem,inoel),jdime)

c     evaluation of the adjunct of the Jacobi-matrix

djac1=(xjacm(1,2)*xjacm(2,3)-xjacm(2,2)*xjacm(1,3))

djac2=(xjacm(2,1)*xjacm(1,3)-xjacm(1,1)*xjacm(2,3))

djac3=(xjacm(1,1)*xjacm(2,2)-xjacm(2,1)*xjacm(1,2))

c     evaluation of the Jacobi-determinant

djacb=dsqrt(djac1**2 + djac2**2 + djac3**2)

c     evaluation of the normal unit vector

vnu(1)=djac1/djacb

vnu(2)=djac2/djacb

vnu(3)=djac3/djacb

return

end

c     _________________

subroutine matrxs(b,pinf,psup,hsl,gsl,

*                  shape,deriv,nmult,imult,jdiag,npg,

*                  coord,inc,lelno,iencl,encl,

*                  wg1b,wg2b,eta1,ndiint)

c     subroutine for assembling the global BE matrices

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/num/    pi

dimension      npg(3),qsi(3),wg1b(12),wg2b(12),

*               hsl(nnoel),gsl(nnoel),

*               b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               nmult(nno),imult(nno,5),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),

*               encl(nno),eta1(nse,npgse)

c     pointer vector of a full matrix

jdiag(1)=1

do ino=2,nno

jdiag(ino)=jdiag(ino-1)+ino

enddo

if (infb.eq.2) then

do ino=1,nno

encl(ino)=0.

enddo

endif

do 20 ino=1,nno

if(nmult(ino) .gt. 0)then

do ik=1,nmult(ino)

if(ino .gt. imult(ino,ik))then

il1=imult(ino,ik)

il2=ino

if (infb.eq.2) encl(il2)=encl(il1)

do jno=1,nno

if(il1 .le. jno)then

ip1=jdiag(jno)+il1-jno

aa=psup(ip1)

else

ip1=jdiag(il1)+jno-il1

aa=pinf(ip1)

endif

if(il2 .le. jno)then

ip2=jdiag(jno)+il2-jno

psup(ip2)=aa

else

ip2=jdiag(il2)+jno-il2

pinf(ip2)=aa

endif

b(il2,jno)=b(il1,jno)

enddo

go to 20

end if

enddo

endif

c     co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coord(ino,idime)

enddo

c     total number of elements (nte), including enclosing elements

nte=ne

if (infb.eq.2) nte=ne+nee

do 60 ielem=1,nte

c     identifying the singular element(idees) and the local order

c     of the singular point(iorps)

if(icpt.eq.1)then

do 40 inoel= 1,nnoel

kk=inc(ielem,inoel)

if(nmult(kk) .eq. 0)then

if(ino .eq. kk) then

idees=1

iorps=inoel

go to 50

end if

else

do 45 ii=1,nmult(kk)

if(ino .eq. kk  .or.  ino .eq. imult(kk,ii)) then

idees=1

iorps=inoel

go to 50

end if

45   continue

endif

40 continue

idees=0

iorps=9

endif

50 call integs(qsi,ielem,npg,idees,iorps,shape,

*            deriv,gsl,hsl,coord,inc,lelno,iencl,wg1b,wg2b,

*            eta1,ndiint)

do 60 inoel=1,nnoel

icl = inoel

if (iencl(ielem).eq.0) then

icg = inc(ielem,inoel)

if(ino .le. icg)then

ip1=jdiag(icg)+ino-icg

psup(ip1)=psup(ip1) + hsl(icl)

else

ip2=jdiag(ino)+icg-ino

pinf(ip2)=pinf(ip2) + hsl(icl)

endif

b(ino,icg)=b(ino,icg) + gsl(icl)

endif

if (iencl(ielem).eq.1) then

encl(ino)=encl(ino)+hsl(icl)

endif

60 continue

20 continue

return

end

c     _________________

subroutine vsourc(coord,vs,coorp,bb,nmult,imult)

c     subroutine for considering the contributions of concentrated

c     volume sources/forces

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/num/    pi

common/rw/     ire,iwr

dimension      qsi(3),coord(nno+nnoe,ndime),vs(nspo),

*               coorp(nspo,ndime),bb(nno),

*               nmult(nno),imult(nno,5)

do 20 ino=1,nno

if(nmult(ino) .gt. 0)then

do ik=1,nmult(ino)

if(ino .gt. imult(ino,ik))then

il1=imult(ino,ik)

il2=ino

bb(il2)=bb(il1)

go to 20

endif

enddo

endif

c     co-ordinates of the point with concentrated source

do idime=1,ndime

qsi(idime)= coord(ino,idime)

enddo

do ispo=1,nspo

c      calculation of the radius vector

call radio(qsi,coorp,rmod,ispo)

if(rmod.eq.0.)then

write(iwr,1000)ino,(qsi(idime),idime=1,ndime)

stop

endif

c      fundamental solution at the loaded point

call fs3d1(rmod,fkeu)

bb(ino)=bb(ino)+ fkeu*vs(ispo)

enddo

20 continue

1000 format(/5x,'the solution is singular at point ',

*       /5x,'ino =',i5,'coordinates: ',1x,3d15.4)

return

end

c     ________________

subroutine radio(qsi,coorp,rmod,ispo)

c     subroutine for evaluation of the radius vector and

c     correspondent modulus

implicit double precision (a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

dimension      r(3),qsi(3),coorp(nspo,ndime)

c     radius vector

do idime=1,ndime

r(idime)=(coorp(ispo,idime)-qsi(idime))

enddo

c     modulus of the radius vector

rmod=0.

do idime=1,ndime

rmod=rmod+ r(idime)**2

enddo

rmod=dsqrt(rmod)

return

end

c     ________________

subroutine fs3d1(rmod,fkeu)

c     subroutine for evaluation of the fundamental potential kernel

c     at radius rmod

implicit double precision (a-h,o-z)

common/num/    pi

fkeu = -1.d+00/(4.*pi*rmod)

return

end

c     __________________

subroutine innerps(hip,gip,hsl,gsl,u,p,sp,vs,coori,coorp,

*                   shape,deriv,npg,

*                   coord,inc,lelno,iencl,wg1b,wg2b,

*                   eta1,ndiint)

c     subroutine for calculating the potentials at internal points

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/num/    pi

common/rw/     ire,iwr

dimension      hip(npoi,nno),gip(npoi,nno),

*               u(nno),p(nno),coori(npoi,ndime),

*               hsl(nnoel),gsl(nnoel),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),sp(npoi),vs(nspo),

*               coorp(nspo,ndime),eta1(nse,npgse),

*               wg1b(12),wg2b(12),npg(3),qsi(3)

do ino=1,npoi

sp(ino)=0.d+00

enddo

do 10 ino=1,npoi

c     co-ordinates of internal source points

do idime=1,ndime

qsi(idime)= coori(ino,idime)

enddo

do 10 ielem=1,ne

idees=0

iorps=9

c     integration over elements

call integs(qsi,ielem,npg,idees,iorps,shape,

*            deriv,gsl,hsl,

*            coord,inc,lelno,iencl,wg1b,wg2b,

*            eta1,ndiint)

ilg=ino

do 10 inoel=1,nnoel

icl = inoel

if (iencl(ielem).eq.0) then

icg = inc(ielem,inoel)

hip(ilg,icg)= hip(ilg,icg) + hsl(icl)

gip(ilg,icg)= gip(ilg,icg) + gsl(icl)

endif

10 continue

c     contribution of concentrated sources

if(ivols.eq.1)then

do ino=1,npoi

c       co-ordinates of the point with concentrated source

do idime=1,ndime

qsi(idime)= coori(ino,idime)

enddo

do ispo=1,nspo

c        calculation of the radius vector

call radio(qsi,coorp,rmod,ispo)

if(rmod.eq.0.)then

write(iwr,1000)ino,(qsi(idime),idime=1,ndime)

stop

endif

c        fundamental solution at the loaded point

call fs3d1(rmod,fkeu)

sp(ino)=sp(ino) + fkeu*vs(ispo)

enddo

enddo

endif

c     evaluation of the potentials at internal points

do jno=1,nno

do ino=1,npoi

sp(ino)=sp(ino) + gip(ino,jno)*p(jno)

*                  - hip(ino,jno)*u(jno)

enddo

enddo

1000 format(/5x,'the solution is singular at point ',

*       /5x,'ino =',i5,'coordinates: ',1x,3d15.4)

return

end

c     _________________

subroutine matrx0(hsg,hsl,b,pinf,psup,shape,deriv,

*                  h1g,g1g,h1,h2,g1,g2,

*                  nmult,imult,jdiag,nste3,npg,iter,

*                  coord,inc,lelno,iencl,encl,

*                  wg1b,wg2b,eta1,ndiint)

c     subroutine for assembling the global BE matrices for the

c     1st time step

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/num/    pi

common/temp/   delt,niter

dimension      npg(3),qsi(3),wg1b(12),wg2b(12),

*               h1(nnoel),h2(nnoel),g1(nnoel),g2(nnoel),

*               hsl(nnoel),hsg(nno*nno),

*               b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               h1g(nno*nno*nste3),g1g(nno*nno*nste3),

*               nmult(nno),imult(nno,5),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),eta1(nse,npgse),

*               encl(nno)

c     pointer vector of a full matrix

jdiag(1)=1

do ino=2,nno

jdiag(ino)=jdiag(ino-1)+ino

enddo

if (infb.eq.2) then

do ino=1,nno

encl(ino)=0.

enddo

endif

do 20 ino=1,nno

if(nmult(ino) .gt. 0)then

do 35 ik=1,nmult(ino)

if(ino .gt. imult(ino,ik))then

il1=imult(ino,ik)

il2=ino

il3=(il1-1)*nno

i1=(il2-1)*nno

if (infb.eq.2) encl(il2)=encl(il1)

do 37 jno=1,nno

if(il1 .le. jno)then

ip1=jdiag(jno)+il1-jno

aa=psup(ip1)

else

ip1=jdiag(il1)+jno-il1

aa=pinf(ip1)

endif

if(il2 .le. jno)then

ip2=jdiag(jno)+il2-jno

psup(ip2)=aa

else

ip2=jdiag(il2)+jno-il2

pinf(ip2)=aa

endif

b(il2,jno)=b(il1,jno)

hsg(i1+jno)=hsg(il3+jno)

h1g(i1+jno)=h1g(il3+jno)

37     g1g(i1+jno)=g1g(il3+jno)

go to 20

end if

35 continue

endif

c     co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coord(ino,idime)

enddo

nte=ne

if (infb.eq.2)nte=ne+nee

c     loop over the elements

do 60 ielem=1,nte

c     initializing the coefficients of H- und G-matrix

do inoel=1,nnoel

ii=inoel

h1(inoel)=0.d+00

h2(inoel)=0.d+00

g1(inoel)=0.d+00

g2(inoel)=0.d+00

hsl(inoel)=0.d+00

enddo

c     identifying the singular element (idees) and the local order

c     of the singular point (iorps)

do 40 inoel= 1,nnoel

kk=inc(ielem,inoel)

if(nmult(kk) .eq. 0)then

if(ino .eq. kk) then

idees=1

iorps=inoel

go to 50

endif

else

do 45 ii=1,nmult(kk)

if(ino .eq. kk  .or.  ino .eq. imult(kk,ii)) then

idees=1

iorps=inoel

go to 50

end if

45   continue

endif

40 continue

idees=0

iorps=9

50 continue

call integ0(qsi,ielem,npg,iter,idees,iorps,shape,

*            deriv,h1,h2,g1,g2,hsl,

*            coord,inc,lelno,iencl,wg1b,wg2b,eta1,ndiint)

ilg=ino

i1=(ino-1)*nno

do 60 inoel=1,nnoel

icl = inoel

if(iencl(ielem).eq.0) then

icg = inc(ielem,inoel)

if(ilg .le. icg)then

ip1=jdiag(icg)+ilg-icg

psup(ip1)=psup(ip1) + h2(icl)

else

ip2=jdiag(ilg)+icg-ilg

pinf(ip2)=pinf(ip2) + h2(icl)

endif

b(ilg,icg)=b(ilg,icg) + g2(icl)

h1g(icg+i1)=h1g(icg+i1) + h1(icl)

g1g(icg+i1)=g1g(icg+i1) + g1(icl)

hsg(icg+i1)=hsg(icg+i1) + hsl(icl)

endif

if(iencl(ielem).eq.1) then

encl(ilg)=encl(ilg)+hsl(icl)

endif

60 continue

20 continue

return

end

c     ________________

subroutine matrx(shape,deriv,h1g,g1g,h1,h2,g1,g2,

*                 nmult,imult,nste3,npg,iter,

*                 coord,inc,lelno,wg1b,wg2b,eta1,ndiint)

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/num/    pi

common/temp/   delt,niter

dimension      npg(3),qsi(3),wg1b(12),wg2b(12),

*               h1(nnoel),h2(nnoel),g1(nnoel),g2(nnoel),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               h1g(nno*nno*nste3),

*               g1g(nno*nno*nste3),

*               nmult(nno),imult(nno,5),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),eta1(nse,npgse)

x1=dfloat(iter)/dfloat(nste3)

x2=dint(x1)

x3=x1-x2

if(x3 .eq. 0.)then

nbl=idint(x1)

else

nbl=idint(x2) + 1

endif

itb=iter - (nbl-1)*nste3

if(iter .ge. 2)then

itb1=itb

if(iter .eq. (nbl-1)*nste3+1)itb1=nste3+1

ilg0=(itb1-2)*nno*nno

endif

ilg1=(itb-1)*nno*nno

c     fields "h1g" and "h2g" on disk write and initialize

if(iter .gt. nste3)then

irek=(nbl-2)*nste3*nno+(itb-1)*nno

do 190 i=1,nno

irek=irek+1

ipi=(itb-1)*nno*nno+(i-1)*nno+1

ipf=(itb-1)*nno*nno+i*nno

write(unit=1,rec=irek)(h1g(j),j=ipi,ipf)

190   write(unit=3,rec=irek)(g1g(j),j=ipi,ipf)

do 180 i=ilg1+1,itb*nno*nno

h1g(i)=0.

180   g1g(i)=0.

endif

do 20 ino=1,nno

if(nmult(ino) .gt. 0)then

do 35 ik=1,nmult(ino)

if(ino .gt. imult(ino,ik))then

il3=(imult(ino,ik)-1)*nno

i1=(ino-1)*nno

do 38 jno=1,nno

h1g(ilg1+i1+jno)=h1g(ilg1+il3+jno)

g1g(ilg1+i1+jno)=g1g(ilg1+il3+jno)

h1g(ilg0+i1+jno)=h1g(ilg0+il3+jno)

38     g1g(ilg0+i1+jno)=g1g(ilg0+il3+jno)

go to 20

endif

35  continue

endif

c     co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coord(ino,idime)

enddo

c     loop over the elements

do 60 ielem=1,ne

c     initializing coefficients of H- and G-matrix

do inoel=1,nnoel

h1(inoel)=0.d+00

h2(inoel)=0.d+00

g1(inoel)=0.d+00

g2(inoel)=0.d+00

enddo

c     identifying the singular element(idees) and the local order

c     of the singular point(iorps)

if(icpt.eq.1)then

if(iter .le. icorr)then

do 40 inoel= 1,nnoel

kk=inc(ielem,inoel)

if(nmult(kk) .eq. 0)then

if(ino .eq. kk) then

idees=1

iorps=inoel

go to 50

end if

else

do 45 ii=1,nmult(kk)

if(ino .eq. kk  .or.  ino .eq. imult(kk,ii)) then

idees=1

iorps=inoel

go to 50

end if

45   continue

endif

40 continue

endif

idees=0

iorps=9

endif

50   continue

c     integration over elements

call integ(qsi,ielem,npg,iter,idees,iorps,shape,

*           deriv,h1,h2,g1,g2,

*           coord,inc,lelno,wg1b,wg2b,eta1,ndiint)

i1=(ino-1)*nno

do 60 inoel=1,nnoel

icg = inc(ielem,inoel)

icl = inoel

h1g(icg+i1+ilg0)=h1g(icg+i1+ilg0) + h2(icl)

g1g(icg+i1+ilg0)=g1g(icg+i1+ilg0) + g2(icl)

h1g(icg+i1+ilg1)=h1g(icg+i1+ilg1) + h1(icl)

g1g(icg+i1+ilg1)=g1g(icg+i1+ilg1) + g1(icl)

60 continue

20 continue

return

end

c     _______________

subroutine vsti(coord,coorp,gbg,vs,omeb,tp0b,itvarb,

*                u,fc,nmult,imult,iter,itli1,nste5)

c     subroutine for consideration of the contribution of

c     time-dependent volume sources

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

common/temp/   delt,niter

common/rw/     ire,iwr

dimension      qsi(3),coord(nno+nnoe,ndime),

*               coorp(nspo,ndime),gbg(nno*nspo*nste5),

*               vs(nspo),fc(nspo),u(nno),

*               itvarb(nspo),tp0b(nspo),

*               omeb(nspo),nmult(nno),imult(nno,5)

if(iter.le.itli1)then

c      determination of the limits of the interval, in which

c      the time-integrated potential kernel exists

riter=dfloat(iter)

riter1=riter

riter2=riter-1.

rinf=riter2*prop(1)*delt

rmax=riter1*prop(1)*delt

ilg0=(iter-1)*(nno)*(nspo)

do 20 ino=1,nno

if(nmult(ino) .gt. 0)then

do ik=1,nmult(ino)

if(ino .gt. imult(ino,ik))then

il3=(imult(ino,ik)-1)*(nspo)

i1=(ino-1)*(nspo)

do jno=1,(nspo)

gbg(ilg0+i1+jno)=gbg(ilg0+il3+jno)

enddo

go to 20

endif

enddo

endif

c      co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coord(ino,idime)

enddo

i1=(ino-1)*nspo

do ispo=1,nspo

c       calculation of the radius vector

call radio(qsi,coorp,rmod,ispo)

if(rmod.eq.0. .and. iter.eq.1)then

write(iwr,1000)ino,(qsi(idime),idime=1,ndime)

stop

endif

if(rmod .gt. rinf .and. rmod .lt. rmax)then

icg=ispo

c         fundamental solution at the loaded point

call fs3d2(rmod,fkeu)

gbg(icg+i1+ilg0)=gbg(icg+i1+ilg0) + fkeu

endif

enddo

20  continue

endif

do ino=1,nno

u(ino)=0.

enddo

if(iter .le. itli1)then

itf=1

else

itf=iter-itli1+1

endif

do 200 it=(iter),itf,-1

call varvs(vs,omeb,tp0b,itvarb,fc,it)

it2=iter-it+1

it2a =(it2-1)*(nno)*(nspo)

do 200 ino=1,nno

j1= it2a + (ino-1)*nspo

do 200 jno=1,nspo

j1=j1+1

200 u(ino)=u(ino) + gbg(j1)*fc(jno)

1000 format(/5x,'the solution is singular at point ',

*       /5x,'ino =',i5,'coordinates: ',1x,3d15.4)

return

end

c     ________________

subroutine vsti2(coori,coorp,gbg,vs,omeb,tp0b,itvarb,

*                 u,fc,iter,itli1,nste5)

c     subroutine for consideration of the contribution of

c     time-dependent volume sources when evaluating the

c     response at internal points

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

common/temp/   delt,niter

common/rw/     ire,iwr

dimension      qsi(3),coori(npoi,ndime),

*               coorp(nspo,ndime),

*               gbg(npoi*nspo*nste5),

*               vs(nspo),fc(nspo),u(npoi),

*               itvarb(nspo),tp0b(nspo),omeb(nspo)

if(iter.le.itli1)then

c      determination of the limits of the interval, in which

c      the time-integrated potential kernel exists

riter=dfloat(iter)

riter1=riter

riter2=riter-1.

rinf=riter2*prop(1)*delt

rmax=riter1*prop(1)*delt

ilg0=(iter-1)*(npoi)*(nspo)

do ipoi=1,npoi

c       co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coori(ipoi,idime)

enddo

i1=(ipoi-1)*nspo

do ispo=1,nspo

c        calculation of the radius vector

call radio(qsi,coorp,rmod,ispo)

if(rmod.eq.0. .and. iter.eq.1)then

write(iwr,1000)ipoi,(qsi(idime),idime=1,ndime)

stop

endif

if(rmod .gt. rinf .and. rmod .lt. rmax)then

icg=ispo

c          fundamental solution at the loaded point

call fs3d2(rmod,fkeu)

gbg(icg+i1+ilg0)=gbg(icg+i1+ilg0) + fkeu

endif

enddo

enddo

endif

do ino=1,npoi

u(ino)=0.

enddo

if(iter .le. itli1)then

itf=1

else

itf=iter-itli1+1

endif

do it=(iter),itf,-1

call varvs(vs,omeb,tp0b,itvarb,fc,it)

it2=iter-it+1

it2a =(it2-1)*(npoi)*(nspo)

do ino=1,npoi

j1= it2a + (ino-1)*nspo

do jno=1,nspo

j1=j1+1

u(ino)=u(ino) + gbg(j1)*fc(jno)

enddo

enddo

enddo

1000 format(/5x,'the solution is singular at internal point ',

*       /5x,'ipoi=',i5,'coordinates: ',1x,3d15.4)

return

end

c     ________________

subroutine fs3d2(rmod,fkeu)

c     subroutine for evaluation of the fundamental potential kernel

c     at the radius rmod

implicit double precision (a-h,o-z)

common/num/    pi

fkeu= 1.d+00/(4.*pi*rmod)

return

end

c     __________________

subroutine innerpt(h1g,g1g,h1,h2,g1,g2,

*                   coori,shape,deriv,

*                   nste3,npg,iter,

*                   coord,inc,lelno,wg1b,wg2b,eta1,ndiint)

c     subroutine for calculating the matrices necessary for the

c     determination potentials at internal points

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/num/    pi

common/temp/   delt,niter

dimension      npg(3),qsi(3),wg1b(12),wg2b(12),

*               coori(npoi,ndime),

*               h1(nnoel),h2(nnoel),g1(nnoel),g2(nnoel),

*               h1g(npoi*nno*nste3),g1g(npoi*nno*nste3),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),eta1(nse,npgse)

x1=dfloat(iter+1)/dfloat(nste3)

x2=dint(x1)

x3=x1-x2

if(x3 .eq. 0.)then

nbl=idint(x1)

else

nbl=idint(x2) + 1

endif

itb=(iter+1) - (nbl-1)*nste3

itb1=itb

if(iter+1 .eq. (nbl-1)*nste3+1)itb1=nste3+1

ilg0=(itb1-2)*npoi*nno

ilg1=(itb-1)*npoi*nno

c     fields "h1g" and "h2g" on disk write and initialize

if(iter+1 .gt. nste3)then

irek=(nbl-2)*nste3*npoi+(itb-1)*npoi

do i=1,npoi

irek=irek+1

ipi=(itb-1)*npoi*nno+(i-1)*nno+1

ipf=(itb-1)*npoi*nno+i*nno

write(unit=20,rec=irek)(h1g(j),j=ipi,ipf)

write(unit=21,rec=irek)(g1g(j),j=ipi,ipf)

enddo

do i=ilg1+1,(itb)*npoi*nno

h1g(i)=0.

g1g(i)=0.

enddo

endif

do 20 ino=1,npoi

c     co-ordinates of the source point

do idime=1,ndime

qsi(idime)= coori(ino,idime)

enddo

c     loop over the elements

do ielem=1,ne

c      initializing coefficients of H- and G-matrix

do inoel=1,nnoel

h1(inoel)=0.d+00

h2(inoel)=0.d+00

g1(inoel)=0.d+00

g2(inoel)=0.d+00

enddo

c      integration over elements

idees=0

iorps=9

call integ(qsi,ielem,npg,iter,idees,iorps,shape,

*            deriv,h1,h2,g1,g2,

*            coord,inc,lelno,wg1b,wg2b,eta1,ndiint)

i1=(ino-1)*nno

do inoel=1,nnoel

icg = inc(ielem,inoel)

icl = inoel

h1g(icg+i1+ilg0)=h1g(icg+i1+ilg0) + h2(icl)

g1g(icg+i1+ilg0)=g1g(icg+i1+ilg0) + g2(icl)

h1g(icg+i1+ilg1)=h1g(icg+i1+ilg1) + h1(icl)

g1g(icg+i1+ilg1)=g1g(icg+i1+ilg1) + g1(icl)

enddo

enddo

20 continue

return

end

c     _______________

subroutine intp(h1g,g1g,h2g,g2g,p0,

*                nste3,iter,itlim,nmplim,itblim,u,ua,pa)

c     subroutine for obtaining the potentials at internal points

c     at time step "iter"

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

dimension      u(npoi),pa(nno),ua(nno),

*               h2g(nno),g2g(nno),h1g(nno*npoi*nste3),

*               g1g(nno*npoi*nste3),p0(nno)

c     initializing potential vector "u"

if(ivols.eq.0)then

c      there is no volume source

do ino=1,npoi

u(ino)=0.d+00

enddo

endif

x1=dfloat(iter+1)/dfloat(nste3)

x2=dint(x1)

x3=x1-x2

if(x3 .eq. 0.)then

nbl=idint(x1)

else

nbl=idint(x2) + 1

endif

itb=(iter+1) - (nbl-1)*nste3

if((iter+1 .le. nste3) .or. (itlim+1.le.nste3))then

itf=max0(1,iter-itlim)

do 200 it=(iter),itf,-1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

it2=(iter+1)-it

do 200 inoi=1,npoi

j1=(it2-1)*npoi*nno + (inoi-1)*nno

do 200 jno=1,nno

j1=j1+1

200   u(inoi)=u(inoi) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

if((iter+1.gt.nste3) .and. (itlim+1.gt.nste3))then

irek1=0

if(iter .le. itlim)then

nmpl=(nbl-2)*nste3 + itb

else

nmpl=nmplim

endif

do 190 it=(iter),(iter+1-nmpl),-1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

do 190 inoi=1,npoi

irek1=irek1+1

read(unit=20,rec=irek1)(h2g(jno),jno=1,nno)

read(unit=21,rec=irek1)(g2g(jno),jno=1,nno)

do 190 jno=1,nno

190   u(inoi)=u(inoi) + g2g(jno)*pa(jno) - h2g(jno)*ua(jno)

iti=iter-nmpl

if(iter .le. itlim)then

itf=itb

itf0=1

itb2=itb

else

itf=iti-(nste3-itblim)+1

itf0=itf-itblim

itb2=itblim

endif

if(itb2 .lt. nste3)then

it2=itb2-1

do 210 it=iti,itf,-1

it2=it2+1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

ipp=it2*npoi*nno

do 210 ino=1,npoi

j1=ipp + (ino-1)*nno

do 210 jno=1,nno

j1=j1+1

210     u(ino)=u(ino) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

if(itb2 .gt. 1)then

it2=0

do 220 it=itf-1,itf0,-1

it2=it2+1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

ipp=(it2-1)*npoi*nno

do 220 ino=1,npoi

j1=ipp + (ino-1)*nno

do 220 jno=1,nno

j1=j1+1

220     u(ino)=u(ino) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

endif

c     consideration of the initial conditions (fluxes).

c     initial potentials are considered to be homogeneous

if(iter .le. itlim)then

do ino=1,npoi

j1=(itb-1)*npoi*nno + (ino-1)*nno

do jno=1,nno

j1=j1+1

u(ino)=u(ino) + g1g(j1)*p0(jno)

enddo

enddo

endif

write(unit=22,rec=iter)(u(ino),ino=1,npoi)

return

end

c     _________________

subroutine integs(qsi,ielem,npg,idees,iorps,shape,

*                 deriv,gsl,hsl,

*                 coord,inc,lelno,iencl,wg1b,wg2b,

*                 eta1,ndiint)

c     subroutine for the numerical evaluation of the

c     boundary integrals (stationary problems)

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/num/    pi

common/propf/  prop(5),npropb

common/cicpt/  icorr,icpt,nmsdi,nmnos

dimension      npg(3),wg1b(12),wg2b(12),

*               hsl(nnoel),gsl(nnoel),

*               qsi(3),asdt(3),r(3),derr(3),vnu(3),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),

*               eta1(nse,npgse)

c     determination of the number of integration subelements

if(icpt.eq.0)nsdint=1

if(icpt.eq.1)call dnsdin(idees,iorps,nsdint)

c     determination of the area of the triangular

c     integration subelements (asdt)

if(nsdint .eq. 1) go to 50

if(nsdint .eq. 2) then

asdt(1)=2.d+00

asdt(2)=2.d+00

else

asdt(1)=1.d+00

asdt(2)=2.d+00

asdt(3)=1.d+00

end if

c     initializing the coefficients of the H- and G-matrix

50 do inoel=1,nnoel

gsl(inoel)=0.d+00

hsl(inoel)=0.d+00

enddo

c     numerical integration

do 20 isdint=1,nsdint

ipgsdi=0

do 20 ise=1,nse

ipgse=0

do 20 igauss=1,npg(1)

do 20 jgauss=1,npg(2)

ipgse=ipgse+1

ipgsdi=ipgsdi+1

c     determination of the radius vector, its modulus and

c     its derivatives at the integration point "ipgsdi" from "isdint"

if(icpt.eq.0)then

call vectra(ielem,1,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

if(icpt.eq.1)then

call vectra(ielem,iorps,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

c     determination of the jacobi-determinant of the element surface at

c     the integration point "ipgsdi" from the "isdint"-th integration

c     subelement

if(icpt.eq.0)then

call jacre8(ielem,1,ipgsdi,isdint,deriv,djacb,vnu,

*              coord,inc,ndiint)

endif

if(icpt.eq.1)then

call jacre8(ielem,iorps,ipgsdi,isdint,deriv,djacb,vnu,

*              coord,inc,ndiint)

endif

c     determination of the derivative of radius "r" with respect to

c     the outward normal direction (dr/dn)

derrn=0.

do 40 idime=1,3

40 derrn=derrn + derr(idime)*vnu(idime)

c     determination of the "dsup1"

c     at the integration point (ipgsdi,isdint)

if(nsdint .eq. 1) then

dsup1= djacb*djacse*wg1b(igauss)*wg2b(jgauss)

else

dsup1= djacb*(1. + eta1(ise,ipgse))*

*         asdt(isdint)*(.25)*djacse*wg1b(igauss)*wg2b(jgauss)

end if

c     integration of the stationary fundamental kernels

c     - fundamental flux kernel (HSL =local matrix)

has  = -(derrn)/(4.*pi*rmod**2)

c     - fundamental potential kernel (GSL= local matrix)

if (iencl(ielem).eq.0) then

gas = 1.d+00/(4.*pi*rmod)

endif

do 20 inoel=1,nnoel

if(icpt.eq.0)dsup=shape(ipgsdi,isdint,inoel,1)*dsup1

if(icpt.eq.1)dsup=shape(ipgsdi,isdint,inoel,iorps)*dsup1

if (iorps.ne.inoel) then

hsl(inoel) = hsl(inoel) + has*dsup

endif

if (iencl(ielem).eq.0) then

if(lelno(ielem,inoel).eq.1)then

gsl(inoel) = gsl(inoel) + gas*dsup

endif

endif

20 continue

return

end

c     _________________

subroutine integ0(qsi,ielem,npg,iter,idees,iorps,shape,

*                 deriv,h1,h2,g1,g2,hsl,

*                 coord,inc,lelno,iencl,

*                 wg1b,wg2b,eta1,ndiint)

c     subroutine for numerical evaluation of the boundary integrals

c     at the 1st time iteration

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/num/    pi

common/propf/  prop(5),npropb

common/temp/   delt,niter

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/vafv/   ivafv

dimension      npg(3),qsi(3),asdt(3),r(3),derr(3),vnu(3),

*               hsl(nnoel),h1(nnoel),h2(nnoel),g1(nnoel),

*               g2(nnoel),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),iencl(ne+nee),

*               eta1(nse,npgse),wg1b(12),wg2b(12)

c     determination of the limits of the interval, in which

c     the time-dependent potential and flux kernels exist

c     at the 1st time step

riter=dfloat(iter)

rinf=0.d+00

rmax=prop(1)*delt

c     determination of the number of integration subdomains

if(icpt.eq.0)nsdint=1

if(icpt.eq.1)call dnsdin(idees,iorps,nsdint)

c     determination of the area of the triangular

c     integration subdomains (asdt)

if(nsdint .eq. 1) go to 50

if(nsdint .eq. 2) then

asdt(1)=2.d+00

asdt(2)=2.d+00

else

asdt(1)=1.d+00

asdt(2)=2.d+00

asdt(3)=1.d+00

end if

50   continue

c     numerical integration

do 20 isdint=1,nsdint

ipgsdi=0

do 20 ise=1,nse

ipgse=0

do 20 igauss=1,npg(1)

do 20 jgauss=1,npg(2)

ipgse=ipgse+1

ipgsdi=ipgsdi+1

c     determination of the radius vector, its modulus and

c     its derivatives at the integration point "ipgsdi" from "isdint"

if(icpt.eq.0)then

call vectra(ielem,1,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

if(icpt.eq.1)then

call vectra(ielem,iorps,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

c     determination of the jacobi-determinant of the element surface at

c     the integration point "ipgsdi" from the "isdint"-th integration

c     subelement

if(icpt.eq.0)then

call jacre8(ielem,1,ipgsdi,isdint,deriv,djacb,vnu,

*              coord,inc,ndiint)

endif

if(icpt.eq.1)then

call jacre8(ielem,iorps,ipgsdi,isdint,deriv,djacb,vnu,

*              coord,inc,ndiint)

endif

c     determination of the derivative of the radius "r" with respect to

c     the outward normal direction (dr/dn)

derrn=0.

do 40 idime=1,3

40 derrn=derrn + derr(idime)*vnu(idime)

c     determination of "dsup1"

c     at the integration point (ipgsdi,isdint)

if(nsdint .eq. 1) then

dsup1= djacb*djacse*wg1b(igauss)*wg2b(jgauss)

else

dsup1= djacb*(1. + eta1(ise,ipgse))*

*         asdt(isdint)*(.25)*djacse*wg1b(igauss)*wg2b(jgauss)

end if

if(rmod.gt.rinf .and. rmod.lt.rmax .and. iencl(ielem).eq.0) then

c     determination of auxiliary constants

c     (for the time-dependent kernels)

ca1 = 1./(4.*pi*rmod**2)

ca2 = 1./(4.*pi*rmod)

ca3 = rmod/(prop(1)*delt)

endif

c     integration of the stationary fundamental flux kernels

c     (HSL=local matrix)

has = -(derrn)/(4.*pi*rmod**2)

c     integration of the time-dependent potential and flux kernels

if(rmod.gt.rinf .and. rmod.lt.rmax .and. iencl(ielem).eq.0) then

ha1 = ca1*(riter-1.)*derrn

if(idees.eq.1)ha2s=0.d+00

ha2 = -ca1*riter*derrn

if(ivafv.eq.0)then

c         boundary fluxes and potentials are linearly interpolated

ga1 = ca2*(1.- riter + ca3)

ga2 = ca2*(riter - ca3)

endif

if(ivafv.eq.1)then

c         constant time interpolation for boundary fluxes and linear

c         for potentials.

ga1 = 0.d+00

ga2 = ca2

endif

endif

if((rmod .ge. rmax).and.(idees.eq.1))then

ha2s=-has

endif

do 20 inoel=1,nnoel

if(icpt.eq.0)dsup=shape(ipgsdi,isdint,inoel,1)*dsup1

if(icpt.eq.1)dsup=shape(ipgsdi,isdint,inoel,iorps)*dsup1

hsl(inoel) = hsl(inoel) + has*dsup

if(rmod.gt.rinf .and. rmod.lt.rmax .and. iencl(ielem).eq.0) then

h1(inoel) = h1(inoel) + ha1*dsup

if(inoel.eq.iorps)then

h2(inoel) = h2(inoel) + ha2s*dsup

else

h2(inoel) = h2(inoel) + ha2*dsup

endif

if(lelno(ielem,inoel) .eq. 1)then

g1(inoel) = g1(inoel) + ga1*dsup

g2(inoel) = g2(inoel) + ga2*dsup

endif

endif

if(rmod.ge.rmax .and. inoel.eq.iorps)then

h2(inoel) = h2(inoel) + ha2s*dsup

endif

20 continue

return

end

c     ________________

subroutine integ(qsi,ielem,npg,iter,idees,iorps,shape,

*                 deriv,h1,h2,g1,g2,

*                 coord,inc,lelno,wg1b,wg2b,eta1,ndiint)

c     subroutine for numerical evaluation of the boundary integrals

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/dsube/  deltal,djacse,nordm,nnose,nse,npgse

common/num/    pi

common/propf/  prop(5),npropb

common/temp/   delt,niter

common/cicpt/  icorr,icpt,nmsdi,nmnos

common/vafv/   ivafv

dimension      npg(3),qsi(3),asdt(3),r(3),derr(3),vnu(3),

*               h1(nnoel),h2(nnoel),g1(nnoel),g2(nnoel),

*               shape(nse*npgse,nmsdi,nnoel,nmnos),

*               deriv(nse*npgse,nmsdi,ndiint,nnoel,nmnos),

*               coord(nno+nnoe,ndime),inc(ne+nee,nnoel),

*               lelno(ne,nnoel),eta1(nse,npgse),

*               wg1b(12),wg2b(12)

c     determination of the limits of the interval, in which

c     the time-dependent potential and flux kernels exist

riter=dfloat(iter)

riter1=riter

riter2=riter-1.

rinf=riter2*prop(1)*delt

rmax=riter1*prop(1)*delt

c     determination of the number of integration subdomains

if(icpt.eq.0)nsdint=1

if(icpt.eq.1)call dnsdin(idees,iorps,nsdint)

c     determination of the area of the triangular

c     integration subdomains (asdt)

if(nsdint .eq. 1) go to 50

if(nsdint .eq. 2) then

asdt(1)=2.d+00

asdt(2)=2.d+00

else

asdt(1)=1.d+00

asdt(2)=2.d+00

asdt(3)=1.d+00

end if

c     initializing the coefficients of the H- und G-matrix

50 do inoel=1,nnoel

h1(inoel)=0.d+00

h2(inoel)=0.d+00

g1(inoel)=0.d+00

g2(inoel)=0.d+00

enddo

c     numerical integration

do 20 isdint=1,nsdint

ipgsdi=0

do 20 ise=1,nse

ipgse=0

do 20 igauss=1,npg(1)

do 20 jgauss=1,npg(2)

ipgse=ipgse+1

ipgsdi=ipgsdi+1

c     determination of the radius vector, its modulus and

c     its derivatives at the integration point "ipgsdi" from "isdint"

if(icpt.eq.0)then

call vectra(ielem,1,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

if(icpt.eq.1)then

call vectra(ielem,iorps,ipgsdi,isdint,shape,qsi,r,rmod,derr,

*              coord,inc)

endif

if(rmod .gt. rinf .and. rmod .lt. rmax)then

c     determination of the jacobi-determinant of the element surface at

c     the integration point "ipgsdi" from the "isdint"-th integration

c     subelement

if(icpt.eq.0)then

call jacre8(ielem,1,ipgsdi,isdint,deriv,djacb,vnu,

*                coord,inc,ndiint)

endif

if(icpt.eq.1)then

call jacre8(ielem,iorps,ipgsdi,isdint,deriv,djacb,vnu,

*                coord,inc,ndiint)

endif

c     determination of the derivative of radius "r" with respect to

c     the outward normal direction (dr/dn)

derrn=0.

do 40 idime=1,3

40   derrn=derrn + derr(idime)*vnu(idime)

c     determination of the "dsup1"

c     at the integration point (ipgsdi,isdint)

if(nsdint .eq. 1) then

dsup1= djacb*djacse*wg1b(igauss)*wg2b(jgauss)

else

dsup1= djacb*(1. + eta1(ise,ipgse))*

*         asdt(isdint)*(.25)*djacse*wg1b(igauss)*wg2b(jgauss)

endif

c     determination of auxiliary constants

ca1 = 1./(4.*pi*rmod**2)

ca2 = 1./(4.*pi*rmod)

ca3 = rmod/(prop(1)*delt)

c     integration of the time-dependent potential and flux kernels

ha1 = ca1*(riter-1.)*derrn

ha2 = -ca1*riter*derrn

if(ivafv.eq.0)then

c         boundary fluxes and potentials are linearly interpolated

ga1 = ca2*(1.- riter + ca3)

ga2 = ca2*(riter - ca3)

endif

if(ivafv.eq.1)then

c         constant time interpolation for boundary fluxes and linear

c         for potentials.

ga1 = 0.d+00

ga2 = ca2

endif

do 30 inoel=1,nnoel

if(icpt.eq.0)dsup=shape(ipgsdi,isdint,inoel,1)*dsup1

if(icpt.eq.1)dsup=shape(ipgsdi,isdint,inoel,iorps)*dsup1

h1(inoel) = h1(inoel) + ha1*dsup

h2(inoel) = h2(inoel) + ha2*dsup

if(lelno(ielem,inoel) .eq. 1)then

g1(inoel) = g1(inoel) + ga1*dsup

g2(inoel) = g2(inoel) + ga2*dsup

endif

30   continue

endif

20 continue

return

end

c     _________________

subroutine dnsdin(idees,iorps,nsdint)

c     subroutine for determination of the

c     number of integration subdomains

implicit double precision(a-h,o-z)

if(idees .eq. 1) then

do 10 inloi =1,7,2

if(inloi .eq. iorps) then

nsdint=2

go to 20

end if

10 continue

nsdint=3

20 continue

else

nsdint=1

end if

return

end

c     _________________

subroutine fators(b,pinf,psup,nmult,imult,ifip,jdiag,encl)

c     subroutine for obtaining the matrices A e B

c     (introduction of the boundary conditions) and

c     LU-decomposition of matrix A

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

common/num/    pi

dimension      b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               nmult(nno),imult(nno,5),ifip(nno),encl(nno)

do ino=1,nno

if(nmult(ino) .gt. 0)then

il=ino

do 35 ik=1,nmult(ino)

ic1=imult(ino,ik)

if(il .le. ic1)then

ip1=jdiag(ic1)+il-ic1

psup(ip1)=0.d+00

else

ip1=jdiag(il)+ic1-il

pinf(ip1)=0.d+00

endif

35   continue

endif

enddo

c     constant potential criterion for determination of

c     c(qsi)+hsg(qsi)

do ino=1,nno

if(infb.eq.0)then

ip=jdiag(ino)

psup(ip)=0.

endif

if(infb.eq.1)then

ip=jdiag(ino)

psup(ip)=1.

endif

if(infb.eq.2)then

ip=jdiag(ino)

psup(ip)=-encl(ino)

endif

if(ino .eq. 1) then

do jno=2,nno

ip=jdiag(ino)

ip2=jdiag(jno)+ino-jno

psup(ip)=psup(ip)-psup(ip2)

enddo

else

if(ino .eq. nno) then

do jno=1,nno-1

ip=jdiag(ino)

ip2=jdiag(ino)+jno-ino

psup(ip)=psup(ip)-pinf(ip2)

enddo

else

do jno=1,ino-1

ip=jdiag(ino)

ip2=jdiag(ino)+jno-ino

psup(ip)=psup(ip)-pinf(ip2)

enddo

do jno=ino+1,nno

ip=jdiag(ino)

ip2=jdiag(jno)+ino-jno

psup(ip)=psup(ip)-psup(ip2)

enddo

endif

endif

enddo

c     columns interchange of the matrices HSG and GSG

do il=1,nno

do jno=1,nno

if(ifip(jno).eq.1)then

if(jno .ge. il)then

ip=jdiag(jno)+il-jno

aa=psup(ip)

psup(ip)=-b(il,jno)

else

ip=jdiag(il)+jno-il

aa=pinf(ip)

pinf(ip)=-b(il,jno)

endif

b(il,jno)= -aa

endif

enddo

enddo

c     decomposition of matrix A

do 70 j=2,nno

c     effective size of the line or column j

ihj=jdiag(j)-jdiag(j-1)

c     column or line of the first non-null coefficient

c     of the line or column j

il=j-ihj+1

ia=il

if(il .eq. j)go to 70

if(ihj .eq. j)then

pinf(jdiag(j)+1-j)=pinf(jdiag(j)+1-j)/psup(jdiag(1))

ia=il+1

endif

do 80 i=ia,j-1

k=jdiag(j)+i-j

ihi=jdiag(i)-jdiag(i-1)

ic=i-ihi+1

ix=max0(il,ic)

if(ix .gt. i-1)go to 80

do 90 m=ix,i-1

k1=jdiag(j)+m-j

k2=jdiag(i)+m-i

pinf(k)=pinf(k)-pinf(k1)*psup(k2)

90 psup(k)=psup(k)-pinf(k2)*psup(k1)

80 pinf(k)=pinf(k)/psup(jdiag(i))

do 100 m=il,j-1

k=jdiag(j)+m-j

100 psup(jdiag(j))=psup(jdiag(j))-pinf(k)*psup(k)

70 continue

return

end

c     _________________

subroutine fatora(hsg,b,pinf,psup,nmult,imult,ifip,jdiag,encl)

c     subroutine for obtaining the matrices A e B

c     (introduction of the boundary conditions) and

c     LU-decomposition of matrix A

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

common/num/    pi

dimension      hsg(nno*nno),

*               b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               nmult(nno),imult(nno,5),ifip(nno),encl(nno)

do ino=1,nno

if(nmult(ino) .gt. 0)then

ll=(ino-1)*nno

do 35 ik=1,nmult(ino)

ic1=imult(ino,ik)

ip=jdiag(ino)

if(ino .le. ic1)then

ip1=jdiag(ic1)+ino-ic1

psup(ip)=psup(ip) + psup(ip1)

psup(ip1)=0.d+00

else

ip1=jdiag(ino)+ic1-ino

psup(ip)=psup(ip) + pinf(ip1)

pinf(ip1)=0.d+00

endif

35   hsg(ll+ic1)=0.d+00

endif

enddo

c     constant potential criterion for calculation of

c     c(qsi)+hsg(qsi)

do ino=1,nno

ll=(ino-1)*nno

if(infb .eq. 0) then

hsg(ll+ino)=0.

end if

if (infb .eq. 1) then

hsg(ll+ino)=1.

end if

if(infb .eq. 2) then

hsg(ll+ino)= -encl(ino)

endif

if(ino .eq. 1) then

do jno=2,nno

hsg(ll+ino)= hsg(ll+ino) - hsg(ll+jno)

enddo

else

if(ino .eq. nno) then

do jno=1,nno-1

hsg(ll+ino)=hsg(ll+ino) - hsg(ll+jno)

enddo

else

do jno=1,ino-1

hsg(ll+ino)=hsg(ll+ino) - hsg(ll+jno)

enddo

do jno=ino+1,nno

hsg(ll+ino)=hsg(ll+ino) - hsg(ll+jno)

enddo

endif

endif

enddo

c     addition of the coefficients c(qsi)+hsg(qsi) to the

c     correspondent positions of the lines of H2G

do ino=1,nno

ll=(ino-1)*nno

ip=jdiag(ino)

psup(ip)=psup(ip) + hsg(ll+ino)

enddo

c     columns interchange of the matrices H2G and G2G

do il=1,nno

do jno=1,nno

if(ifip(jno) .eq. 1)then

if(jno .ge. il)then

ip=jdiag(jno)+il-jno

aa=psup(ip)

psup(ip)=-b(il,jno)

else

ip=jdiag(il)+jno-il

aa=pinf(ip)

pinf(ip)=-b(il,jno)

endif

b(il,jno)= -aa

endif

enddo

enddo

c     decomposition of matrix A

do 70 j=2,nno

c     effective size of the line or column j

ihj=jdiag(j)-jdiag(j-1)

c     column or line of the first non-null coefficient

c     of the line or column j

il=j-ihj+1

ia=il

if(il .eq. j)go to 70

if(ihj .eq. j)then

pinf(jdiag(j)+1-j)=pinf(jdiag(j)+1-j)/psup(jdiag(1))

ia=il+1

end if

do 80 i=ia,j-1

k=jdiag(j)+i-j

ihi=jdiag(i)-jdiag(i-1)

ic=i-ihi+1

ix=max0(il,ic)

if(ix .gt. i-1)go to 80

do 90 m=ix,i-1

k1=jdiag(j)+m-j

k2=jdiag(i)+m-i

pinf(k)=pinf(k)-pinf(k1)*psup(k2)

90 psup(k)=psup(k)-pinf(k2)*psup(k1)

80 pinf(k)=pinf(k)/psup(jdiag(i))

do 100 m=il,j-1

k=jdiag(j)+m-j

100 psup(jdiag(j))=psup(jdiag(j))-pinf(k)*psup(k)

70 continue

return

end

c     _________________

subroutine vectos(b,pinf,psup,p,ifip,jdiag,u,pa)

c     subroutine for obtaining the right hand side vector of the

c     system of algebraic equations

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

dimension      u(nno),pa(nno),

*               b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               p(nno),ifip(nno)

c     multiplication "b*p(iter)"

if(ivols .eq.0)then

c      there is no volume source

do ino=1,nno

u(ino)=0.d+00

enddo

endif

do 20 jno=1,nno

do 20 ino=1,nno

20 u(ino)=u(ino) + b(ino,jno)*p(jno)

c     forward substitution

do 110 i=2,nno

ih=jdiag(i)-jdiag(i-1)

ic=i-ih+1

if(ic .eq. i)go to 110

do 120 j=ic,i-1

120 u(i)=u(i)-pinf(jdiag(i)+j-i)*u(j)

110 continue

c     backsubstitution

do 130 i=nno,2,-1

u(i)=u(i)/psup(jdiag(i))

ih=jdiag(i)-jdiag(i-1)

ic=i-ih+1

if(ic .eq. i)go to 130

do 140 j=ic,i-1

140 u(j)=u(j)-psup(jdiag(i)+j-i)*u(i)

130 continue

u(1)=u(1)/psup(1)

c     determination of the vectors storing the boundary values

do 50 ino=1,nno

if(ifip(ino) .eq. 1)then

pa(ino)=u(ino)

u(ino)=p(ino)

else

pa(ino)=p(ino)

end if

50 continue

return

end

c     __________________

subroutine outputs(u,pa,sp)

c     subroutine for outputting the results of stationary analyses

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/rw/     ire,iwr

dimension      u(nno),pa(nno),sp(npoi)

c     output of boundary values

write(iwr,230)

230 format(/5x,'potentials',/5x,'node',15x,'u')

do ino=1,nno

write(6,210)ino,u(ino)

enddo

write(iwr,240)

240 format(/5x,'fluxes ',/5x,'node',15x,'p')

do ino=1,nno

write(6,210)ino,pa(ino)

enddo

210 format(3x,i6,3d17.8)

c     output of potetials at internal points

if (npoi.ne.0)then

write(iwr,250)

250  format(/5x,'potentials at internal points',/5x,'node',15x,'u')

do ipoi=1,npoi

write(6,210)ipoi,sp(ipoi)

enddo

endif

return

end

c     _________________

subroutine vector(b,pinf,psup,h1g,g1g,h2g,g2g,

*                  p,p0,omega,temp0,itvar,ifip,jdiag,nste3,iter,

*                  itlim,nmplim,itblim,u,ua,pa,fc)

c     subroutine for obtaining the right hand side vector of the

c     system of algebraic equations and evaluation of the time

c     response at the current time step

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/propf/  prop(5),npropb

dimension      u(nno),pa(nno),ua(nno),fc(nno),

*               h2g(nno),g2g(nno),

*               b(nno,nno),pinf((nno*(nno+1))/2),

*               psup((nno*(nno+1))/2),jdiag(nno),

*               h1g(nno*nno*nste3),

*               g1g(nno*nno*nste3),p(nno),p0(nno),omega(nno),

*               temp0(nno),itvar(nno),ifip(nno)

x1=dfloat(iter)/dfloat(nste3)

x2=dint(x1)

x3=x1-x2

if(x3 .eq. 0.)then

nbl=idint(x1)

else

nbl=idint(x2) + 1

endif

itb=iter - (nbl-1)*nste3

c     determination of the time variation of the flux function

call varfc(p,p0,omega,temp0,itvar,ifip,fc,iter)

c     multiplication "b*fc(iter)"

if(ivols.eq.0)then

c      then there is no volume source

do ino=1,nno

u(ino)=0.d+00

enddo

endif

do jno=1,nno

do ino=1,nno

u(ino)=u(ino) + b(ino,jno)*fc(jno)

enddo

enddo

if(iter .eq. 1)go to 40

if((iter .le. nste3) .or. (itlim.le.nste3))then

if(iter .le. itlim)then

itf=1

else

itf=iter-itlim

endif

do 200 it=(iter-1),itf,-1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

it2=iter-it

do 200 ino=1,nno

j1=(it2-1)*nno*nno + (ino-1)*nno

do 200 jno=1,nno

j1=j1+1

200   u(ino)=u(ino) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

if((iter.gt.nste3) .and. (itlim.gt.nste3))then

irek1=0

if(iter .le. itlim)then

nmpl=(nbl-2)*nste3 + itb

else

nmpl=nmplim

endif

do 190 it=(iter-1),(iter-nmpl),-1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

do 190 ino=1,nno

irek1=irek1+1

read(unit=1,rec=irek1)(h2g(jno),jno=1,nno)

read(unit=3,rec=irek1)(g2g(jno),jno=1,nno)

i1=ino

do 190 jno=1,nno

190   u(ino)=u(ino) + g2g(jno)*pa(jno) - h2g(jno)*ua(jno)

iti=iter-nmpl-1

if(iter .le. itlim)then

itf=itb

itf0=1

itb2=itb

else

itf=iti-(nste3-itblim)+1

itf0=itf-itblim

itb2=itblim

endif

if(itb2 .lt. nste3)then

it2=itb2-1

do 210 it=iti,itf,-1

it2=it2+1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

ipp=it2*nno*nno

do 210 ino=1,nno

j1=ipp + (ino-1)*nno

do 210 jno=1,nno

j1=j1+1

210     u(ino)=u(ino) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

if(itb2 .gt. 1)then

it2=0

do 220 it=itf-1,itf0,-1

it2=it2+1

read(unit=8,rec=it)(ua(ino),ino=1,nno)

read(unit=9,rec=it)(pa(ino),ino=1,nno)

ipp=(it2-1)*nno*nno

do 220 ino=1,nno

j1=ipp + (ino-1)*nno

do 220 jno=1,nno

j1=j1+1

220     u(ino)=u(ino) + g1g(j1)*pa(jno) - h1g(j1)*ua(jno)

endif

endif

c     consideration of the initial conditions (fluxes)

c     initial potentials can still not be taken into account

40 if(iter .le. itlim)then

do 150 ino=1,nno

j1=(itb-1)*nno*nno + (ino-1)*nno

do 150 jno=1,nno

j1=j1+1

150   u(ino)=u(ino) + g1g(j1)*p0(jno)

endif

c     evaluation of the time response at the current time step

c     forward substitution

do 110 i=2,nno

ih=jdiag(i)-jdiag(i-1)

ic=i-ih+1

if(ic .eq. i)go to 110

do 120 j=ic,i-1

120 u(i)=u(i)-pinf(jdiag(i)+j-i)*u(j)

110 continue

c     backsubstitution

do 130 i=nno,2,-1

u(i)=u(i)/psup(jdiag(i))

ih=jdiag(i)-jdiag(i-1)

ic=i-ih+1

if(ic .eq. i)go to 130

do 140 j=ic,i-1

140 u(j)=u(j)-psup(jdiag(i)+j-i)*u(i)

130 continue

u(1)=u(1)/psup(1)

do 50 ino=1,nno

if(ifip(ino) .eq. 1)then

pa(ino)=u(ino)

u(ino)=fc(ino)

else

pa(ino)=fc(ino)

end if

50 continue

write(unit=8,rec=iter)(u(ino),ino=1,nno)

write(unit=9,rec=iter)(pa(ino),ino=1,nno)

return

end

c     _________________

subroutine output(ifip,u,pa,ndnopl,sp)

c     subroutine for output of the analysis results

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

common/temp/   delt,niter

common/vafv/   ivafv

common/rw/     ire,iwr

dimension      u(nno),pa(nno),ifip(nno),ndnopl(nnopl),

*               sp(npoi)

if(nnopl.ne.0 .or. npoi.ne.0)open(unit=11,file='replot.out')

c     storing data for plotting

write(unit=11,fmt=200)

if(nnopl .ne. 0)then

do inopl=1,nnopl

igl=ndnopl(inopl)

if(ifip(igl).eq.0)then

write(unit=11,fmt=205)ndnopl(inopl)

do iter=1,niter

tp=dfloat(iter)*delt

read(unit=8,rec=iter)(u(ino),ino=1,nno)

write(unit=11,fmt=230)tp,u(igl)

enddo

endif

if(ifip(igl).eq.1) then

write(unit=11,fmt=215)ndnopl(inopl)

do iter=1,niter

tp=dfloat(iter)*delt

read(unit=9,rec=iter)(pa(ino),ino=1,nno)

write(unit=11,fmt=230)tp,pa(igl)

enddo

endif

enddo

endif

if(npoi .ne. 0)then

write(unit=11,fmt=220)

do ipoi=1,npoi

igl=ipoi

write(unit=11,fmt=205)ipoi

do iter=1,niter

tp=dfloat(iter)*delt

read(unit=22,rec=iter)(sp(i),i=1,npoi)

write(unit=11,fmt=230)tp,sp(igl)

enddo

enddo

endif

200 format('===============',

*	    /'boundary values')

205 format(/1x,'node   =',i5,3x,'(potential)')

215 format(/1x,'node   =',i5,3x,'(normal flux)')

220 format(/,'=============================',

*	      /'quantities at internal points')

230 format(1x,d15.8,3x,d15.8)

return

end

c     ________________

subroutine dmaxe(dmax,coor1,nn)

c     subroutine for determination of the maximum distance

c     between "nn" nodes of a same group

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

dimension      x1(3),x2(3),coor1(nn,ndime)

dmax=0.d+00

do i1=1,nn-1

do idime=1,ndime

x1(idime)=coor1(i1,idime)

enddo

do j2=i1+1,nn

do idime=1,ndime

x2(idime)=coor1(j2,idime)

enddo

d=0.

do i=1,ndime

d=d+(x2(i)-x1(i))**2

enddo

d=dsqrt(d)

if(d .gt. dmax)dmax=d

enddo

enddo

write(6,20)dmax

20 format(5x,'dmax = ',d12.5)

return

end

c     ________________

subroutine dmaxd(dmax,coor1,coor2,nn1,nn2)

c     subroutine for determination of the maximum distance

c     between nodes of two different groups g1 and g2 of nodes

implicit double precision(a-h,o-z)

common/contrc/ infb,nno,ne,nnoel,ndime,npoi,

*               ntp,nnoe,nee,nnopl,ivols,nspo

dimension      coor1(nn1,ndime),coor2(nn2,ndime),

*               x1(3),x2(3)

dmax=0.d+00

do i1=1,nn1

do idime=1,ndime

x1(idime)=coor1(i1,idime)

enddo

do j2=1,nn2

do idime=1,ndime

x2(idime)=coor2(j2,idime)

enddo

d=0.

do i=1,ndime

d=d+(x2(i)-x1(i))**2

enddo

d=dsqrt(d)

if(d .gt. dmax)dmax=d

enddo

enddo

write(6,20)dmax

20 format(5x,'dmax = ',d12.5)

return

end