In recent versions of LAMMPS, per-contact ‘shear’ history is output using the following syntax:

compute 	1 all pair/local p1 p2 p3 p4 p5 p6 p7 p8 p9 {…}

This is not even remotely self-explanatory. The purpose of this document is to explain (1) how many of these per-contact outputs are available in your simulation and (2) what each ‘p*’ entry means.

# How many of these p* entries do I have?

It depends whether or not rolling resistance and/or energy tracing are active:

| Rolling resistance | Energy tracing | Number of p* entries |
| --- | --- | --- |
| Inactive | Inactive | 14 |
| Inactive | Active | 18 |
| Active | Inactive | 41 |
| Active | Active | 45 |

# What do they mean?

We can divide the entries into three groups (base, rolling resistance, energy tracing) and discuss them separately. The order of these groups is always the same:

- Base (14 p* entries)

- Energy tracing, if active (4 p* entries)

- Rolling resistance, if active (27 p* entries)

## 1 - Base (14 p* entries)

- x-component of shear force

- y-component of shear force

- z-component of shear force

- ccel = [magnitude of normal force]/[distance between particle centres]

- tag of first particle in contact

- tag of second particle in contact

- x position of first particle

- y position of first particle

- z position of first particle

- radius of first particle

- x position of second particle

- y position of second particle

- z position of second particle

- radius of second particle

## 2 - Energy tracing, if active (4 p* entries)

- Energy dissipated by friction at the contact

- Normal component of strain energy

- Tangential component of strain energy

- [Vacant]

## 3 – Rolling resistance, if active (27 p* entries)

A comprehensive explanation of these variables is given in Chapter 6 of Xin Huang’s PhD thesis.

- x-component of rolling contribution to relative displacement increment, dUr

- y-component of rolling contribution to relative displacement increment, dUr

- z-component of rolling contribution to relative displacement increment, dUr

- x-component of accumulated rolling contribution to relative displacement, Ur

- y-component of accumulated rolling contribution to relative displacement, Ur

- z-component of accumulated rolling contribution to relative displacement, Ur

- x-component of sliding contribution to relative displacement increment, dUs

- y-component of sliding contribution to relative displacement increment, dUs

- z-component of sliding contribution to relative displacement increment, dUs

- x-component of accumulated sliding contribution to relative displacement, Us

- y-component of accumulated sliding contribution to relative displacement, Us

- z-component of accumulated sliding contribution to relative displacement, Us

- x-component of moment increment due to rolling resistance in local coordinates, dMx

- y-component of moment increment due to rolling resistance in local coordinates, dMy

- z-component of moment increment due to twisting resistance in local coordinates, dMz

- Accumulated value of dMx in local coordinates

- Accumulated value of dMy in local coordinates

- Accumulated value of dMz in local coordinates

- x-component of moment increment due to rolling resistance in global coordinates, dMX

- y-component of moment increment due to rolling resistance in global coordinates, dMY

- z-component of moment increment due to twisting resistance in global coordinates, dMZ

- Accumulated value of dMX in global coordinates

- Accumulated value of dMY in global coordinates

- Accumulated value of dMZ in global coordinates

- Equivalent area tangential contact stiffness, ks{bar}

- Stored (κ + 1) for rolling resistance model

- Stored (κ + 1) for twisting resistance model