# MTDS Library Programmer's Reference Manual

Rev: 2016-09-26

## Introduction

This document provides an overview of the Multi-Touch Display System (MTDS) and documentation on the MTDS library. The MTDS Library is the core library used to build applications programs using the MTDS system.

The MTDS Library documentation includes a description of core concepts used by the system in general and the library in particular and then provides detailed documentation on all of the API functions used to access the system facilities.

## Use of the MTDS Library

In order to use the MTDS library, the library header file mtds.h must be included as follows:

#include <mtds.h>

The mtds.h header file (or other header files included by it) define the various object classes, structure types and symbols that make up the interface to the MTDS system.

In addition to mtds.h, the header file ProtoDefs.h defines symbols related to the communications protocol between the host library and the display board. In particular, the symbols defining the error codes returned by the firmware running on the display board are defined in ProtoDefs.h. In order to have access to the symbols defining the error codes, ProtoDefs.h may also be included in the user's program.

The mtds.h header file defines the objects classes used by the system. Most of the other data structures and symbols used by the system are defined in the header file MtdsDefs.h. Although many of these symbols and data structures are described in this document, reference to MtdsDef.h is required to find the full range of symbols defined for the system.

The primary programmatic interface to the MTDS system is accomplished by called object member functions, or methods, defined by the object classes declared in mtds.h. There are three primary object classes defined: the MTDS object class defines the interface to the graphics system and basic control and operation of the display hardware; the MTFS object class defines the interface to the file system on a micro-SD card installed in the SD card connector on the display board; and the MTWS class defines the interface to the windowing system maintained by the firmware running on the display board.

The library defines three object instances, one for each of the primary object classes defined for the system. These object instances are declared as follows:

MTDS mtds;		// object used for access to graphics and basic services

MTFS mtfs;		// object used for access to file system services

MTWS mtws;		// object used to access to windowing system

The facilities of the MTDS system are accessed by making calls to object member functions using one of these three objects. For example, the graphics system function to set a pixel on the graphics display could be called as follows:

mtds.SetPixel(hdsDisp, 10, 20, clrWhite)

See the sections below describing the MTDS Class Interface Functions, MTFS Class Interface Functions and the MTWS Class Interface Functions for detailed information about the specific functions and their use.

## System Architecture

The Multi-Touch Display System is made up of the following parts:

Host Board and User Application Program

MyDisp Library.

MTDS Library

Display Board

MTDS Firmware running on the display board.

Communications interface and protocol between host board and display board

Although not technically part of the MTDS system, the host board is the hardware being used to run an applications program making use of the system. The applications program is built using one or more of the libraries that are part of the system.

The MyDisp Library is an applications programer's library that provides a simplified, high level interface to the system for building applications running on the host board.

The MTDS library is the core applications programming library. It provides access to all of the features of the system. All applications programs using the MTDS system make use of this library, either directly or indirectly. The MyDisp library uses the MTDS library to access system facilities.

The display board contains the physical display panel, display controller, touch controller and the SD card interface. It also contains a microcontroller running the MTDS firmware.

The MTDS firmware is a program that runs on the microcontroller on the display board. This firmware contains the control software to manage the display and touch controller, the graphics engine responsible for rendering the graphical elements, the file system code for access to the file system on the SD card, the windowing system engine, and the communications code for the interface with the host board.

The host applications program communicates (via the interface functions in the various MTDS interface object classes) with the firmware running on the display board. The physical interface is accomplished using an SPI port. The logical interface is a command/response protocol defined and implemented by the low level communications management code in the MTDS library and the MTDS firmware running on the display board.

The communications interface involves sending command packets and data packets from the host to the display board and by sending status packets and data packets from the display board back to the host board. In virtually all cases, calling one of the class functions in one of the MTDS system interface objects (e.g. the mtds object instance) involves the host sending a command packet to the display board, waiting for the display board to process the command and then the host receiving a status packet back from the display board. The command packet contains a command class code, a command code and any parameters required by the command. The status packet contains a status code indicating successful completion of the command or an error status code. It also contains return values being returned from the display board to the host.

In some cases, (for example, when writing to a file, or when reading from a file) the communications interface involves sending one or more data packets from the host to the display or from the display to the host. In these cases, a command packet is sent by the host, one or more data packets are sent or received, and then a status packet is received by the host.

The communications between the host application program and the display board firmware is managed by low level code contained in the MTDS library.

## System Startup and Initialization

At system startup, the display board automatically performs its power-on reset processing and initializes itself. Before the user sketch can start to make use of the MTDS system, the host runtime library must be initialized and communications with the display board must be established.

One of the begin() function variants must be called before most other MTDS functions may be called. This function initializes the library run-time system and then establishes communications between the host board and the display board. It returns true if it is able to successfully establish contact with the display board.

Once initialization is successful and communications has been established with the display board, begin() will return true. The begin() function makes several attempts to establish communications with the display board before returning failure. In some cases, mostly depending on how fast the host system board is, it is possible that the display board will not have completed its reset initialization before the begin() function gives up trying to establish communications.

If begin() returns false, the host sketch should wait a few seconds and then try again. This can be repeated several times before deciding that the display system is really not working and giving up.

If there is some reason to stop using the MTDS system, the end() function can be called to disconnect from the display board. Once end() has been called, it is necessary to call begin() again before again using any of the MTDS system functions.

Most of the basic MTDS system properties are set automatically by the display board when it powers on. The display board will power on with the following properties set:

Display is enabled

Backlight is on and brightness is set at 60%

Display orientation is set to portrait mode

The display is cleared to clrBlack

Touch panel sensitivity is set to 2

Touch movement threshold is set to 5 in both X and Y directions

## Core Concepts Used by the MTDS System

The following describes some of the key concepts used by the MTDS system. This provides general background information about how the system functions and provides a context for understanding the sections describing the interface functions and their operation.

### Handles and Dynamic Objects

There are a number of object types that are defined and used by the MTDS library and the corresponding firmware running on the display board. These objects can be dynamically created and destroyed by the display board firmware and exist within the dynamic memory heap managed by the display board processor. A handle is a token or “magic number” used to refer to one of these objects. A function that creates or allocates one of these dynamic objects will return the handle to the created object. Functions that operate on one of these dynamic objects will take the handle of the object to be operated on as one of its parameters. Once the object is no longer needed by the programmer's sketch, it should be released, closed, or deleted depending on the particular object type.

The mtds.h header file defines C++ data types for each of the following handle types. A variable of one of the these types can be declared as show in the following examples:

HDS	hdsDisp;	// variable to hold HDS for the display

HBMP	hbmpBtn;	// bitmap to hold the image for a button

The following handle types are defined by the MTDS library:

HDS	Drawing State handle. A drawing state object contains all of the contextual information required by the graphics system to render graphics. All rendering of graphics within the MTDS environment involves obtaining, initializing, using and then releasing an HDS. See the section on Drawing State Objects for more detailed information.

HBMP	Bitmap handle. A bitmap handle refers to a bitmap maintained by the display board firmware. In general, this means a bitmap that exists within the dynamic memory heap on the display board. There is one special bitmap handle that refers to the bitmap corresponding to the frame buffer of the display itself. See the section on Bitmap Objects for more detailed information about bitmaps.

HBR	Brush handle. A brush handle refers to brush object. A brush is a special type of bitmap that is used for area fills. There are a small number of predefined brushes, but most brushes are created by the programmer's sketch. Predefined brushes have handles that are statically defined symbols in the mtds.h header file. See the section on Brush Objects for more detailed information about brushes.

HWIN	Window handle. The MTDS library supports a rudimentary windowing system. The window handle refers to a window within this windowing system. See the section on the Windowing System for more detailed information.

HFNT	Font handle. A font is a collection of bitmap, or raster, patterns used to draw text. The MTDS system only supports bitmapped or “raster” fonts, it does not support vector fonts. The MTDS font format is a simple internal format and is not the same as the raster font format used by other systems such as Microsoft Windows or Apple OS X. There are a number of predefined fonts stored in the flash memory of the display shield processor. These predefined fonts have handles that are statically defined in the mtds.h header file. Dynamically defined fonts are loaded from font files stored on the SD card. See the section on Font Objects for more detailed information.

HMEM	Memory handle. This is a handle to a dynamically allocated memory object that exists within the dynamic memory heap managed by the processor on the display board. These memory objects provide temporary, scratch-pad, memory that can be used by the sketch programmer for temporary storage of any kind of data needed.

HDIR	Directory handle. This is a handle to a directory within the file system on an SD card plugged into the micro-SD connector on the display board. The primary use for these is for searching through the files within a directory.

HFIL	File handle. This is a handle to an open file within the file system on an SD card plugged into the connector on the display board. These are used when reading/writing data on the file or querying some kinds of information about the file.

### Drawing State Object

One of the fundamental concepts in the MTDS system related to drawing, or rendering, graphical elements is that of a drawing state, or DS. The drawing state encapsulates the context, or the current settings, of all of the controllable properties that are used for rendering by the graphics engine. These properties are such things as the current foreground and background colors, the current pen pattern, the currently selected brush used for fills and so on. It also, in particular, includes the handle specifying the bitmap that is going to be drawn upon. The drawing state is stored in a dynamic object, called a DS, managed and maintained by the graphics engine firmware running on the processor on the display board.

The following properties are stored in the DS:

Foreground color

Background color

Transparency color

Background transparency mode

Bitmap drawing intensity (used by DrawBitmap())

Pen pattern for linear elements (line, arc, perimeter of ellipse, etc)

Brush for area fill (interior of ellipse, rectangle, etc)

Font for drawing text

Raster op specifying the combining mode used for drawing linear objects and text

Coordinate of the current drawing position.

Clipping rectangle

Handle to the bitmap used for the drawing surface

The MTDS system supports multiple simultaneous sets of the drawing state, or context, by providing multiple drawing state objects. These drawing state objects are maintained by the firmware running on the display board and are referenced by a handle, or HDS. There is a cache of DS objects maintained by the system. The GetDS() function is called to retrieve the next available one from the cache. GetDS() will return the handle (HDS) of the next available DS object and mark the object as in-use. When a DS is no longer needed, it should released and returned to the cache by passing its handle to ReleaseDs().

One of the key properties maintained by a DS object is the handle to the bitmap used for the “drawing surface”. This is the bitmap on which all rendering will occur when using the corresponding HDS. An HDS obtained by calling GetDs() will initially have its drawing surface set to null. Before any rendering can be done using the DS, it is necessary to assign a drawing surface bitmap to it by calling the SetDrawingSurface() function. Normally, this will be the handle of a memory bitmap obtained from CreateBitmap() or LoadBitmap(), but it can also be the display bitmap.

There is one distinguished DS maintained by the system that refers to the physical display. The handle to the display DS can be obtained by calling GetDisplayDs(). When the handle to the display DS is obtained by calling GetDisplayDs(), its drawing surface will have already been set to the display bitmap. Once the display DS is no longer needed, it should be released back to the system by passing its handle to ReleaseDs(). Once the HDS for the display has been obtained by calling GetDisplayDs(), subsequent calls to GetDisplayDs() will fail until the display HDS has been released by calling ReleaseDs();

The display bitmap can be assigned as the drawing surface for an HDS obtained by calling GetDs(). The function GetDisplayBitmap() returns the bitmap handle of the display bitmap for this purpose. This makes it possible to have multiple simultaneous contexts for drawing directly onto the display. It is also possible to call GetDisplayDS() to obtain the HDS for the display, then call GetDrawingSurface() to obtain the display bitmap handle from it, and then to call SetDrawingSurface() to set the display bitmap as the drawing surface bitmap for another DS.

It is expected that there can be multiple drawing state objects simultaneously set to have the same bitmap as the drawing surface. This allows multiple sets of drawing properties to be maintained simultaneously for drawing onto a given bitmap.

The procedure for drawing onto an off-screen bitmap (memory bitmap) is as follows:

Create the off-screen bitmap using either CreateBitmap() or LoadBitmap()

Obtain a handle to a drawing state object

Assign the bitmap to the DS by calling SetDrawingSurface()

Configure other DS properties as desired

Call rendering functions passing in the HDS

Once the HDS is no longer needed, it must be released back to the system by calling ReleaseDS(). Once the off-screen bitmap is no longer needed, the memory resources it uses must be released back to the system by calling DestroyBitmap(). A bitmap should not be destroyed if it is assigned as the drawing surface for an active HDS. Before destroying the bitmap, the HDS should be released, or the bitmap should be unassigned by calling SetDrawingSurface() passing in either hbmpNull or the value 0 as the handle to the drawing surface bitmap. The display bitmap should not be destroyed, although, the system will ignore any attempt to destroy it.

When drawing onto an off-screen bitmap, all drawing is clipped to the boundaries of the drawing surface bitmap. When drawing onto the display, all drawing is clipped to the boundaries of the display.

When an HDS is obtained from the system either by calling GetDs() or GetDisplayDs(), its state is initialized to the system default state. If a particular set of drawing properties is going to be used frequently, an HDS can be obtained and configured with the desired set of properties and then stored in a global variable for use whenever required.

The DS cache is limited in size, making DS objects a scarce resource in the system. If the DS cache becomes exhausted GetDs() will return an error. To prevent exhaustion of the DS cache, any time a DS is allocated within a function for limited time use, it must be released before the function returns, or it will be lost to the system until the next time the display hardware is reset.

When an HDS is obtained by calling GetDs(), the state of the DS is initialized as follows:

Foreground Color			clrWhite

Background Color			clrBlack

Transparency Color			clrBlack

Background Transparency Mode	bkOpaque

Bitmap drawing intensity		100%

Pen Pattern				penSolid

Drawing Mode			drwCopyPen

Fill Brush				hbrWhite

Drawing Surface Bitmap		null

Font					null

Current Position			0, 0

### Coordinate System and Current Position

The MTDS library uses signed 16-bit integers to represent the X and Y coordinates used in the system. This allows the coordinate system to extend between +/- 32,767 in the X and Y directions. This represents a range of possible coordinate values that greatly exceeds the screen dimensions or the dimensions of the largest possible off-screen bitmap. Any drawing that occurs outside the bounding rectangle of the containing bitmap (either memory bitmap or the physical display) will be clipped.

The MTDS library uses a left handed coordinate system with the origin (0,0) at the upper left corner of the display. X values increase to the right, Y values increase going down. The same is true for off-screen bitmaps.

The graphics engine in the MTDS system maintains the coordinates of the “current position” as part of the data in the drawing state object. Many of the rendering functions use this current position as the starting coordinate when drawing. The current position associated with a DS is set using the MoveTo() function and can be obtained using the GetCurPos() function.

The model for drawing with functions that use the current position is to use the MoveTo() function to set the current position and then call the drawing function that will use that current position. For example, to draw a line, use MoveTo() to set the current position to the starting point of the line followed by LineTo() specifying the ending point of the line. The LineTo() function updates the current position to the endpoint that was drawn to. In this way, a series of connected lines could be drawn by calling MoveTo(hds, x1,y1) followed by LineTo(hds,x2,y2), LineTo(hds,x3,y3), and so on.

Functions whose name ends in To, update the current position to the last point drawn. For example, LineTo(hds,x,y) draws a line from the current position to x,y updating the current position to be x,y. The function Line(hds,x,y) draws a line from the current position to x,y, but leaves the current position unchanged.

### Color Values

The MTDS system uses two different types of color values. In most cases, a color value is an unsigned 32-bit integer specifying the Red, Green and Blue components making up the color. In a few cases, a color value is a 1-bit value specifying color on or color off. Depending on the circumstances, the 1-bit color value can refer to white for color on and black for color off or, it can refer to foreground color for color on and background color for color off.

Generally, a color value will be a 32-bit unsigned integer. The MTDS system uses 8-bits to represent the red component, 8-bits to represent the green component and 8-bits to represent the blue component of a color (8:8:8 color values). This allows the expression of more than 16 million distinct colors. The R ,G and B color components are stored in the lower 24 bits of a 32-bit unsigned int. The upper 8-bits are stored as 0 and are not used.

The use of 32-bit RGB values allows for the expression of color in a generic way without regard to the actual color space supported by a given hardware implementation. In most cases, the underlying display hardware supports a more restrictive color space. In particular, the Multi-Touch Display Shield and the PmodMTDS products use a display that supports 16-bit pixels stored using 5-bits for Red, 6-bits for Green and 5-bits for Blue (5:6:5). This allows specifying 65536 realizable colors.

Color values are automatically translated between the generic 32-bit color values used externally and the display dependent 16-bit color values used internally by the system. When translating an external color value to an internal color value, the low order bits of each color component are truncated. When translating an internal color value to an external color value, the low order bits of each color component are set to 0. The function GetNearestColor() can be used to obtain the nearest realizable color on the specific display board for any generic 32-bit color value.

Macros are provided for creating a color value specifying the red, green and blue components or for extracting the red, green or blue components from a color value. See the descriptions below for documentation on interface functions and macros.

The MTDS header files define symbols for some common colors, but any unsigned 32-bit integer value is a valid color value, although not necessarily realizable on any specific display hardware:

clrBlack		0x00000000			black

clrDkGray		0x00606060			dark gray

clrMedGray		0x00808080			medium gray

clrLtGray		0x00C0C0C0			light gray

clrWhite		0x00FFFFFF			white

clrRed		0x00FF0000			red

clrDkRed		0x00800000			dark red

clrGreen		0x0000FF00			green

clrDkGreen		0x00008000			dark green

clrBlue		0x000000FF			blue

clrDkBlue		0x00000080			dark blue

clrYellow		0x00FFFF00			yellow

clrDkYellow		0x00808000			dark yellow

clrCyan		0x0000FFFF			cyan

clrDkCyan		0x00008080			dark cyan

clrMagenta		0x00FF00FF			magenta

clrDkMagenta	0x00800080			dark magenta

clrLtBlueGray	0x00B8B8FF			light blue gray

clrMedBlueGray	0x007878FF			medium blue gray

### Foreground Color and Background Color

The foreground and background colors are used when drawing linear type graphical elements (lines, arcs, the perimeters of ellipses and rectangles) and text. When drawing one of these elements, pixels that are set (i.e. '1') in the pen or the character font correspond to the foreground, and are drawn with the foreground color. Pixels in the pen or font that are not set ('0') correspond to the background and are drawn using the background color.

The current foreground and background colors are stored as part of the drawing context maintained in a DS. When a handle to a DS is first obtained by calling GetDs() or GetDisplayDs() the foreground color is initialized to clrWhite and the background color to clrBlack. The foreground and background colors can be set by calling SetFgColor() and SetBgColor(). The current settings of the foreground and background colors can be obtained by calling GetFgColor() and GetBgColor().

### Pen Pattern and Background Transparency Mode

A pen pattern is an unsigned 16-bit integer specifying which pixels to draw when drawing a linear type graphics element. When drawing one of these type of objects, the pen pattern currently selected in the HDS is used.

When drawing, the '1' bits (bits that are set) in the pen correspond to foreground pixels and are drawn with the currently selected foreground color. Bits that are '0' (not set) correspond to background pixels. The drawing of background pixels depends on the current background transparency mode.

When the background transparency mode is set to bkOpaque, background pixels are drawn using the currently selected background color. If the background transparency mode is bkTransparent, the background pixels are not drawn. This allows the background pixels to “show through” what is being drawn. The transparency mode is set to bkOpaque by default.

The pen pattern is advanced by one bit position for each pixel position in the graphical element being drawn. Once the complete pattern has been drawn, it repeats as more pixels are drawn. The pen pattern is reset to the beginning when MoveTo() or SetPen() is called. Thus, if a call is made to MoveTo() followed by a series of calls to LineTo(), the pen pattern will cycle through the series of line segment drawn, forming a continuous pattern, and thus causing the pattern to be drawn “around the corners”.

The pen is used by the functions Line(), LineTo(), PolyLine(), PolyLineTo(), Arc(), ArcTo() and FrameRect(). It is also used by the functions Ellipse(), Rectangle(), and RoundRect() when drawing the perimeters of these elements. Rectangle() and RoundRect() draw from the upper left corner, clockwise around the perimeter. Ellipse() draws from the upper center of the element, clockwise around the perimeter. If it is desired to not have the perimeter drawn for these elements, the pen can be set to penNull (or the value 0). When the pen is set to null, these functions do not draw the perimeter of the element.

The background transparency mode also applies when drawing text. When drawing text, the '1' pixels in the font are drawn with the foreground color, the '0' pixels are drawn with the background color or not drawn depending on the background transparency mode. Similarly to how linear object are drawn, if the background transparency mode is set to bkTransparent, the background will “show through” the text.

In addition to its function with drawing linear graphical elements and text, the background transparency mode is also used when rendering bitmaps using the function DrawBitmap(). When using DrawBitmap() with transparency set to bkOpaque, all pixels of the source bitmap are drawn. When the transparency mode is set to bkTransparent, source pixels whose color matches the currently selected transparency color are not drawn. Similarly to the effect with linear graphical elements and text, this allows the background to “show through” when rendering a bitmap using DrawBitmap(). The transparency color is set using SetTransColor(). The transparency color is set to clrBlack by default.

The MTDS header files define symbols for some common pen patterns, but any unsigned 16-bit value can be used for the pen:

penNull		0x0000

penSolid		0xFFFF

penDash		0xC3C3

penDot		0x9999

penDashDot		0xF18F

penDashDotDash	0xE667

penDashDotDot	0xC663

### Transparency Color

The transparency color is used by the DrawBitmap() function. The DrawBitmap() function is used to copy pixels from an off-screen, memory, bitmap onto the display, or onto another off-screen bitmap. When copying the bitmap data, when transparency mode is set to bkTransparent, pixels whose color matches the current transparency color are not copied, thus leaving the existing pixels in the destination unchanged in those locations. This allows the previous content of those pixels in the destination to “show through” the bitmap being drawn.

The current transparency color is stored as part of the drawing context maintained in a DS. When a handle to a DS is first obtained by calling GetDs() or GetDisplayDs() the transparency color is initialized to clrBlack. The transparency color can be set by calling SetTransColor(). The current setting of the transparency color can be obtained by calling GetTransColor().

### Drawing Intensity

An intensity can be specified that will be used when a bitmap is drawn using DrawBitmap(). The intensity is set using the SetIntensity() function, and specifies a percentage value used to scale the brightness of each pixel drawn when rendering the bitmap. The default value for intensity is 100, indicating that bitmap images are to be drawn at 100% intensity. The allowed range of values is 0 to 200. When rendering the bitmap, the brightness of each pixel is scaled by the intensity value. For example if intensity is set to 50 (meaning 50% intensity), each pixel will be drawn at half the normal brightness.

Because the red, green and blue color components within a pixel are stored using a small number of bits with a very limited range of values, color shifts and saturation can occur when using intensity values very far from 100. Intensity values outside the range of 50-150 will generally cause the colors to clamp to black or saturate to white, or cause extreme color shifts.

### Bitmap Objects

A bitmap is a graphical object made up of a rectangular array of picture elements, or pixels. Bitmaps are used to store image data, such as photographs, icons, the faces of buttons and so on. Bitmaps are always rectangular in shape and are described by their width, height and pixel format. The width and height are simply the number pixels in the X, or horizontal direction, and Y, or vertical direction. The pixel format determines the number of colors available for each pixel in the bitmap. Bitmaps are uniform in that all pixels in the bitmap have the same pixel format. Bitmaps are always stored in the internal color format used by the display hardware. The MTDS system currently supports 16-bit per pixel (16-bpp) color bitmaps and 1-bit per pixel (1-bpp) monochrome bitmaps.

There is one special bitmap object defined by the system, the display bitmap, used to represent the frame buffer memory in the display controller. The contents of this bitmap determine what appears on the physical display. Other bitmaps, called memory bitmaps or off-screen bitmaps, are stored in memory managed by the firmware running on the display board. Off-screen bitmaps are never directly visible, but their contents can be copied onto the display quickly. It is also possible to copy portions of the display into off-screen bitmaps, as well. Off-screen color bitmaps (16-bpp) are stored in the same pixel color format as the display frame buffer.

All MTDS library functions that draw graphical elements, render them into bitmaps; either the display bitmap or off-screen bitmaps in display board memory. All graphics drawing operations operate equivalently whether rendering directly onto the display or onto an off-screen, memory bitmap. In order to draw onto a bitmap, its handle is assigned as the drawing surface for a drawing state object (DS) by calling SetDrawingSurface(). Once this has been done, all drawing done using that DS will occur on the specified bitmap.

Each bitmap exists within its own signed, 16-bit coordinate system. This coordinate system defines a virtual coordinate space spanning +/-32K in X and +/-32K in Y. The actual bitmap is a “viewport” onto this virtual coordinate space, with the upper-leftmost pixel of the bitmap at the origin, or 0,0 point. When using any of the rendering functions to draw onto a bitmap, any coordinates in the virtual coordinate space can be used. The rendering functions will clip the graphical elements being drawn to the visible surface of the bitmap. Depending on the coordinates specified and the graphical element being drawn, the clipping can result in all, some, or none of the pixels making up the graphical element to be rendered actually being being drawn on the bitmap itself. This is not an error, it is simply a natural consequence of the view that a bitmap is a viewport onto a larger virtual coordinate space.

It is possible to copy portions of one bitmap onto another. Copying of pixels from one bitmap to another, or from one location within a bitmap to another location within the same bitmap can be accomplished using the BitBlt() function. BitBlt stands for Bit Block Transfer, and is one of the fundamental operations used to manipulate bitmaps. When using the BitBlt() function to copy pixels between color and monochrome bitmaps, automatic color conversion is performed. See the section on Bit Block Transfer operations and the documentation for the BitBlt() function itself for more details.

In addition to the use of BitBlt(), it is also possible to use DrawBitmap() to copy pixels from one bitmap the another. The difference between BitBlt() and DrawBitmap() is that BitBlt() makes use of raster-operations (ROP codes) to specify how the source and destination pixels are combined when writing to the destination bitmap. DrawBitmap() instead, uses the background transparency mode, transparency color and bitmap drawing intensity to control how the source pixels are copied to the destination bitmap.

Off-screen, memory, bitmaps are created using the CreateBitmap() function. The width, height and pixel format (16-bpp or 1-bpp) are specified when the bitmap is created. If successful, the CreateBitmap() function returns a handle (HBMP) to the created bitmap. This handle is then used to refer to the bitmap in all future function calls that make use of the bitmap. When a bitmap is initially created, it is erased to the color clrBlack. Once a memory bitmap is no longer needed, the memory that it occupies should be returned to the system by calling DestroyBitmap(), passing the handle of the bitmap to be destroyed. The display bitmap should not be destroyed, although the system will ignore any attempt to do so. The dimensions and color format of a bitmap can be determined by calling GetBitmapDimensions().

### Device Independent Bitmaps

The MTDS system is able to make use of bitmaps stored in files on an SD card. Bitmaps in files are stored in the Device Independent Bitmap (DIB) format. The LoadBitmap() function will load a bitmap in DIB format from a file and convert it into an internal, or display, format bitmap in memory. The LoadBitmap() function returns the handle of the memory bitmap created. A memory bitmap created by LoadBitmap() is in all ways equivalent to a bitmap created by CreateBitmap(), except that its dimensions and initial contents are determined by the DIB information in the source bitmap file. In particular, this means that a bitmap created by LoadBitmap() can be selected as the drawing surface for a DS.

The DIB format supports a virtually unlimited number of color formats, compression formats and bitmap sizes. The LoadBitmap() function will fail if the DIB contains a bitmap that requires more memory to store than is currently available in the system, or if the DIB is in an unsupported color format or compression format.

There are numerous versions of the DIB file format and options for storing the data within a DIB. The following are not supported:

OS/2 version 1, also the same as Windows Version 2, format

OS/2 version 2 format

Compressed bitmaps, for example bitmaps containing JPEG data. Only RGB or “bitfields” bitmaps are supported.

Top down DIBs. Normally, DIB data is stored bottom-up. The format allows storage in top-down format, but these bitmaps are not supported.

DIBs containing 4-bpp image data are not supported

The DIB format allows for the use of a color mapping table to convert raw pixel data to the final colors to be displayed. Color tables are only supported for 8-bpp DIBs. Generally, this is the only supported pixel format that makes use of color mapping tables.

The LoadBitmap() function will convert the color format of the DIB data into the internal color format used by the display hardware and memory bitmaps. This conversion is only supported for a limited number of DIB formats. The DIB format conversion supports 1-bpp, 16-bpp, 24-bpp and 32-bpp DIBs. For the 16-bpp, 24-bpp and 32-bpp pixel formats, there is a “standard” format for how the color information is stored in the pixel. The DIB format allows for the specification of color masks to identify which bits within the pixel correspond to the R, G, and B data. The use of these color masks is not supported and only the “standard” pixel color formats are supported.

The standard DIB format for 32-bpp bitmaps stores 24 bits of pixel color data and can store 8 bits of alpha channel. The alpha channel can be used for transparency and pixel blending. Most 32-bit DIBs don't use the alpha channel and the value stored there is 0. Most 32-bpp DIBs contain exactly the same pixel data as a 24-bpp DIB. The MTDS DIB loader ignores the alpha channel if it is used by the DIB.

The standard DIB format for 16-bpp bitmaps stores 15 actual bits of data for each pixel. The pixel data is stored in a 5:5:5 format (5-bits red, 5-bits green, 5-bits blue) with one bit unused. The internal color format used by the display is 5:6:5 (5-bits red, 6-bits green, 5-bits blue). The 5-bit green data from a 16-bpp DIB is turned into 6-bit data by shifting left one bit position. The 5:6:5 color data is obtained from 24-bpp and 32-bpp bitmaps using the most significant 5 or 6 bits from the DIB data.

When creating bitmap files for use with the MTDS system, 16-bpp bitmaps are preferred over 24-bpp bitmaps which are preferred over 32-bpp bitmaps as there is less data to read and consequently, they will load faster. The maximum amount of realizable color information is obtained using 24-bpp bitmaps, but 16-bpp bitmaps will load faster.

A bitmap in memory can be written to a file in DIB format on the SD card using the StoreBitmap() function. This function will create a file on the SD card and write the image content of the bitmap in bottom-up DIB format as a 16-bpp bitmap (or a 1-bpp bitmap for monochrome bitmaps). The StoreBitmap() function makes use of color masks to identify the color format of the 16-bpp pixel data and the data is stored in the 16-bit 5:6:5 format rather than the standard 15-bit 5:5:5 format. PC image editing software will not be able to load these bitmap files if it doesn't support this format. This also constitutes an exception to the rule that the DIB loader doesn't support color masks. If the DIB data is in 16-bpp format and the color masks match the internal format, the DIB loader will accept the file. This means that the DIB loader in LoadBitmap() can load DIB files created by SaveBitmap().

Loading a DIB file into memory is a time consuming process. The MTDS display firmware does not make use of a real time operating system. An attempt is made to keep up with touch events while loading a DIB, but it is possible that the touch system may miss touch panel activity while loading a large DIB into memory.

### Brush Objects

A brush is a special kind of bitmap used for area fill when drawing filled shapes. Brushes are 8 pixels by 8 pixels in size with 16-bpp color. When an area is filled, the brush pattern is repeated left to right and top to bottom as needed to cover the entire area. The brush pattern is normalized to the screen coordinates so that if multiple filled objects are drawn adjacently, the patterns will line up. The graphical elements Ellipse, Rectangle and RoundRect are filled using a brush when they are drawn.

A brush handle (HBR) is used to refer to a brush. When a DS is initially obtained by calling GetDs() or GetDisplayDs(), the handle for the associated brush is set to hbrWhite, a predefined solid white brush. A brush is assigned to a DS by calling SetBrush(), passing the handle to the DS and the handle to the brush.

Filling the area of a graphical element, such as an Ellipse, can be suppressed by setting a null brush in the DS. This is done using SetBrush() with the predefined brush handle hbrNull. When the brush in a DS is null, no fill is performed when drawing a filled shape.

There are a small number of predefined brush handles defined by the system, but most brushes are created by calling one of several brush creation functions. A brush creation function will return a handle (HBR) to the created brush if it succeeds. When a brush that has been created is no longer needed, the memory that it occupies should be released and returned to the system by calling DestroyBrush(). A brush should not be destroyed while it is selected into a DS that is being actively used. The system will produce unpredictable results in this case.

A brush having all of the pixels set to the same color is called a solid brush. A solid brush is created by calling the function CreateSolidBrush() specifying the desired color for the brush. A solid brush can be created using any possible color value.

All predefined brushes are solid brushes. The following brush handles are predefined in the MTDS header files:. Predefined brushes should not be destroyed, although the system will ignore any attempt to do so:

hbrNull		- the null brush used to suppress area fill

hbrWhite		- a solid brush with the color value clrWhite (0x00FFFFFF)

hbrLtGray		- a solid brush with the color value clrLtGray (0x00C0C0C0)

hbrMedGray		- a solid brush with the color value clrMedGray (0x00808080)

hbrDkGray		- a solid brush with the color value clrDkGray (0x00606060)

hbrBlack		- a solid brush with the color value clrBlack (0x00000000)

hbrRed		- a solid brush with the color value clrRed (0x00FF0000)

hbrGreen		- a solid brush with the color value clrGreen (0x0000FF00)

hbrBlue		- a solid brush with the color value clrBlue (0x000000FF)

hbrLtBlueGray	- a solid brush with the color value clrLtBlueGray (0x00B8B8FF)

hbrMedBlueGray	- a solid brush with the color value clrMedBlueGray (0x007878FF)

A pattern brush is one having one of several crosshatch or checkerboard patterns. A pattern brush is created using the CreatePatternBrush() function. When creating a pattern brush, the pattern, a foreground color and a background color are specified. The resulting brush will have the pixels making up the pattern set to the specified foreground color and the other pixels set to the specified background color. There are a set of predefined patterns that can be used to create a patterned brush. The patterns are various crosshatch and checkerboard patterns. The following patterns are defined in the MTDS header files:

idpsHorizontal	- horizontal lines

idpsVertical	- vertical lines

idpsFwDiagonal	- diagonal lines that go up to the right

idpsBkDiagonal	- diagonal lines that go up to the left

idpsCross		- a crosshatch pattern of horizontal and vertical lines

idpsDiagCross	- a crosshatch pattern of FwDiagonal and BkDiagonal lines

idpsChecker		- a coarse checkerboard pattern

idpsBlock		- a finer checker board pattern than idpsChecker

idpsHalfTone	- a very fine checkerboard pattern with alternate pixels set

A brush containing any arbitrary collection of pixels can be created using CreateCustomBrush(). This function takes a handle to a 16-bpp color bitmap with dimensions of at least 8x8 pixels as a parameter. It copies the upper leftmost 8x8 pixels to create the brush.

### Font Objects

A font is a special data structure used by the text rendering engine in the MTDS display device firmware to describe the physical appearance of the characters making up the font. Fonts in the MTDS system are bit-mapped or raster fonts. Vector fonts are not supported. A font must be selected into the drawing state object before text can be drawn. When a DS is first obtained, the font is set to null. A font is selected into the DS by calling SetFont(), passing the handle to the DS and the handle to the font.

A font is essentially an array of monochrome bitmaps with some header information describing the width and height of the bitmaps and the inter-character spacing for the font. The text rendering engine supports both mono-spaced and proportionally spaced fonts.

Version 1 of the MTDS display device firmware only provides a single font, hfntConsole. Future versions of the system may provide additional built-in fonts and provide the ability to load fonts from files stored on the SD card.

### Bit Block Transfer – BitBlt

A bitmap is a rectangular array of pixels. The bit block transfer, BitBlt, operation is the copying of a rectangular portion of one bitmap, called the source, to another bitmap, the destination. This can be used to copy pixels from one bitmap to another, or to copy pixels from one location to another within the same bitmap.

In its simplest form, BitBlt merely involves copying the pixels from the source, overwriting the pixels in the destination. The implementation of BitBlt in the MTDS display device firmware allows the specification of a raster operation to be performed during the transfer. The raster operation specifies a function to be executed to combine the color value of the source pixels with the color value of the destination pixels to produce the value written to the destination. The MTDS BitBlt operation also allows the combination of source or destination pixels with pixels from the brush selected into the destination DS. Different raster op codes can be used to invert colors, mask colors , perform highlighting and in combination with brushes can be used for half-toning.

The BitBlt operation performs color conversion when copying pixels between bitmaps with different color formats. The MTDS system supports monochrome bitmaps and color bitmaps.

When performing a BitBlt from a monochrome bitmap to a color bitmap, pixels that are set ('1' bits) in the monochrome bitmap are converted to the current foreground color set in the destination DS. Pixels that are not set ('0' bits) are converted to the background color.

When performing a BitBlt from a color bitmap to a monochrome bitmap, pixels in the source bitmap whose color matches the background color are converted to black, all other pixels are converted to white. The color conversion if performed before the raster operation function is applied.

### Raster Operations and Drawing Mode

A raster operation is a boolean or logical function used to combine the source pixels, destination pixels, and brush pattern pixels to produce the final value written to the destination when performing a Bit Block Transfer, BitBlt, operation or a Pattern Block Transfer, PatBlt, operation. The raster operation (rop) to be used is specified as a parameter to the BitBlt() and PatBlt() functions. These are specified using a rop code symbol, for example ropSrcCopy, defined in MtdsDefs.h.

When using the BitBlt() function, the raster operation can involve source pixels, destination pixels and brush pattern pixels. When using the PatBlt() function, the raster operation can only involve destination pixels and brush pattern pixels, as there is no source operand.

There are 256 possible boolean functions of three variables. Version 1 of the MTDS display device firmware supports 32 of them. They are named in the following table along with the boolean function implemented. In this table, the letter S refers to a source bitmap pixel, the letter D refers to a destination bitmap pixel and the letter P refers to a brush pattern pixel. The result of applying the specified function produces the value written to the destination bitmap pixel:

ropBlack			clrBlack, i.e. 0

ropWhite			clrWhite, i.e. 1

ropSrc				S

ropSrcNot			~S

ropDst				D

ropDstNot			~D

ropSrcDstAnd			S & D

ropSrcDstOr			S | D

ropSrcDstXor			S ^ D

ropSrcDstAndNot		~(S & D)

ropSrcDstOrNot		~(S | D)

ropSrcDstXorNot		~(S ^ D)

ropSrcDstNotAnd		S & ~D

ropSrcNotDstAnd		~S & D

ropSrcDstNotOr		S | ~D

ropSrcNotDstOr		~S | D

ropPat				P

ropPatNot			~P

ropPatDstAnd			P & D

ropPatDstOr			P | D

ropPatDstXor			P ^ D

ropPatDstAndNot		~(P & D)

ropPatDstOrNot		~(P | D)

ropPatDstXorNot		~(P ^ D)

ropPatNotDstAnd		~P & D

ropPatDstNotAnd		P & ~D

ropPatNotDstOr		~P | D

ropPatDstNotOr		P | ~D

ropPatSrcAnd			P & S

ropPatSrcOr			P | S

ropPatSrcXor			P ^ S

ropPatSrcNotOrDstOr	(P | ~S) | D

When applying the raster operation function, the bitwise boolean operations are performed on all of the bits of the color values of the source, destination, and brush pattern pixels, as appropriate. The source and destination bitmaps are specified via the parameters to the BitBlt() function. The brush pattern pixels are specified by the brush pattern selected into the destination DS.

In addition to the above symbols, there are addition symbols defined that are synonyms of some of these symbols, as follows:

ropNotSrcErase		ropSrcDstOrNot

ropNotSrcCopy		ropSrcNot

ropSrcErase			ropSrcDstNotAnd

ropDstInvert			ropDstNot

ropPatInvert			ropPatNot

ropSrcInvert			ropSrcNot

ropSrcAnd			ropSrcDstAnd

ropMergePaint		ropSrcNotDstOr

ropMergeCopy		ropPatSrcAnd

ropSrcCopy			ropSrc

ropSrcPaint			ropSrcDstOr

ropPatCopy			ropPat

ropPatPaint			ropPatSrcNotOrDstOr

ropSrcXor			ropSrcDstXor

ropSrcNand			ropSrdDstAndNot

ropSrcNor			ropSrcDstOrNot

ropsrcXnor			ropSrcDstXorNot

A drawing mode is a boolean or logical function used to combine pen or font pixels with the existing pixel values in the bitmap when rendering graphical elements using a pen or a font. The drawing mode is stored as a property of the drawing state object and is set using the SetDrwRop() function. Drawing modes are specified using a drw code symbol, for example: drwCopyPen, defined in MtdsDefs.h.

There are sixteen possible boolean functions of two variables, and the MTDS system supports all of them. They are named in the following table along with the boolean function implemented. In this table, the letter P refers to a pixel of the pen and the letter D refers to a pixel in the bitmap being written to. The result of applying the specified function produces the value written to the destination pixel:

drwBlack			clrBlack

drwPenDstOrNot		~(P | D)

drwPenNotDstAnd		~P & D

drwPenNot			~P

drwPenDstNotAnd		P & ~D

drwPenNot			~P

drwPenDstXor		P ^ D

drwPenDstAndNot		~(P & D)

drwPenDstAnd		P & D

drwPenDstXorNot		~(P ^ D)

drwDst				D

drwPenNotDstOr		~P | D

drwPen			P

drwPenDstNotOr		P | ~D

drwPenDstOr			P | D

drwWhite			clrWhite

When applying the drawing mode functions, the foreground color is used for pen pixels that are set and the background color is used for pen pixels that are not set. The actual color value of the destination pixel is used for destination pixels. The logical functions are applied using bitwise operations on all bits of the color values. So, for example, the expression P | D means to perform a bitwise OR of the color value of the pen with the color value of the destination pixel.

In addition to the above symbol names for drawing operations, the following synonyms are defined in MtdsDefs.h:

drwNotOrPen			drwPenDstOrNot

drwAndNotPen		drwPenNotDstAnd

drwCopyNotPen		drwPenNot

drwAndPenNot		drwPenDstNotAnd

drwNot			drwDstNot

drwXorPen			drwPenDstXor

drwNotAndPen		drwPenDstAndNot

drwAndPen			drwPenDstAnd

drwNotXorPen		drwPenDstXorNot

drwNop			drwDst

drwOrNotPen			drwPenNotDstOr

drwCopyPen			drwPen

drwOrPenNot			drwPenDstNotOr

drwOrPen			drwPenDstOr

### Messaging System

The MTDS system provides a messaging system that is used for the display device to communicate information about asynchronous events, such as touch panel activity, to the application program.

The MTDS display device firmware maintains a queue to which message records are written as events of interest occur. When the system initializes, a fixed cache of message records is created and initialized. When a message is posted to the queue, a message record is removed from the cache, initialized with data about the event and then placed at the end of the message queue. If the message record cache is exhausted, the queue is full and no additional messages can be posted to the queue until some messages have been read from the queue.

The message system is used by the MTDS display device firmware to inform the application program about events of interest. It is also available to the application program for its own use. In some cases, it may be useful for one part of the application program to communicate with another part by passing messages using the messaging system. For this reason, in addition to reading messages from the message queue, the application can also post messages to the queue.

Message records are of a fixed size (16 bytes). There is a generic message structure defined, but different types of messages may use variants of the generic message structure. The first three members of all message structure variants are the same (tms, hwin, msg), but the other members will vary as long as they fit within the 16-byte fixed length of a message.

The generic message structure is as follows:

tms		- timer stamp for the message

hwin		- window handle for the recipient of the message

msg		- message identifier indicating the specific type of message

val1		- generic message value field

val2		- generic message value field

The tms member gives the time stamp for when the message was written to the message queue. This time stamp is in milliseconds since the display board was powered on or reset.

The hwin member contains the window handle of the intended recipient of the message. If the windowing system is not being used, this can be ignored.

The msg member contains an identifier indicating the specific type of message. For example, for touch messages use message identifiers in the range msgFingerFirst to msgFingerLast. These are identifiers such as msgFinger1Down, or msgFinger2Move. If the application program is using the message system for its own internal communications and needs to define application specific message identifiers, they should be numbered starting with msgUserFirst.

The function GetMsgStatus() is used to determine if there is one or more messages waiting in the message queue. If there are messages in the queue, the first one can be read from the queue by calling GetMsg(). This will remove the message from the queue, place the message event record back into the event record cache and return the contents of the message record. The function PeekMsg() will return the contents of the first message on the queue without removing it from the queue. The application can place a message in the queue by calling either PostMsg() or PushMsg(). PostMsg() places the message at the end of the queue and PushMsg() places the message at the head of the queue.

### Touch Panel Operation

The MTDS system supports a touch panel interface. The system is architected to support up to five point multi-touch. Refer to the documentation for a given display board for information on the number of simultaneous touches supported by the board. The currently existing display board products provide 2-point multi-touch. This means that the system can identify and track the presence of two fingers touching the display simultaneously.

The messaging system described above is used to provide touch information to the application program. Each time that a touch event occurs, a message is posted to the message queue. The application program learns about touch activity by polling the message queue using the GetMsgStatus() and GetMessage() functions.

Touch event messages use a structure of type MTCH to report information about the touch event. The MTCH structure is a variant of the generic MEVT structure used by the messaging system. The MTCH structure has the following fields:

tms		- message time stamp

hwin		- window handle of the window where the touch event occurred

msg		- touch message code

xco		- x location where the touch occurred

yco		- y location where the touch occurred

wgt		- “weight” of the touch (range: 0-15)

spd		- “speed” of movement of the touch (range: 0-15)

As with other messages in the messaging system, the time stamp in tms is the number of milliseconds since the display board was reset. This is the time when the touch event was written to the message queue.

The hwin member gives the window handle of the window in which the touch occurred. If the windowing system is not being used, the hwin member will contain the window handle of the desktop window and can be ignored.

The msg member indicates the actual touch action for the event. This will be one of the touch finger messages: e.g. msgFinger1Down, msgFinger1Move, msgFinger1Up, msgFinger2Down, etc. A finger down message indicates that a finger that was not previously touching the display is now touching the display. A finger move message indicates that a finger that was previously touching the display is now at a different location without leaving the display. A finger up message indicates that a finger that was touching the display is no longer touching.

The first finger to touch the display is reported as finger 1. If finger 1 is touching the display and another finger touches it, this will be reported as finger 2. The finger identity remains constant for as long as the finger is touching the display. So, for example, the first finger touches the display and is reported as finger 1. A second finger touches the display and is reported as finger 2. The first finger leaves the display. The finger being reported as finger 2 remains finger 2 even though there is now only a single finger touching. If another finger touches the display while finger 2 is still in contact, it will be reported again as finger 1.

The xco and yco members report the location of the touch event in screen coordinates (pixel resolution). For finger down and finger move messages, the actual coordinate of the touch is reported. For finger up messages, the last know position of the finger is reported.

In order to reduce the number of finger move messages reported, it is possible to set a movement delta threshold for x and y. This threshold value defaults to 5 pixels in x and y. This means that a finger must move at least five pixels from its last reported position before a move message is generated and posted to the message queue. The SetTchMoveDelta() function can be used to set this. A separate threshold ca be set for movement in x and y.

The move delta threshold places a granularity on the resolution of the touch move messages that will be generated. Setting the move delta thresholds to 1 will give the highest resolution of touch move information, but will result in many finger move messages being written to the message queue, increasing the chance of the queue becoming full and finger move messages being lost. Setting the move delta threshold to too large of a value makes the movement granularity very coarse and may result in no finger move messages being written to the queue at all.

The wgt member gives an indication of the area being covered by the finger touching the display. The value range for this member is 0-15.

The spd member gives an indication of the speed that a finger is moving for touch move messages. The value range for this member is 0-15.

In addition to controlling the movement delta threshold of finger move messages, it is also possible to control the touch panel sensitivity using the SetTchSensitivity() function. The default value for sensitivity is 2 and the range is 0-7. Setting the sensitivity to too high of a value will result in erratic behavior of the touch panel. In general, this setting should not be changed, but in some circumstances, such as the touch panel being behind some kind of screen protection, it may be necessary to increase the sensitivity.

The application program should not post touch messages to the message queue.

### Dynamic Memory Management

The firmware running on the display board maintains a dynamic memory heap used for dynamically allocating and deallocating memory objects as needed by the graphics engine, file management system or the user application.

When certain system objects are created, such as bitmaps or brushes, files are opened, and so on, a memory object is allocated from this dynamic memory pool. When the graphical object is destroyed or the file closed, the dynamic memory object is freed.

The dynamic memory manager uses a handle based system. When a memory object is allocated, a handle to it is returned. The handle can be turned into a pointer by locking the memory object. The pointer is released by unlocking the memory object. Unlocked memory objects can be moved around by the memory manager as needed to coalesce or garbage collect the heap. This allows for very efficient use of the dynamic memory pool. In general, all memory objects are unlocked between system API calls. The only memory objects that are locked are the ones being referenced by the API call being processed. This allows for very high utilization of the dynamic memory pool.

When a memory allocation request is made, the memory manager searches the heap for an unallocated memory block that is large enough to satisfy the allocation request. When such a memory block is found, it subdivides it into one piece that is used to satisfy the allocation request and another piece that contains whatever unallocated memory is left over. If the memory manager isn't able to find a free memory block that is large enough, it performs garbage collection on the heap. The heap is garbage collected by moving unlocked, allocated memory blocks around to form the largest possible unallocated memory blocks. Once garbage collection has been accomplished, it once again searches for a memory block that is large enough to satisfy the current allocation request. If one is still not found, then the allocation request fails and an out of memory condition is returned.

Although generally not needed, the application program can control some of the operating parameters of the dynamic memory manager. The memory allocation scheme and the garbage collection scheme used by the memory manager can be set by the applications program using the MTDS::SetMemMode() function. This can be used to select one of two allocation schemes and one of two garbage collection schemes as follows: mmFindFirst or mmFindNext, and mmCollectFull or mmCollectPartial.

When using the mmFindFirst allocation scheme, the memory manager starts at the beginning heap each time and stops when it finds the first unallocated memory block large enough to satisfy the requested allocation. When using he mmFindNext allocation scheme, the memory manager starts at the point where it found the free memory block for the most recent allocation and starts searching for a useable memory block at that point.

When using the mmCollectFull garbage collection scheme, when the memory manager performs garbage collection, it coalesces the entire memory heap. When using the mmCollectPartial scheme, it only coalesces enough of the heap to satisfy the current allocation request, leaving the rest unchanged.

The amount of dynamic memory available to the system varies depending on the particular display board and firmware version. The Multi-Touch Display Shield and PmodMTDS products have approximately 450K of dynamic memory available.

In addition to the automatic use of dynamic memory made by the system as graphical objects are created or destroyed and files are opened or closed, the system provides the ability for the applications program to also make use of the dynamic memory heap. Functions are provided for the applications program to allocate and deallocate dynamic memory objects and to write data to and read data from them. This allows the applications program to use the dynamic memory on the display board for temporary data storage. See the documentation of the Memory Management functions below for more information about this facility.

### SD File System

The MTDS system provides a set of API functions that can be used to access the file system on a micro-SD card installed in the connector on the display board. There are functions within the MTDS object class for loading and storing certain graphical objects (such as bitmaps) from or to files on the SD card. In addition, the MTFS object class provides a full set of API functions for direct access to the file system.

The MTFS object class and corresponding device firmware support FAT16 and FAT32 file systems on the SD card with support for long file names and directories.

Before the file system on the SD card can be accessed, it must be mounted. The application program can explicitly mount the SD card using the MTFS function MountFs() with the fMount parameter set to true. If the application program doesn't explicitly mount the SD card, it will be done implicitly by the MTDS system on the first API function call that requires access to the file system. If the SD card has been written to, the application program should unmount the file system before the SD card is removed. This is done by again calling MountFs() with the fMount parameter set to false. If the SD card is removed without closing opened files and un-mounting the file system, it is possible for the file system on the SD card to be corrupted. This is not an issue if no writes have been made to the SD card.

The file system code allows setting a current directory. This directory will then become the root for any relative path strings (in particular, a file name without a path) that are passed as file names. Full paths from the root of the file system can also be passed when passing a file name string. The longest string that can be passed as a path/file name string is defined by the symbol cchNameMax defined in MtdsDefs.h.

Each time that a file is opened, a memory object is allocated on the dynamic memory heap to control access to that file. Because of this, the only limit to the number of files that can be open for access simultaneously is the amount of free memory available in the memory heap. Files must be closed when access is no longer needed to recover this memory on the dynamic memory heap.

Searching a directory to discover what files are contained in it is accomplished using OpenDir(), ReadDir() and CloseDir(). OpenDir() takes a path to the directory. It allocates a dynamic memory object to control access to the directory and returns a handle to the opened directory. Following this, ReadDir() is called to successively get information about each entry contained within the directory. Each time ReadDir() is called, it returns information about the next entry in the directory. After the last entry in the directory has been returned, ReadDir() will return a File Not Found error status to indicate that the end of the directory has been reached. Once access to the directory is no longer needed, it must be closed by calling CloseDir() to recover the dynamic memory object used for directory access. Any number of directories can be opened simultaneously, but each must be closed when access is no longer needed.

The information about directory entries returned by ReadDir() is returned by filling in a FINF structure. The FINFO structure contains the following members:

cbFile		- Length of the file in bytes

date		- Last access date for the file

time		- Last access time for the file

attrib		- File attribute bits

rgchName[]	- Character string giving the short, 8.3 version of the file name

szName[]	- Character string giving the full file name for the file

The file date and time are stored packed into two 16-bit unsigned integers. In the date, the day of the month is stored in bits 0-4, the month is stored in bits 5-8, and the year is stored in bits 9-15. The year is stored relative to 1980. To obtain the correct year, add 1980 to the value stored in the date member.

The time is stored with the seconds stored in bits 0-4, the minutes stored in bits 5-10 and the hour stored in bits 11-15. Because only 5 bits are used for storing the seconds, they are stored with a resolution of 2 seconds. The turn the seconds value into the correct value it must be multiplied by 2.

The attrib field is a byte containing file attribute bits. Symbols are defined in MtdsDefs.h defining the attribute bits. The attrib value will be the bitwise or of one or more of these. They have the following meaning:

atMtdsReadOnly	- The file is read only

atMtdsHidden	- The file is hidden. It shouldn't be shown in a directory listing

atMtdsSystem	- The file is a system file

atMtdsVolume	- The directory entry is actually a volume label

atMtdsDirectory	- The “file” is actually a directory

atMtdsArchive	- The file has been modified

### Windowing System

### Real Time Clock

A simple real time clock is maintained by the firmware running on the display board. This clock maintains the date and time and is updated approximately every hundred milliseconds. This real time clock is used internally by the MTDS firmware to set the modification date and time for files when they are written to. It is accessible by the application program, but is not otherwise used by the system.

There is no real time clock hardware on the display board. The date and time are maintained by the software using a timer interrupt. It uses the processor clock for the microcontoller running the MTDS firmware as the time base and is only as accurate as the oscillator used by the processor. This means that it may drift up to several seconds per day. Since there is no real time clock hardware, there is also no battery to maintain the time, so the date and time gets reset each time the display board powers on or is reset. The default date and time are initialized to midnight, January 1, 2016.

If the application program has access to a reliable clock source, such as an application connected to the internet that has access to a time server, the date and time can be set and updated periodically. This will ensure that the modification date and time written to files is accurate. Otherwise, the modification date and time will be reasonably accurate relative to midnight, January 1, 2016. This allows a relative determination of when the file was written.

The MTDS functions SetDateTime() and GetDateTime() are used to access the real time clock.

### Error Reporting System and Error Codes

All API functions in the MTDS system either can not fail, or they return an indication of whether or not the function succeeded. In most cases, the functions return true if they succeeded or false if they failed. Some functions use the return value to return data to the application. In these cases, the function description will indicate a special return value that indicates that an error occurred.

All symbols described in this section are defined in the header file ProtoDefs.h.

Each of the interface object classes, MTDS, MTFS and MTWS, define API functions GetLastError() that can be used to query an error code indicating the nature of the failure of the last API function to be called. In each case, there are two variants of the function: GetLastError() just returns the error status code; GetLastError(pcls, pcmd) additionally provides the command class and the command code for the command that failed.

From the perspective of the command protocol used to communicate between the host and the display board, there are four subsystems used. Each of these subsystems has a corresponding command class, set of command codes and set of status codes. The command code values and status code values are reused for each of these subsystems. Therefore to completely disambiguate the the interpretation of a status code, it is necessary to know the command class of the command. In general, though, the applications programer knows that he called an MTFS function, for example, so he knows that what he gets will be a file system status code.

The following subsystems/command classes are defined:

clsCmdUtil		- Utility commands

clsCmdGdi		- Graphics system commands

clsCmdFs		- File system commands

clsCmdWin		- Windowing system commands

Future versions of the MTDS system may define additional command classes.

There are a few status codes that are shared by all subsystems as follows:

staCmdSuccess	- Always used to indicate that no error occurred

staCmdError	- A non-specific communications error occurred talking to the display

staCmdOutOfMemory	- The dynamic memory heap on the display board is out of memory

staCmdInternalError	- This indicates detection of an anomalous communications state. Probably the result of a bug in the communications code.

staCmdSync	- The host has lost communications synchronization with the display board. Call mtds.end() followed by mtds.Begin() to reset the system.

The following status codes are defined for the clsCmdUtil command class:

staUtilBadParameter	- An invalid parameter value was specified.

staUtilNotSupported	- The function called is not supported in this firmware version.

staUtilEventCacheExhausted	-There are no events in the event cache. It is necessary to call GetMessage() to retrieve some of the events from the queue.

staUtilInvalidMemoryHandle	- The value passed as a memory handle isn't valid.

The following status codes are defined for the clsCmdGdi command class:

staGdiBadParameter	- An invalid value was passed for a parameter to the function.

staGdiBadDataLength	- This indicates a communications error. This size of a command or data packet is inconsistent with the amount of data actually sent.

staGdiInvalidDs	- The drawing state handle passed is invalid.

staGdiInvalidBrush	- The brush handle passed is invalid.

staGdiInvalidFont	- The font handle passed is invalid.

staGdiInvalidBitmap	- The bitmap handle passed is invalid.

staGdiFormatNotSupported	- The file passed to LoadBitmap() contains a DIB in an unsupported format.

staGdiDiskError	- A physical disk error occurred when accessing the SD card while trying to load a graphical resource.

staGdiFileNotFound	- The specified file wasn't found when trying to load a graphical resource.

staGdiFileCreateError	- An error occurred in creating the output file when trying to store a graphical resource

staGdiFileReadError	- An error reading from the SD card while loading a graphical resource.

staGdiFileWriteError	- An error writing to the SD card wile trying to store a graphical resource.

staGdiFileDataError	- An internal inconsistency was found in the data file while trying to load a graphical resource. This likely means that the data file is corrupted.

The following status codes are defined for the clsCmdFs command class:

staFsDiskError	- A physical disk error was detected.

staFsInternalError	- An inconsistent internal state was detected in the file manager system. This is likely the result of a bug in the system.

staFsNotReady	- The physical disk (SD card) is not ready. SD card not in socket, SD card is defective, etc.

staFsFileNotFound	- Requested file not found in the directory.

staFsPathNotFound	- The specified path contains a directory that doesn't exist.

staFsInvalidName	- The specified name contains invalid characters.

staFsAccessDenied	- Requested access to the file is invalid.

staFsExist	- The file already exists and therefore can't be created.

staFsInvalidObject	- Invalid file or directory object. The SD card is likely corrupted or not format with a FAT file system.

staFsWriteProtect	- The physical drive is write protected.

staFsInvalidDrive	- The logical drive number is invalid.

staFsNotEnabled	- Error accessing the SD card

staFsNoFileSystem	- No FAT file system found on the SD card.

staFsAborted	- The operation was aborted due to a bad parameter.

staFsTimeout	- Operation timed out waiting to the SD card to become ready

staFsLocked	- File sharing violation

staFsOutOfMemory	- Out of memory in the dynamic memory heap.

staFsTooManyFiles	- Too many open files

staFsInvalidParameter	- An invalid value was passed for one of the function parameters.

staFsInvalidHandle	- An invalid file handle was specified.

The following status codes are defined for the clsCmdWin command class:

staWinBadParameter	- An invalid value was passed for one of the function parameters.

staWinInvalidHwin	- An invalid window handle was specified.

staWinDataDataLength	- The size of a data item was different than expected. This may indicate a communication error between the host and the display, or may be the result of a bug in the system.

staWinNoString	- The specified window does't have a window string.

staWinDsPoolExhausted	- BeginUpdate() failed because there are no free items in the DS pool. Call ReleaseDS() to free some up.

staWinHwinNotFound	- The specified window handle is correctly formed but the window doesn't exist.

### Structure Data Types

The MTDS library defines a number of structures that are used for passing parameters to or retrieving values from the system. The following structure types are defined in mtds.h:

##### Point Type

PNT is used to specify the coordinate of a point.

struct PNT {

int16_t	xco;

int16_t	yco;

};

##### Line Type

LINE is used to specify the two end points of a line.

struct LINE {

int16_t	xcoStart;

int16_t	ycoStart;

int16_t	xcoEnd;

int16_t	ycoEnd;

};

##### Rectangle Type

RCT is used to specify the corners of a rectangle.

struct RCT {

int16_t	xcoLeft;	// upper left corner

int16_t	ycoTop;

int16_t	xcoRight;	// lower right corner

int16_t	ycoBottom;

};

##### Bitmap Dimensions Type

BMPD is used to return the dimensions of a bitmap.

struct BMPD {

int16_t	dxco;		// bitmap width

int16_t	dyco;		// bitmap height

int16_t	cbpp;		// pixel format (bits per pixel)

};

##### Date and Time Information

DTM is used to hold a date and time for setting or querying the real time clock.

struct DTM {

uint16_t	sec;		// time in seconds

uint16_t	min;		// time in minutes

uint16_t	hour;		// time in hours

uint16_t	day;		// day of month

uint16_t	month;	// month of year

uint16_t	year;		// year

};

##### File Information

FINF is used for information about a file in the SD file system

struct FINF {

uint32_t	cbFile;	// length of the file

uint32_t	date;		// last modification date

uint32_t	time;		// last modification time

uint8_t	attrib;	// file attributes

uint8_t	rgchName[];	// file name in 8.3 format

uint8_t	szName[]	// long file name

};

##### Touch Event Message

MTCH is used to hold information about a touch event

struct MTCH {

uint32_t	tms;		// message time stamp

HWIN		hwin;		// window handle

uint16_t	msg;		// message identifier

int16_t	xco;		// x coordinate of the touch event

int16_t	yco;		// y coordinate of the touch event

uint8_t	wgt;		// “weight” of the touch event

uint8_t	spd;		// “speed” of the touch event

};

##### Memory Heap Status Type

MSTAT is used to return status information about the dynamic memory heap on the display board.

struct MSTAT {

uint32_t	mrStat;	// heap consistency check status

uint32_t	chmemTotal;	// total number of memory handles available

uint32_t	chmemAlloc;	// number of memory handles in use

uint32_t	chmemLock;	// number of locked memory handles

uint32_t	cmblkTotal;	// total number of blocks in the memory pool

uint32_t	cbAlloc;	// number of bytes in allocated memory blocks

uint32_t	cbFree;	// number of bytes in free memory blocks

};

## MTDS Class Interface Functions (Methods)

There are several general sets of interface functions defined in the MTDS object class: 1) General purpose control and status functions; 2) Messaging system functions; 3) Memory management functions; and 4) Graphics system functions. The graphics system functions are further subdivided into functions that manage resources used by the graphics system and functions that render graphical elements, either directly onto the display, or onto off-screen memory objects.

### General Control and Status Functions

A number of functions are provided that are used to manage the display and the graphics system in general. These functions can be used to initialize the system and then control various conditions of the display or the touch panel. Other than the begin() function that is used to initialize the system none of these functions need be used, as reasonable default values are used for the other properties that can be controlled.

##### MTDS::begin()

Parameters:

None

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function initializes the MTDS runtime system and establishes communications with the display board. One of the begin() function variants must be called before any other MTDS functions, except GetLastError() or FInitialized() can be called.

It initializes the system to use the default pin for selecting the display board and the default clock frequency for the SPI communications port to talk to the display board.

##### MTDS::begin(pinSel, frqSpi)

Parameters:

pinSel		- Pin number for pin to use for select for display board

frqSpi		- SPI clock frequency to use

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function initializes the MTDS runtime system and establishes communications with the display board. One of the begin() function variants must be called before any other MTDS functions can be called, except GetLastError() or Finitialized().

This function is used when a non-standard select pin or a non-standard SPI clock frequency is to be used.

The value in pinSel is the digital pin number to use for the chip select pin used to enable the display board for SPI communications. The symbol pinMtdsSelStd can be used here to specify the default chip select pin.

The value in frqSpi is the SPI clock frequency in HZ to use for the SPI port used to communicate with the display board. The symbol frqMtdsSpiDefault can be used here to specify the default SPI clock frequency.

Calling this function as follows: begin(pinMtdsSelStd, frqMtdsSpiDefault) will produce exactly the same result as calling begin().

Care should be taken when specifying the SPI clock frequency. Specifying a frequency that is too high can result in erratic behavior of the display board, especially when touch activity is being used.

##### MTDS::end()

Parameters:

None

Return Value:

None

Errors:

None

Description:

This function is used to terminate operation of the MTDS system and disable communications with the display board. Once this function has been called no MTDS library functions except Finitialized() and GetLastError() should be called. Following use of this function, one of the variants of begin() must be called to reinitialize the system.

##### MTDS::FInitialized()

Parameters:

None

Return Values:

Returns true if the MTDS system has been successfully initialized.

Errors:

None

Description:

This function can be used to determine if the MTDS library and display board have been initialized. This function will return true if the system has been successfully initialized or false if it has not.

##### MTDS::GetLastError(pcls, pcmd)

Parameters:

pcls	- Pointer to variable to receive command class

pcmd	- Pointer to variable to receive command code

Return Values:

Returns the error status value for the last MTDS operation.

Errors:

None

Description:

This function can be used to determine if the last MTDS operation attempted was successful or not. Generally, most functions return an indication of whether or not they succeeded. If a function fails, more detailed information about the nature of the failure can be obtained using this function.

The return value of the function will be an error code, or the value staCmdSuccess if no error occurred. The variable pointed to by pcls will be set to the command class of the last command sent to the display board. The variable pointed to by pcmd will be set to the command code of the last command sent to the display board. Interpretation of the status code returned depends on the command class of the command that failed.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTDS::GetLastError()

Parameters:

none

Return Values:

Returns the error status value for the last MTDS operation.

Errors:

None

Description:

This variant of the GetLastError() function only returns the error status of the last MTDS operation without returning the class or command codes of the last operation.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTDS::GetElapsedTime()

Parameters:

None

Return Value:

This function returns the execution time of the last MTDS function

Errors:

None

Description:

This function returns the elapsed time in milliseconds for the execution of the last MTDS function. This includes the time required to send the command packet and any required data packets to the display board, the time for the display board to execute the command, and the time for the display board to send any response or return data packets back to the host board.

##### MTDS::EnableStatusPin(idPin, fEn)

Reserved for future use.

##### MTDS::GetStatusPin(idPin)

Reserved for future use.

##### MTDS::SetStatusPin(idPin, st)

Reserved for future use.

##### MTDS::SetDisplayEnable(st)

Parameters:

st	- Display enable state to set (true for display on)

Return Value:

None

Errors:

Returns true if the function succeeded, false if not.

Description:

This function enables or disables the display. When the display is disabled, the display controller is turned off and the backlight power supply is turned off. This will significantly reduce the power used by the display board.

To turn the display on, pass the value true for st. To turn the display off, pass the value false. The display is on by default when the system initializes.

When the display is turned on, the display board firmware must reinitialize the display controller. This can take several seconds to complete. This function does not return until the initialization of the display controller has completed.

This function will only fail if there is a communications error between the host board and the display board.

##### MTDS::GetDisplayEnable()

Parameters:

None

Return Value:

Returns the current enable state of the display.

Errors:

Returns true if successful, false if not.

Description:

This function returns whether or not the display is currently enabled. It returns true if the display is enabled and false if it is not.

This function will only fail if there is a communications error between the host board and the display board.

##### MTDS::SetDisplayOrientation(dso)

Parameters:

dso	- Display orientation to set.

Return Value:

None

Errors:

Returns true if successful, false if not.

Description:

This function is used to set the display orientation. The display can be set to portrait mode, specified by dsoPortrait, or landscape mode specified by dsoLandscape. The display is in portrait mode by default after initialization.

Portrait mode has the X axis along the shorter side of the display and the Y axis along the longer side of the display.

Landscape mode has the X axis along the longer side of the display and the Y axis along the shorter side of the display.

In both orientations, the upper left corner of the display is the origin with the Y axis increasing going down.

This function will fail if an invalid value is specified for dso.

##### MTDS::GetDisplayOrientation()

Parameters:

None

Return Value:

Returns the current display orientation.

Errors:

Returns true if successful, false if not.

Description:

This function returns the current orientation of the display: dsoPortrait for portrait mode and dsoLandscape for landscape mode. The display will be in portrait mode by default after initialization.

This function will only fail if there is a communications error between the host board and the display board.

##### MTDS::SetDisplayBacklight(val)

Parameters:

val	- Display backlight brightness to set (0-100).

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function is used to set the brightness level of the display backlight. The value passed in val is in the range 0-100 and specifies the percent of brightness for the backlight. A value of 0 will turn the backlight off. A value of 100 specifies full brightness. The default value when the system is initialized is 60.

This function will fail if the specified value is out of range.

##### MTDS::GetDisplayBacklight()

Parameters:

None

Return Value:

Returns the current backlight brightness setting.

Errors:

Returns -1 if error occurs.

Description:

This function returns the current setting for the display backlight brightness.

##### MTDS::SetBacklightMode(val)

Parameters:

val	- Display backlight brightness mode to select

Return Value:

None

Errors:

Returns true if successful, false if not.

Description:

This function is used to specify the method used to control backlight brightness. The allowed values are dbmPwm, which specifies to use pulse width modulation for controlling backlight brightness, or dbmDbc, which specifies to use the digital brightness control mechanism in the display backlight power supply controller. The default when the system initializes is to use dbmDbc. This is not normally changed by the user sketch and this function will not normally be called.

This function fails if an invalid value is specified for val.

##### MTDS::ClearDisplay(clr)

Parameters:

clr	- Color value used to erase the display.

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function is used to erase the display to the specified color. Erasing the display essentially means filling the display frame buffer with the specified color value.

This function will only fail if there is a communications error between the host board and the display board.

##### MTDS::SetTchSensitivity(val)

Parameters:

val	- Touch panel sensitivity to set (1-7).

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function is used to set the sensitivity of the touch controller. The allowed range of values is 1-7. Larger values means more sensitive, smaller values are less sensitive. The default value when the system initializes is 2.

Normally the touch sensitivity should not be changed. Setting the sensitivity too high can result in erratic behavior of the touch panel. The sensitivity might need to be increased, for example, if the display panel is placed behind a cover, such as a glass window.

This function will fail if the specified sensitivity value is out of range.

##### MTDS::GetTchSensitivity()

Parameters:

None

Return Value:

Returns the current touch sensitivity setting

Errors:

Returns 0 if error occurs.

Description:

This function is used to obtain the current touch sensitivity setting.

This function will only fail if there is a communication error between the host board and the display board.

##### MTDS::SetTchMoveDelta(dxco, dyco)

Parameters:

dxco	- Movement delta in X to set.

dyco	- Movement delta in Y to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used to set the touch system movement delta threshold in X and Y. The movement delta threshold is used to determine when a finger movement has taken place. Movement of the touch point by amounts less than the current threshold is not treated as movement and does not generate a touch movement message.

The setting of the movement thresholds determines the granularity of movement in touch move messages and also controls the frequency of touch movement messages being written to the message queue. Setting the threshold values to 1 can result in a large number of movement messages being written to the message queue, increasing the possibility of the queue overflowing and messages being lost. Setting the threshold to values that are too large will make the movement granularity very coarse and possibly result in no touch movement messages being written to the message queue at all.

The default values set when the system is initialized are X threshold = 5 and Y threshold = 5. This means that, by default, finger movement of less than 5 pixels on the display will not generate a finger movement message in the message queue.

##### MTDS::GetTchMoveDelta(pdxco, pdyco)

Parameters:

pdxco	- Pointer to variable to receive X movement threshold.

Pdyco	- Pointer to variable to receive Y movement threshold.

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function returns the current X and Y touch movement thresholds. The variable specified by pdxco will be set to the X threshold and the variable specified by pdyco will be set to the Y threshold.

##### MTDS::SetDateTime(pdtm)

Parameters:

pdtm	- pointer to DTM structure giving the date and time to set.

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function is used to set the current date and time in the real time clock maintained by the firmware running on the display board.

##### MTDS::GetDateTime(pdtm)

Parameters:

pdtm	- Pointer to a DTM structure to receive the current date and time.

Return Value:

None

Errors:

Returns true if successful, or false if not.

Description:

This function returns the current date and time from the real time clock maintained by the firmware running on the display board.

### Messaging System Functions

The following functions are used to access the message queue maintained by the firmware running on the display board. See the section on the Messaging System above for detailed information about the operation and use of the messaging system and the message queue.

##### MTDS::GetMsgStatus()

Parameters:

none

Return Value:

Returns true if there is a message waiting in the message queue.

Errors:

none

Description:

This function returns the status of the message queue in the display board. If the least significant bit of the return value is set, there is one or more messages in the message queue.

##### MTDS::ClearMsgQueue()

Parameters:

none

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function removes all messages from the message queue.

Note that since touch activity is asynchronous it is possible for touch messages to have been placed into the queue after the queue has been cleared but before this function has returned. This function does not guarantee that the queue is empty, but it will contain no messages from prior to the time that the function was called.

##### MTDS::PeekMsg(pevt)

Parameters:

pevt		- pointer to event structure to fill in

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function returns the contents of the first message in the message queue, but does not remove it from the queue. The parameter pevt points to a structure of type MEVT used to return the contents of the message.

##### MTDS::GetMsg(pevt)

Parameters:

pevt		- pointer to event structure to fill in

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function returns the contents of the first message in the message queue and removes if from the queue. The parameter pevt points to a structure of type MEVT used to return the contents of the message.

##### MTDS::PostMsg(pevt)

Parameters:

pevt		- pointer to event structure to post to the message queue

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will place the message specified by the contents of the MEVT structure pointed to by pevt onto the message queue. The message is placed at the end of the queue.

If the message queue is full, the message is not posted and an error is returned.

##### MTDS::PushMsg(pevt)

Parameters:

pevt		- pointer to event structure to push to the message queue

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will place the message specified by the contents of the MEVT structure pointed to by pevt onto the message queue. The message is placed at the head of the queue.

If the message queue is full, the message is not posted and an error is returned.

### Memory Management Functions

The memory management functions provide a mechanism for the host sketch to make use of memory contained within the dynamic memory heap of the graphics processor on the display board. The memory heap maintained by the shield processor is a pool of dynamic memory used to cache graphics objects, provide dynamic working memory for the graphics and other software sub-systems on the shield and to provide dynamic memory for use by the sketch running on the host processor.

The amount of dynamic memory available depends on the specific implementation of the display board hardware and firmware. The Multi-Touch Display Shield and PmodMTDS have approximately 450K bytes of dynamic memory available.

##### MTDS::GetFreeMem()

Parameters:

None

Return Value:

Returns the number of bytes of free memory available on the display board.

Errors:

Returns 0 if an error occurs.

Description:

This function will return the number of free bytes available in the dynamic memory heap on the display board.

##### MTDS::GetMemStatus(pmstat)

Parameters:

pmstat		- Pointer to a MSTAT structure to be filled in.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will check the dynamic memory heap on the display board for internal consistency and then return statistics about the memory heap. If internal errors are detected in the memory heap, the mrStat member of the MSTAT structure will be filled in with an error code indicating the first error detected. In this case, no other members of the MSTAT structure will be filled in. If no internal errors are detected in the heap it will fill in the other members of the MSTAT structure with information about the state of the heap. See the sections above on Dynamic Memory Management and the Error Reporting System for more detailed information about dynamic memory and the dynamic memory heap as well as descriptions of the error codes that may be returned.

This function will only return false if there is a communications error with the display board. A return value of true means that the command succeeded in getting information about the memory heap status, not that the heap itself is error free. It is necessary to check the mrStat member of the MSTAT structure to determine the error status of the heap itself.

The MSTAT object contains the following fields:

mrStat		- Heap consistency status code

chmemTotal	- Total number of memory handles available in the heap

chmemAlloc	- Number of allocated memory handles

chmemLock	- Number of locked memory handles

cmblkTotal	- Total number of memory blocks in the heap

cbAlloc	- Total number of bytes in allocated memory blocks

cbFree		- Total number of bytes in free memory blocks

##### MTDS::SetMemMode(mm)

Parameters:

mm		- Memory allocation and compaction modes to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function sets the operating mode of the memory allocator in the dynamic memory management system. The mode specifies the memory allocation strategy to use and the garbage collection/heap compaction method to use.

The value specified in mm is the bitwise or (or arithmetic sum) of the symbols specifying the allocation strategy and the compaction method. One allocation strategy and one compaction method should be specified.

The allocation strategies allowed are:

mmFindFirst

mmFindNext

When using the mmFindFirst strategy, the memory allocator starts at the beginning of the heap each time and stops when it finds the first available memory block that is large enough to satisfy the memory allocation request, or when it reaches the end of the heap. The mmFindNext allocation strategy starts searching at the point in the heap where the last successful allocation occurred and stops at the first available memory block that is large enough to satisfy the request or when the search has wrapped around and come back to the starting point.

The heap compaction methods allowed are:

mmCompactFull

mmCompactPartial

Heap compaction is performed when the memory allocator has been unsuccessful at finding a free memory block that is large enough to satisfy the allocation request. When the compaction method is mmCompactFull, the garbage collector compacts the entire heap. Following this, the allocator will start at the beginning of the heap and search for a suitable memory block. When the compaction mode is mmCompactPartial, the garbage collector starts compacting the heap from the beginning and stops compacting as soon as it has created a memory block large enough to satisfy the allocation request.

##### MTDS::AllocMem(cbReq)

Parameters:

cbReq		- The size in bytes of the requested memory block

Return Value:

Returns the handle to the resulting memory block.

Errors:

Returns 0 if an error occurs.

Description:

This function will allocate a memory block of at least the requested size from the dynamic memory heap on the display board. The memory block will be at least as large as the requested size, but may be larger.

The granularity of memory blocks in the dynamic memory heap is four bytes and they are allocated in multiples of four bytes in length. The requested size will be rounded up to the next multiple of four bytes if it is not already a multiple of four.

This function will fail if it is not possible to allocate a memory block of the requested size. Compaction of the dynamic memory heap may result as a side effect of calling this function.

##### MTDS::GetMemSize(hmem)

Parameters:

hmem		- Handle to the memory object to query

Return Value:

This function returns the actual size, in bytes, of the specified memory object.

Errors:

Returns 0 if an error occurs.

Description:

This function will return the actual size, in bytes, of the specified memory object. A memory object may be larger than the size requested when it was created and this function returns the actual size of the memory object rather than the size that was requested when it was created.

This function will fail if the value specified in hmem is not a valid memory handle.

##### MTDS::FreeMem(hmem)

Parameters:

hmem		- Handle to the memory object

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used to free a memory object and release the memory space it uses back to the dynamic memory heap. Memory objects should be freed by calling this function as soon as they are no longer needed.

This function will fail if the value specified in hmem is not a valid memory handle.

##### MTDS::ReadMem(hmem, ibRead, cbRead, rgbBuf, pcbRes)

Parameters:

hmem		- Handle to the memory object

ibRead		- Byte offset from the beginning of the memory object.

cbRead	- The number of bytes to read from the memory object.

rgbBuf		- Pointer to the buffer to receive the data read.

pcbRes		- Pointer to variable to receive the actual number of bytes read.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will read the specified number of bytes from the specified memory object into the specified local memory buffer. The value in ibRead specifies an index, or offset, from the beginning of the memory object to the first byte to be read and the value in cbRead specifies the number of bytes to read.

The parameter pcbRes is a pointer to a variable that will be filled in with the actual number of bytes read. The actual number of bytes read can be less than the amount requested if the combination of cbRead+ibRead is larger than the actual size of the memory object.

This function will fail if the value specified in hmem is not a valid memory handle.

##### MTDS::WriteMem(hmem, ibWrite, cbWrite, rgbBuf, pcbRes)

Parameters:

hmem		- Handle to the memory object.

ibWrite	- Byte offset from the beginning of the memory object.

cbWrite	- The number of bytes to write to the memory object.

rgbBuf		- Pointer to the buffer containing the bytes to write.

pcbRes		- Pointer to variable to receive the actual number of bytes written.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will write the specified number of bytes from the specified local memory buffer to the specified memory object in the display board. The value in ibWrite specifies an index, or offset, from the beginning of the dynamic memory object to the location of the first byte to be written. The value in cbWrite specifies the number of bytes to write. The value in rgbBuf is the address of a buffer containing the bytes to be written. The bytes written are taken starting at the beginning of the specified buffer.

It the memory object isn't large enough to contain the bytes being written (i.e. the combination of ibWrite+cbWrite is larger than the current size of the object), an attempt will be made to grow the object. If the attempt to grow the memory object fails, the function call will be aborted and no data will be written to the memory object.

The parameter pcbRes is a pointer to a variable that will be filled in with the actual number of bytes written to the dynamic memory object.

This function will fail if the value specified in hmem is not a valid memory handle.

### Graphics System Functions

The following describe interface functions or methods defined for the MTDS object class. These functions provide the primary interface to the graphics subsystem of the MTDS system. They are divided up into control and resource management functions, functions for rendering graphical elements, and a set of functions for performing operations on rectangles.

#### Graphics System Control and Resource Management Functions

##### MTDS::GetDisplayDs()

Parameters:

None

Return Values:

Returns the handle to the display DS

Errors:

Returns 0 on failure

Description:

This function returns the handle (HDS) to the display DS. The display DS is a distinguished DS that is automatically set to have the display bitmap as the drawing surface. All other DS properties are set to the default values. When it is no longer needed, the display DS must be released by calling ReleaseDs(), passing the HDS value returned. A subsequent call to this function will fail until the HDS has been released.

##### MTDS::GetDs()

Parameters:

None

Return Values:

Returns the handle to an allocated and initialized DS

Errors:

Returns 0 on failure

Description:

This function allocates and initializes the first available DS from the DS cache and returns a handle that can be used to access it. All DS properties are set to the default values. In most cases, a drawing surface bitmap must be assigned by calling SetDrawingSurface() before the DS can be used. A drawing surface bitmap must be set before any graphical rendering operations can be performed.

The DS must be released by passing the returned HDS value to ReleaseDs() when it is no longer needed.

This function will fail if there are no unused DS objects available in the DS cache.

##### MTDS::ReleaseDs(hds)

Parameters:

hds	- The handle to the DS to be released

Return Values:

none

Errors:

Returns true if the function succeeds, false if it fails

Description:

This function is used to release a DS object back to the system when it is no longer needed. If the value specified by hds is the display DS, it is marked as not being in use. If it is a DS allocated from the DS cache, it will go back into the pool of unallocated and available DS objects.

This function will fail if the specified hds value isn't a valid DS handle.

##### MTDS::GetNearestColor(clr)

Parameters:

Return Value:

Errors:

Description:

##### MTDS::SetFgColor(hds, clr)

Parameters:

hds	- Handle to a DS

clr	- Color value to set as the current foreground color

Return Values:

none

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function will set the current foreground color of the specified DS to the specified color value. The foreground color is used when drawing linear objects, such as lines, arcs, or the perimeter of rectangles and when drawing text.

This function will fail if the specified hds isn't a valid DS handle.

##### MTDS::GetFgColor(hds)

Parameters:

hds	- Handle to a DS

Return Values:

Returns the current foreground color for the specified DS

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function returns the currently selected foreground color for the specified DS. It fails if the hds specified isn't a valid DS handle.

##### MTDS::SetBgColor(hds, clr)

Parameters:

hds	- Handle to a DS

clr	- Color value to set as the current background color

Return Values:

none

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function will set the current background color of the specified DS to the specified color value. The background color is used when drawing linear objects, such as lines, arcs, or the perimeter of rectangles and when drawing text when the background transparency mode is set to bkOpaque.

This function will fail if the specified hds isn't a valid DS handle.

##### MTDS::GetBgColor(hds)

Parameters:

hds	- Handle to a DS

Return Values:

Returns the current background color for the specified DS

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function returns the currently selected background color for the specified DS. It fails if the hds specified isn't a valid DS handle.

##### MTDS::SetTransColor(hds, clr)

Parameters:

hds	- Handle to a DS

clr	- Color value to set as the current transparency color

Return Values:

none

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function will set the current transparency color of the specified DS to the specified color value. The transparency color is used by the DrawBitmap() function. See the description of DrawBitmap() for more information about transparency and the transparency color. Note that this is different than the background transparency mode used with drawing linear type graphical  elements and text.

This function will fail if the specified hds isn't a valid DS handle.

##### MTDS::GetTransColor(hds)

Parameters:

hds	- Handle to a DS

Return Values:

Returns the current transparency color for the specified DS

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function returns the currently selected transparency color for the specified DS. It fails if the hds specified isn't a valid DS handle.

##### MTDS::SetIntensity(hds, ity)

Parameters:

hds		- Handle to the drawing state object

ity		- Intensity value to set

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function sets the specified intensity value into the DS specified by hds. The intensity value is an integer in the range 0 – 200. It represents a percentage value with the value 100 meaning 100%. The intensity is used by the DrawBitmap() function to scale the brightness of pixels being copied. The default value of intensity is 100.

##### MTDS::GetIntensity(hds)

Parameters:

hds		- Handle to the drawing state object

Return Value:

Returns the current intensity setting store in the specified DS

Errors:

Returns -1 if an error occurs.

Description:

This function returns the intensity value currently set in the DS specified by hds.

##### MTDS::SetPen(hds, pen)

Parameters:

hds		- Handle to a DS

pen		- Bit pattern specifying the pen to set

Return Values:

none

Errors:

Returns true if the function succeeds, false if it fails.

Description:

This function sets the specified pen pattern as the current pen in the specified DS. The pen is used when drawing linear type objects such as lines, arcs or the perimeter of rectangles. When drawing with a pen, bits that are set to '1' in the pattern will be drawn with the current foreground color. Bits that are set to '0' may be drawn with the current background color depending on the current background transparency mode. See the sections on Pen Pattern and Background Transparency Mode above for more information.

This function fails if the specified hds isn't a valid DS handle.

##### MTDS::GetPen(hds)

Parameters:

hds	- Handle to the drawing state object to query.

Return Value:

Returns the current pen pattern for the specified DS.

Errors:

Returns 0 if an error occurs.

Description:

This function returns the current pen pattern for the specified DS.

The return value will be 0 if an error occurs. The value 0 for the pen is also a valid pen value and indicates a NULL pen.

This function will fail, and return 0, if the value specified in hds is not a valid DS handle, or because of a communications error with the display board. To distinguish between a legitimate value of 0 for the pen and an error, use the GetLastError() function to determine if the command succeeded.

##### MTDS::SetBkMode(hds, bk)

Parameters:

hds	- Handle to a DS

bk	- Background transparency mode to set.

Return Value:

None

Errors:

Returns true if successful, false if it fails.

Description:

This function is used to set the background transparency mode. The mode to set is specified by the parameter bk. The allowed values are bkOpaque or bkTransparent.

The background transparency mode affects the drawing of graphical elements using a pen or of text. When the transparency mode is set to bkOpaque, background pixels are drawn using the currently set background color. When the transparency mode is set to bkTransparent, background pixels are not drawn.

The background transparency mode also affects the operation of DrawBitmap(). When the transparency mode is bkOpaque, DrawBitmap() copies all pixels from the source bitmap to the destination bitmap. When the transparency mode is set to bkTransparent, DrawBitmap() does not copy pixels from the source bitmap whose color matches the current background transparency color.

This function will fail if an invalid value is specified for the background transparency mode.

##### MTDS::GetBkMode(hds)

Parameters:

hds		- Handle to the DS to query

Return Value:

Returns the current background transparency mode for the DS

Errors:

Returns 0 if it fails.

Description:

This function will return the background transparency mode currently set in the specified DS.

This function will only fail if there is a communications error between the host board and the display board.

##### MTDS::SetDrwRop(hds, drw)

Parameters:

hds		- Handle to the DS to set

drw		- Drawing mode raster operation code to set

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function sets the raster operation code to use when drawing graphical elements using a pen or when drawing text. The raster operation specifies how pixels of the element being drawn are combined with the existing pixel values in the destination bitmap. See the section on Raster Operations and Drawing Mode for more detailed information.

This function fails if an invalid drawing mode is specified.

##### MTDS::GetDrwRop(hds)

Parameters:

hds		- Handle to the DS to query.

Return Value:

Returns the current drawing mode raster op from the specified DS.

Errors:

Returns -1 if error occurs.

Description:

This function returns the current drawing mode raster operation code from the specified DS. See the section on Raster Operations and Drawing Mode for more detailed information about drawing modes.

##### MTDS::SetDrawingSurface(hds, hbmp)

Parameters:

hds		- Handle to the DS to set.

hbmp		- Handle to the bitmap to set as the drawing surface.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used to set the handle to the bitmap to use as the drawing surface for the DS whose handle is specified by hds. The drawing surface for a DS is the bitmap on which rendering functions will draw the graphical elements. Once the drawing surface has been set, all future rendering operations will write to that bitmap. The drawing surface bitmap must remain valid, so the bitmap specified by hbmp must not be destroyed as long as it is set as a drawing surface bitmap.

The same bitmap can be the drawing surface for multiple DS objects. This allows for multiple sets of drawing contexts (parameter settings) to be simultaneously extant for drawing onto a given bitmap. Setting up multiple DS objects using the same drawing surface bitmap, but with different parameters can speed up drawing operations if it is frequently necessary to switch parameters while drawing.

When a DS is initially obtained by calling GetDs(), the drawing surface is set to null. The drawing surface can be returned to null by calling SetDrawingSurface() with hbmpNull as the bitmap handle.

##### MTDS::GetDrawingSurface(hds)

Parameters:

hds		- Handle of DS to query

Return Value:

Returns the handle of the bitmap currently set as the drawing surface

Errors:

Returns true if successful, or false if it fails.

Description:

This function returns the handle to the bitmap currently set as the drawing surface for the specified DS.

##### MTDS::SetFont(hds, hfnt)

Parameters:

hds		- Handle to the DS to set.

hfnt		- Handle to the font to set in the DS for drawing text.

Return Value:

None

Description:

This function sets the handle of the font to be used for drawing text in the DS specified by the handle in hds. When a DS object is first obtained by calling GetDs(), the font is set to null. It is necessary to set a font into the DS before text can be drawn.

See the section above on Fonts and Drawing Text for more detailed information.

##### MTDS::GetFont(hds)

Parameters:

hds		- Handle to the DS to query.

Return Value:

Returns the handle to the font currently set in the specified DS

Errors:

Returns true if successful, or false if it fails.

Description:

This function returns the handle of the font currently set in the specified DS.

##### MTDS::GetDisplayBitmap()

Parameters:

None

Return Value:

Returns the handle to the display bitmap.

Error:

Returns 0 if an error occurs.

Description:

This function returns the handle to the display bitmap. The display bitmap represents the frame buffer memory in the display controller. The contents of the display bitmap is visible on the physical display. The display bitmap handle can be used with SetDrawingSurface() to set up a DS object to write onto the physical display.

The display bitmap should not be destroyed, although the system will ignore any attempt to do so. The display bitmap handle can not be passed to the StoreBitmap() function for writing to a file on the SD card.

This function will only fail if there is a communications error talking to the display board.

##### MTDS::CreateBitmap(dxco, dyco, cbpp)

Parameters:

dxco		- Width in pixels for the bitmap to create.

dyco		- Height in pixels for the bitmap to create.

cbpp		- Pixel format as number of bits per pixel (1 or 16)

Return Value:

Returns the handle of the bitmap

Errors:

Returns 0 if the function fails.

Description:

This function is used to create an off-screen bitmap in the memory of the display board. The width and height of the bitmap are specified by dxco and dyco. To create a monochrome bitmap, the value 1 is passed in cbpp. To create a color bitmap the value 16 is passed in cbpp. Other bitmap color formats are not supported for memory bitmaps. See the section on Bitmaps above for more detailed information about bitmaps.

This function will fail if there is not enough free memory available on the display board to create a bitmap with the requested dimensions and pixel format, or if an invalid pixel format is specified.

The primary thing that can be done with the handle to an off-screen bitmap is to select it as the drawing surface for a DS by passing its handle to SetDrawingSurface(). An off-screen bitmap can also be written to a file on an SD card using the StoreBitmap() function.

##### MTDS::DestroyBitmap(hbmp)

Parameters:

hbmp		- Handle to the bitmap to destroy.

Return Value:

None

Errors:

Returns true if successful, or false in it fails.

Description:

This function is used to free the memory associated with an off-screen bitmap when it is no longer needed. Off-screen bitmaps are stored in a dynamic memory heap on the display board and should be freed when no longer needed to return the memory to the heap for other uses. Once a bitmap has been freed, the handle should not be used again, as it no longer refers to a valid bitmap.

Note that the bitmap handles for off-screen bitmaps will be reused by the system as bitmaps are created and destroyed. This means that the same handle value could potentially refer to different bitmaps depending on the sequence of creation and destruction of the bitmaps. It is good programming practice to zero the variable holding a bitmap handle as soon as a bitmap has been destroyed so that the handle isn't accidentally used at a later time to refer to the wrong bitmap.

This function will fail if the value passed in hbmp isn't a valid bitmap handle.

##### MTDS::GetBitmapDimensions(hbmp, pbmpd)

Parameters:

hbmp		- Handle to the bitmap to query

pbmpd		- Pointer to a BMPD structure to be filled in with the bitmap dimensions

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will fill in the specified BMPD structure with the dimensions and pixel format of the specified off-screen bitmap. This is particularly useful after calling LoadBitmap() to create a bitmap from a DIB file on the SD card to determine the size of the bitmap created.

The BMPD structure contains the following members:

dxco	- Width of the bitmap in pixels

dyco	- Height of the bitmap in pixels

cbpp	- Pixel format (1 or 16)

This function will fail if the value specified in hbmp is not a valid bitmap handle.

##### MTDS::StoreBitmap(szName, hbmp)

Parameters:

szName	- Pointer to a “C” string specifying the file name

hbmp		- Handle to an off-screen bitmap

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will write the specified off-screen bitmap to a file on the SD card in device independent bitmap (DIB) format. See the sections on Bitmaps and Device Independent Bitmaps above for more detailed information about off-screen bitmaps and device independent bitmaps.

This function will fail if the file name contains invalid characters for a file name or if the value specified in hbmp is not a valid bitmap handle. It will also fail if there is not an SD card with sufficient free space to contain the resulting file in the SD card socket.

##### MTDS::LoadBitmap(szName)

Parameters:

szName	- Pointer to a “C” string specifying the file name.

Return Value:

Returns a handle to an off-screen bitmap containing the result.

Errors:

Returns 0 if the function fails.

Description:

This function will read the contents of a device independent bitmap (DIB) from the specified file on the SD card and create an off-screen bitmap initialized with the image contained in the file. See the section on Bitmaps above for more detailed information about off-screen bitmaps, device independent bitmaps and the conversions that occur when loading a device independent bitmap into memory.

This function will fail if the file name specified in szName is invalid or does not exist on the SD card. It can also fail if there is insufficient free memory in the dynamic memory heap on the display board to contain the resulting bitmap, or the device independent bitmap contained in the file is in an unsupported format.

If the function succeeds, it returns a bitmap handle to the resulting off-screen bitmap.

The dimensions and color format for the resulting off-screen bitmap can be determined by calling the GetBitmapDimensions() function.

##### MTDS::CreateSolidBrush(clr)

Parameters:

clr	- Color value specifying the color of the brush

Return Value:

Returns the handle to the resulting brush.

Errors:

Returns 0 if it fails.

Description:

This function will create a solid brush of the specified color. A solid brush is one where all of the pixels within the brush are the same color. See the section on Brushes above for more detailed information about brush objects.

This function can fail if there is not sufficient free memory in the dynamic memory heap on the display board to contain the resulting brush.

##### MTDS::CreatePatternBrush(idp, clrFg, clrBg)

Parameters:

idp		- Identifier for the hatch pattern to use for the brush.

clrFg		- Foreground color used to create the hatch pattern.

ClrBg		- Background color used to create the hatch pattern.

Return Value:

Returns the handle to the resulting brush.

Errors:

Returns 0 if the function fails.

Description:

This function creates a brush with a fill pattern of the specified color. The pattern to use is specified by idp. The MTDS system defines a set of fill patterns whose identifiers are defined in the MTDS header files.

The following patterns are defined:

idpsHorizontal	- horizontal lines

idpsVertical		- vertical lines

idpsFwDiagonal	- diagonal lines down and to the right

idpsBkDiagonal	- diagonal lines up and to the left

idpsCross		- cross pattern of horizontal and vertical lines

idpsDiagCross		- cross pattern of fw diagonal and bk diagonal

idpsChecker		- checkerboard pattern

idpsBlock		- finer grained checkerboard pattern

idpsHalftone		- very fine grained checkerboard with alternate pixels set

The resulting brush will have the specified pattern of the specified foreground color on a background of the specified background color.

See the section above on Brushes for more detailed information about brushes and filled areas.

##### MTDS::CreateCustomBrush(hbmp)

Parameters:

hbmp		- Handle to bitmap to use as the pattern

Return Value:

Returns a handle to the resulting brush.

Errors:

Returns 0 if it fails.

Description:

This function will create a custom brush using the specified bitmap. The custom brush is created by copying the upper-leftmost 8 x 8 pixels from the specified bitmap. The bitmap specified by hbmp must be at least 8 x 8 pixels in size and be a 16-bpp color bitmap.

See the section on Brushes above for more detailed information about brushes and filled areas.

This function will fail if there is not enough memory in the dynamic memory heap on the display board to contain the resulting brush or if the specified bitmap is not suitable for use in creating the brush.

##### MTDS::DestroyBrush(hbr)

Parameters:

hbr		- Handle to the brush to destroy.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used when a brush is no longer required to recover the memory in the dynamic memory heap on the display board. A brush should not be destroyed as long as it is selected into DS that is being actively used.

This function will fail if the value specified by hbr is not a valid brush handle.

##### MTDS::SetBrush(hds, hbr)

Parameters:

hds		- Handle to the DS object to set.

hbr		- Handle to the brush to set as the current brush.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function sets the brush specified by hbr as the current fill brush for the specified DS. The specified brush will then be used for area fills when rendering filled graphical elements using the DS. To suppress having the area of a rendered object filled, the value hbrNull can be used for the brush handle. When the DS is set to use a null brush, no fills will be drawn. See the section on Brushes above for more detailed information about brush objects.

This function will fail if the value specified in hds is not a valid DS handle or the value specified in hbr is not a valid brush handle.

##### MTDS::GetBrush(hds)

Parameters:

hds		- Handle to the DS to query.

Return Value:

Returns the handle of the brush currently set in the DS.

Errors:

Returns 0 if it fails.

Description:

This function will return the handle of the brush currently set in the specified DS. The brush currently set in the DS is used for filling the interior area of some graphical elements when they are rendered. See the section on Brushes above for more detailed information about brushes and filled areas.

This function will fail if the value specified in hds is not a valid DS handle.

#### Graphics System Rendering Functions

##### MTDS::SetPixel(hds, xco, yco, clr)

Parameters:

hds		- Handle to the DS object

xco		- X coordinate of the pixel to set.

yco		- Y coordinate of the pixel to set.

clr		- Color value to set the pixel to.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will set the pixel of the drawing surface bitmap associated with the specified DS, at the specified x,y coordinate to the specified color. If the drawing surface of the specified DS is a monochrome bitmap, only the least significant bit of the color value specified by clr is used.

If the specified x,y coordinate is outside the bounding rectangle of the drawing surface bitmap, nothing is drawn.

##### MTDS::GetPixel(hds, xco, yco)

Parameters:

hds		- Handle to the DS object

xco		- X coordinate of the pixel

yco		- Y coordinate of the pixel

Return Value:

Returns the color of the specified pixel

Errors:

Returns 0 if it fails.

Description:

This function will return the color of the pixel within the drawing surface bitmap associated with the specified DS at the specified location. If the drawing surface of the DS is a monochrome bitmap, the returned value will be 0 or 1.

If the specified x,y coordinate is outside the bounding rectangle of the drawing surface bitmap, the value 0 will be returned, corresponding to the color clrBlack. This is not an error.

This function will fail if there is a communications error with the display board or if the value specified in hds is not a valid DS handle, or if there is no drawing surface bitmap associated with the DS. This function returns 0 when an error occurs. Since the value 0 is also a valid color value, to distinguish the color value 0 from the error value 0, call the GetLastError() function to determine if the function call succeeded.

##### MTDS::GetCurPos(hds, ppnt)

Parameters:

hds		- Handle to the DS to query.

ppnt		- Pointer to a PNT structure to receive the coordinate.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will return the coordinate of the current position stored in the specified DS. The x and y values of the coordinate are filled into the xco and yco members of the PNT structure specified by ppnt.

##### MTDS::MoveTo(hds, xco, yco)

Parameters:

hds		- Handle to the DS to set.

xco		- X coordinate

yco		- Y coordinate

Return Value:

None

Errors:

Returns true if successful, false if it fails.

Description:

This function sets the specified x and y values as the coordinate of the current position stored in the specified DS. The current position is used as the starting point for certain graphical element rendering functions.

The specified x,y location can be anywhere in the virtual coordinate space associated with the drawing surface bitmap in the DS. There is no requirement that it be “on” the bitmap.

This function will fail if the value specified in hds is not a valid DS handle.

##### MTDS::MoveTo(hds, ppnt)

Parameters:

hds		- Handle to the DS to set.

ppnt		- Pointer to a PNT structure specifying the coordinate to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function sets the current position for the specified DS to the location specified by the specified PNT structure. This function is identical to the MoveTo(hds, xco, yco) function described above, except that the coordinate of the new position is specified using a PNT structure rather than as separate X and Y values.

##### MTDS::Line(hds, xco, yco)

Parameters:

hds		- Handle to the DS object.

xco		- X coordinate of end point of the line to draw

yco		- Y coordinate of end point of the line to draw

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw a line from the current position set in the specified DS to the end point specified by xco and yco. It draws pixels starting at the current position, up to, but not including the point specified by xco, yco. The pixels are drawn using the current foreground color, background color, background transparency mode and drawing mode set in the DS.

This function leaves the current position in the DS unchanged. It will fail if the value specified by hds is not a valid DS handle.

##### MTDS::Line(hds, ppnt)

Parameters:

hds		- Handle to the DS object

ppnt		- Pointer to a PNT structure specifying the endpoint of the line.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw a line from the current position in the specified DS to the endpoint specified by the values stored in the specified PNT structure. This function is identical in behavior to the Line(hdc, xco, yco) function described above except that the endpoint of the line is specified by the PNT structure rather than separate X and Y values.

This function leaves the current position in the DS unchanged. It will fail if the value specified by hds it not a valid DS handle.

##### MTDS::LineTo(hds, xco, yco)

Parameters:

hds		- Handle to he DS object.

xco		- X coordinate of the endpoint of the line

yco		- Y coordinate of the endpoint of the line.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw a line from the current position store in the DS to the endpoint specified by xco and yco. It draws pixels starting at the current position, up to, but not including the point specified by xco, yco. The pixels are drawn using the current foreground color, background color, background transparency mode and drawing mode set in the DS.

This function updates the current position in the DS to the endpoint specified by xco, yco. This function is identical to the function Line(hds, xco, yco) described above except that it updates the current position. Using this function, it is possible to draw a series of connected line segments by calling MoveTo() to set the starting point followed by a sequence of calls to LineTo() for the endpoint of each line segment.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::LineTo(hds, ppnt)

Parameters:

hds		- Handle to the DS object.

ppnt		- Pointer to a PNT structure specifying the endpoint of the line.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw a line from the current position stored in the specified DS to the endpoint specified by the PNT object referenced by ppnt. This function is identical to the function LineTo(hds, xco, yco) described above except that the endpoint is specified using a PNT structure rather than separate X and Y values.

##### MTDS::PolyLine(hds, cpnt, rgpnt)

Parameters:

hds		- Handle to the DS object

cpnt		- Number of PNT structures in rgpnt.

rgpnt		- Pointer to array of PNT structures giving endpoints of lines

Return Value:

None

Errors:

Returns true if successful, false if it fails.

Description:

This function will draw a series of connected line segments. The parameter rgpnt points to an array of PNT structures giving the endpoints of the line segments. The first line segment begins at the current position stored in the DS. Each element in the rgpnt array specifies the endpoint of one line segment and the starting point of the next line segment. The parameter cpnt specifies the number of elements in the rgpnt array and the number of line segments that will be drawn. The line segments will be drawn using the current foreground color, background color, pen pattern, background transparency mode and drawing mode stored in the specified DS.

This function does not modify the current position stored in the DS.

Except for not modifying the current position in the DS, the result of this function is the same as if LineTo() were called for each point in the rgpnt array. The time required will be less, though, because of the reduced overhead of sending a single command to the display board rather than multiple commands.

##### MTDS::PolyLineTo(hds, cpnt, rgpnt)

Parameters:

hds		- Handle to the DS object.

cpnt		- Number of PNT structures in the rgpnt array.

rgpnt		- Pointer to array of PNT structures giving endpoints of the lines

Return Value:

None

Errors:
	Returns true if successful, or false if it fails.

Description:

This function will draw series of connected line segments. The line segments drawn begin at the current position stored in the DS. The parameter rgpnt specifies an array of PNT structures giving the points to be connected by the line segments. The parameter cpnt specifies the number of PNT structures in the array specified by rgpnt. This function leaves the current position in the DS set to the coordinates of the endpoint of the last line segment drawn.

Other than modifying the current position in the DS at the end, this function is identical to the PolyLine() function described above.

##### MTDS::Arc(hds, xcoLeft, ycoTop, xcoRight, ycoBottom, xcoRad1, ycoRad1, xcoRad2, ycoRad2)

Parameters:

hds		- Handle to the DS object.

xcoLeft	- X coordinate of left side of the bounding rectangle.

ycoTop	- Y coordinate of top side of the bounding rectangle.

xcoRight	- X coordinate of right side of the bounding rectangle.

ycoBottom	- Y coordinate of the bottom side of the the bounding rectangle.

xcoRad1	- X coordinate of endpoint of starting radial line.

ycoRad1	- Y coordinate of the endpoint of starting radial line.

xcoRad1	- X coordinate of endpoint of ending radial line.

ycoRad21	- Y coordinate of endpoint of ending radial line.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function draws an elliptical arc segment. The ellipse is specified by a bounding rectangle described by xcoLeft, ycoTop giving the coordinates of the upper left corner and xcoRight, ycoBottom giving the lower right corner. The ellipse is tangent to the specified bounding rectangle.

The parameters xcoRad1, ycoRad1 and xcoRad2, ycoRad2 specify the endpoints of two radial lines from the center of the ellipse, giving the starting angle and ending angle of the arc. The arc segment is drawn starting at the intersection of the perimeter of the ellipse and radial line 1 clockwise until it reaches the intersection of the perimeter of the ellipse and radial line 2.

It is does not matter whether xcoRad1, ycoRad1 and xcoRad2, ycoRad2 specify points inside or outside of the ellipse. The radial lines that they specify simply set the starting and ending angles of the arc segment drawn.

The arc segment is drawn using the current foreground color, background color, pen pattern, background transparency mode and drawing mode stored in the DS. The current position is not used or modified.

If the endpoints xcoRad1, ycoRad1 and xcoRad2, ycoRad2 specify the same point, a complete ellipse is drawn.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::ArcTo(hds, xcoLeft, ycoTop, xcoRight, ycoBottom, xcoRad1, ycoRad1, xcoRad2, ycoRad2)

Parameters:

hds		- Handle to the DS object

xcoLeft	- X coordinate of left side of the bounding rectangle.

ycoTop	- Y coordinate of top side of the bounding rectangle.

xcoRight	- X coordinate of right side of the bounding rectangle.

ycoBottom	- Y coordinate of the bottom side of the the bounding rectangle.

xcoRad1	- X coordinate of endpoint of starting radial line.

ycoRad1	- Y coordinate of the endpoint of starting radial line.

xcoRad1	- X coordinate of endpoint of ending radial line.

ycoRad21	- Y coordinate of endpoint of ending radial line.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function draws an elliptical arc segment. The ellipse is specified by a bounding rectangle described by xcoLeft, ycoTop giving the coordinates of the upper left corner and xcoRight, ycoBottom giving the lower right corner. The parameters xcoRad1, ycoRad1 and xcoRad2, ycoRad2 specify the endpoints of two radial lines from the center of the ellipse.

A line segment is drawn from the current position stored in the DS to the beginning point of the arc. The arc segment is then drawn starting at the intersection of the perimeter of the ellipse and radial line 1 clockwise until it reaches the intersection of the perimeter of the ellipse and radial line 2. The current position in the DS is updated to the final point on the elliptical arc segment.

The initial line segment and the arc segment are drawn using the current foreground color, background color, pen pattern, background transparency mode and drawing mode stored in the DS. The current position is not used or modified.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::Ellipse(hds, xcoLeft, ycoTop, xcoRight, ycoBottom)

Parameters:

hds		- Handle to the DS object

xcoLeft	- X coordinate of left side of the bounding rectangle.

ycoTop	- Y coordinate of the top side of the bounding rectangle.

xcoRight	- X coordinate of the right side of the bounding rectangle.

ycoBottom	- Y coordinate of the bottom side of the bounding rectangle.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw and fill an ellipse. The shape and placement of the ellipse are described using a bounding rectangle. The ellipse will be drawn tangent to the sides of the specified bounding rectangle. The parameters xcoLeft and ycoTop specify the coordinate of the upper left corner of the bounding rectangle. The parameters xcoRight and ycoBottom specify the coordinate of the lower right corner of the rectangle.

The perimeter of the ellipse is drawn using the current foreground color, background color, pen pattern, background transparency mode and drawing mode set in the DS. The perimeter is drawn in a clockwise direction, starting at the upper center of the ellipse. The interior of the ellipse is filled using the current brush pattern stored in the DS.

If the current pen in the DS is set to penNull the perimeter is not drawn. If the current brush is set to hbrNull, the interior is not filled.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::Rectangle(hds, xcoLeft, ycoTop, xcoRight, ycoBottom)

Parameters:

hds		- Handle to the DS object

xcoLeft	- X coordinate of left side of the rectangle.

ycoTop	- Y coordinate of the top side of the rectangle.

xcoRight	- X coordinate of the right side of the rectangle.

ycoBottom	- Y coordinate of the bottom side of the rectangle.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw the perimeter of and fill the interior of a rectangle. The parameters xcoLeft and ycoTop specify the coordinate of the upper left corner of the rectangle, and the parameters xcoRight and ycoBottom specify the coordinate of the lower right corner.

The perimeter is drawn starting at the upper left corner, proceeding clockwise around the rectangle. It is drawn using the the current foreground color, background color, pen pattern, background transparency mode and drawing mode stored in the DS. The interior of the rectangle is filled using the current brush stored in the DS.

The perimeter will not be drawn if the pen is set to penNull. The interior will not be filled if the brush is set to hbrNull.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::RoundRect(hds, xcoLeft, ycoTop, xcoRight, ycoBottom, dxcoArc, dycoArc)

Parameters:

hds		- Handle to the DS object.

xcoLeft	- X coordinate of left side of the rectangle.

ycoTop	- Y coordinate of the top side of the rectangle.

xcoRight	- X coordinate of the right side of the rectangle.

ycoBottom	- Y coordinate of the bottom side of the rectangle.

dxcoArc	- Width of the arc segments at the corners

dycoArc	- Height of the arc segments at the corners.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will drawn and fill a rectangle with rounded corners. The parameters xcoLeft and ycoTop specify the coordinate of the upper left corner of the rectangle, and the parameters xcoRight and ycoBottom specify the coordinate of the lower right corner. The corners are drawn using arc segments with a width of dxcoArc pixels and a height of dycoArc pixels.

The perimeter is drawn starting at the left end of the upper line segment, then proceeding clockwise around the figure. The perimeter is drawn using the current foreground color, background color, pen pattern, background transparency mode and drawing mode stored in the DS. The interior is filled using the current brush stored in the DS.

The perimeter is not drawn if the pen pattern is set to penNull. The interior is not drawn if the brush is set to hbrNull.

This function will produce unpredictable results if the width of the specified rectangle is less than 2 * dxcoArc or the height is less than 2 * dycoArc.

##### MTDS::TextOut(hds, xco, yco, cchText, rgchText)

Parameters:

hds		- Handle to the DS object.

xco		- X coordinate of starting point of the text.

yco		- Y coordinate of starting point of the text.

cchText	- Number of characters in the text string to draw.

rgchText	- Array of characters for the string to draw.

Return Value:

None

Errors:

Returns true if successful, false if it fails.

Description:

This function will draw the specified text string at the specified location. The text is drawn starting at the location specified by xco and yco. This specifies the location of the upper left corner of the first character drawn. The number of characters drawn is specified by cchText and the characters are contained in the array specified by rgchText.

The text is drawn using the current foreground color, background color, background transparency mode, drawing mode and font stored in the DS.

Note that when a DS if first obtained by calling GetDisplayDs() or GetDs(), the font is initialized to null. It is necessary to set a font into the DS by calling SetFont() before text can be drawn.

The length of the text to be drawn is specified by cchText, so it is not necessary for the array of characters specified by rgchText to be terminated by '\0' as is the case with a usual “C” string. The range of character code values that can be rendered is determined by the font selected.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::TextOut(hds, cchText, rgchText)

Reserved for future use.

##### MTDS::TextOut(hds, szText)

Reserved for future use.

##### MTDS::GetTextExtents(hds, cchText, rgchText, ppntExt)

Parameters:

hds		- Handle to the DS object.

cchText	- Number of characters in the text string

rgchText	- Pointer to the array of characters for the text string

ppntExt	- Pointer to a PNT structure to be filled in.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used to determine the bounding rectangle of the specified text when printed using the properties currently set in the specified DS. The parameter ppntExt points to a PNT structure that will be filled in. The xco member will be set to the width of the bounding rectangle, and the yco member will be set to the height. The width value returned is to the last pixel that would be drawn for the final character in the given string and does not include any inter-character spacing that would occur to the right of the character if it were printed in the middle of a string.

This function will fail if the value specified by hds is not a valid DS handle.

##### MTDS::BitBlt(hdsDst, xcoDst, ycoDst, dxco, dyco, hdsSrc, xcoSrc, ycoSrc, rop)

Parameters:

hdsDst		- Handle to the destination DS.

xcoDst		- X coordinate of upper left corner of destination rectangle.

ycoDst		- Y coordinate of upper left corner of destination rectangle.

dxco		- Width of the destination rectangle in pixels.

dyco		- Height of the destination rectangle in pixels.

hdsSrc		- Handle to the source DS.

xcoSrc		- X coordinate of the upper left corner of the source rectangle.

ycoSrc		- Y coordinate of the upper left corner of the source rectangle.

rop		- Combining mode to use when copying the pixels.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function copies a rectangular region of pixels from one bitmap to another bitmap or to another location within the same bitmap. The source and destination bitmaps are the drawing surface bitmaps associated with the drawing state objects specified by hdsSrc and hdsDst respectively. HdsSrc and hdsDst can be the same value, allowing pixels to be copied within the same bitmap.

The width and height of the rectangle of pixels to be copied is specified by dxco and dyco respectively. The upper left corner of the rectangle in the source bitmap is specified by xcoSrc and ycoSrc. The upper left corner of the rectangle in the destination bitmap is specified by xcoDst and ycoDst.

The source and destination regions can be in the same or different bitmaps and if in the same bitmap are allowed to overlap arbitrarily.

The raster operation code specified by rop indicates how the source pixels are to be combined with the existing pixels in the destination as the copy is performed. In some cases, depending on the raster operation code, pixels of the brush currently selected in the destination DS are also used in the combining operation. The raster operation code specifies a boolean operation to be performed on the source, destination and brush pixels to compute the new pixel value to be stored in the destination. See the section on Raster Operations for more detailed information about raster operation codes. See the section on Bit Block Transfer above for more information about BitBlt in general.

This function will fail if hdsSrc or hdsDst specify invalid DS handles, or the raster operation code specified by rop is invalid.

##### MTDS::PatBlt(hds, xco, yco, dxco, dyco, rop)

Parameters:

hds		- Handle to the DS object.

xco		- X coordinate of the upper left corner of the destination rectangle.

yco		- Y coordinate of the upper left corner of the destination rectangle.

dxco		- Width of the destination rectangle.

dyco		- Height of the destination rectangle.

rop		- Combining mode to use.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will modify a rectangular region in the bitmap assigned as the drawing surface for the specified DS by combining pixels in the bitmap region with pixels in the brush currently selected in the DS. The way that brush pixels are combined with bitmap pixels is specified by the raster operation code specified by rop.

The upper left corner of the rectangle in the bitmap is specified by xco and yco. The width and height of the rectangle is specified by dxco and dyco. Raster operation codes that specify the use of source pixels are not valid for use with this function.

See the section on Raster Operations above for more detailed information.

This function will fail if the value specified by hds is not a valid DS handle, or the raster operation code specified by rop is not valid for use with this function.

##### MTDS::DrawBitmap(hdsDst, xcoDst, ycoDst, dxco, dyco, hdsSrc, xcoSrc, ycoSrc)

Parameters:

hdsDst		- Handle to the destination DS object

xcoDst		- X coordinate of upper left corner of the destination rectangle.

ycoDst		- Y coordinate of upper left corner of the destination rectangle.

dxco		- Width of the source and destination rectangles.

dyco		- Height of the source and destination rectangles.

hdsSrc		- Handle to the source DS object.

xcoSrc		- X coordinate of upper left corner of the source rectangle.

ycoSrc		- Y coordinate of upper left corner of the source rectangle.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function copies a rectangular region of pixels from one bitmap, the source, to a second bitmap, the destination. The source and destination bitmaps are the drawing surface bitmaps associated with the DS objects specified by hdsSrc and hdsDst respectively.

The width and height of the rectangle of pixels to be copied is specified by dxco and dyco respectively. The upper left corner of the region in the source bitmap is specified by xcoSrc and ycoSrc. The upper left corner of the region in the destination bitmap is specified by xcoDst and ycoDst.

This function makes uses of the current background transparency mode and transparency color stored in the destination DS when copying pixels. If the transparency mode is set to bkOpaque, all pixels from the source bitmap are copied to the destination bitmap. If the transparency mode is set to bkTransparent, pixels from the source bitmap whose color matches the transparency color are not copied. This results in the destination pixels not being modified anywhere that the source bitmap is “transparent”, allowing the destination to “show through” in the result.

In addition to transparency, DrawBitmap() also uses the intensity value currently set in the destination DS. The intensity value is used to scale the brightness of pixels being copied from the source to the destination. The intensity value is interpreted as a percentage, and each color component of each pixel copied is scaled by this percentage. If the intensity is set to 100 (meaning 100%), the pixels are copied unchanged. Otherwise, the RGB color components of each pixel are multiplied by the percentage value specified by the current intensity. For example: if the intensity is set to 50 (meaning 50%), each pixel that is copied will be drawn with the brightness being half of the source pixel.

This function is similar to the BitBlt() function described above, except that it doesn't use a raster operation code for combining mode and instead copies pixels with transparency and intensity.

This function is intended to be used to copy pixels between two different bitmaps. If the source and destination bitmaps are the same, it performs a BitBlt() instead using the ropSrcCopy raster operation code.

#### Graphics System Rectangle Operation Functions

The MTDS system provides a set of API functions that can perform logical operation on rectangles. Additionally, functions are provided for rendering rectangular regions on a bitmap in various ways.

##### MTDS::IsRectEmpty(prct)

Parameters:

prct		- Pointer to the RCT structure.

Return Value:

Returns true if the specified rectangle is empty (null).

Errors:

None

Description:

This function tests whether the specified rectangle is empty, or null. An empty rectangle is one where the width or the height are less than or equal to 0. A width less than 0 means that the right side is to the left of the left side. Similarly a height of less than 0 means that the bottom is above the top.

##### MTDS::IsRectEqual(prct1, prct2)

Parameters:

prct1		- Pointer to the first RCT structure.

Prct2		- Pointer to the second RCT structure.

Return Value:

Returns true if the two rectangles are identical.

Errors:

None

Description:

This function tests if the two rectangles specified by prct1 and prct2 are identical, meaning the their width, height, and positions are the same.

##### MTDS::PointInRect(prct, xco, yco)

Parameters:

prct		- Pointer to the RCT structure.

xco		- X coordinate of the point.

yco		- Y coordinate of the point.

Return Value:

Returns true if the specified point is within the specified rectangle.

Errors:

None

Description:

This function tests to see if the point specified by xco and yco is contained within the rectangle specified by prct. It returns true if the specified point is within the rectangle or false if it is not. Points on the perimeter of the rectangle are consider to be within it.

##### MTDS::PointInRect(prct, pnt)

Parameters:

prct		- Pointer to the rectangle.

Pnt		- PNT structure specifying the point.

Return Value:

Returns true if the specified point is within the specified rectangle.

Errors:

None

This function tests to see if the specified point is contained within the specified rectangle. This is identical to the function PointInRect(prct, xco, yco) described above except that the point is specified via a PNT structure rather than separate X and Y values.

##### MTDS::SetRectEmpty(prct)

Parameters:

prct		- Pointer to the RCT structure.

Return Value:

None

Errors:

None

Description:

This function sets the specified rectangle to null by setting all of its members to zero.

##### MTDS::SetRect(prct, xcoLeft, ycoTop, xcoRight, ycoBottom)

Parameters:

prct		- Pointer to the RCT structure.

XcoLeft	- X coordinate of the top left corner of the rectangle.

YcoTop	- Y coordinate of the top left corner of the rectangle.

XcoRight	- X coordinate of the bottom right corner of the rectangle.

YcoBottom	- Y coordinate of the bottom right corner of the rectangle.

Return Value:

None

Errors:

None

Description:

This function initializes the RCT structure specified by prct to the specified values.

##### MTDS::SetRect(prctDst, prctSrc)

Parameters:

prctDst		- Pointer to the destination RCT structure.

prctSrc		- Pointer to the source RCT structure.

Return Value:

None

Errors:

None

Description:

This function initializes the RCT structure specified by prctDst to the values contained in the RCT structure specified by prctSrc.

##### MTDS::MoveRect(prct, dxco, dyco)

Parameters:

prct		- Pointer to the RCT structure.

dxco		- Amount to offset in X.

dyco		- Amount to offset in Y.

Return Value:

None

Errors:

None

Description:

This function moves the rectangle specified by prct by the amount specified by dxco and dyco. Essentially, this means that dxco is added to the X members and dyco added to the Y members of the rectangle.

##### MTDS::GrowRect(prct, dxco, dyco)

Parameters:

prct		- Pointer to the RCT structure.

dxco		- Amount to grow in X.

dyco		- Amount to grow in Y.

Return Value:

None

Errors:

None

Description:

This function will grow or shrink the rectangle specified by prct by the amount specified by dxco and dyco. The position of the upper left corner of the rectangle stays the same and the lower right corner is moved by the specified amount.

##### MTDS::IntersectRect(prctDst, prctOne, prctTwo)

Parameters:

prctDst		- Pointer to the RCT structure to receive the result.

prctOne	- Pointer to the RCT structure giving the first rectangle.

prctTwo	- Pointer to the RCT structure giving the second rectangle.

Return Value:

None

Errors:

None

Description:

This function computes the intersection of the two rectangles specified by prctOne and prctTwo. The intersection of two rectangles is the rectangle giving the common area in which they overlap. If the two rectangles don't overlap, the resulting rectangle will be null, or empty.

The result is placed in the RCT structure specified by prctDst. The rectangle specified by prctDst can be the same RCT structure as specified by either prctOne or prctTwo.

##### MTDS::UnionRect(prctDst, prctOne, prctTwo)

Parameters:

prctDst		- Pointer to the RCT structure to receive the result.

PrctOne	- Pointer to the RCT structure giving the first rectangle.

PrctTwo	- Pointer to the RCT structure giving the second rectangle.

Return Value:

None

Errors:

None

Description:

This function computes the rectangle that is the union of the rectangles specified by prctOne and prctTwo. The union of two rectangles is the minimum rectangle that contains both of the specified rectangles.

The result is placed in the RCT structure specified by prctDst. The rectangle specified by prctDst can be the same RCT structure as specified by either prctOne or prctTwo.

##### MTDS::FrameRect(hds, prct)

Parameters:

hds		- Handle to the DS object

prct		- Pointer to a RCT structure

Return Value:

None

Errors:

Returns true if successful, false if it fails.

Description:

This function draws a border around the rectangle specified by prct. It uses the current foreground color, background color, background transparency mode and drawing mode selected in the DS specified by hds. It does not modify the current position in the DS.

This function fails if the value specified by hds is not a valid handle to a DS object.

##### MTDS::FillRect(hds, prct, hbr)

Parameters:

hds		- Handle to the DS object.

prct		- Pointer to a RCT structure.

hbr		- Handle to the brush to use.

Return Value:

None

Errors:

Returns true if it succeeds, or false if it fails.

Description:

This function fills the rectangle specified by prct, using the brush specified by hbr, using the ropPatCopy raster operation.

This produces the same result as would be achieved by setting the specified brush as the fill brush for the DS and then using PatBlt() with the ropPatCopy raster operation code.

This function will fail if the value of hds is not a valid DS handle, or the value of hbr is not a valid brush handle.

##### MTDS::InvertRect(hds, prct)

Parameters:

hds		- Handle to the DS object.

prct		- Pointer to a RCT structure.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function inverts the pixels within the rectangle specified by prct. Inverting a pixel means replacing its value with the boolean complement of its value. In other words, inverting its bits. This results in the pixels having approximately a complementary color.

This achieves the same result as would be achieved by using PatBlt() with the ropDstInvert raster operation.

This function will fail if the value of hds is not a valid DS handle.

## MTFS Class Interface Functions (Methods)

The MTFS object class provides a set of interface functions used to access the file system on a micro-SD card in the connector on the display board.

##### MTFS::GetLastError(pcls, pcmd)

Parameters:

pcls	- Pointer to variable to receive command class

pcmd	- Pointer to variable to receive command code

Return Values:

Returns the error status value for the last MTFS operation.

Errors:

None

Description:

This function can be used to determine if the last MTFS operation attempted was successful or not. Generally, most functions return an indication of whether or not they succeeded. If a function fails more detailed information about the nature of the failure can be obtained using this function.

The return value of the function will be an error code, or the value staCmdSuccess if no error occurred. The variable pointed to by pcls will be set to the command class of the last command sent to the display board. The variable pointed to by pcmd will be set to the command code of the last command sent to the display board. Interpretation of the status code returned depends on the command class of the command that failed.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTFS::GetLastError()

Parameters:

none

Return Values:

Returns the error status value for the last MTFS operation.

Errors:

None

Description:

This variant of the GetLastError() function only returns the error status of the last MTDS operation without returning the class or command code of the last operation.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTFS::GetElapsedTime()

Parameters:

None

Return Value:

This function returns the execution time of the last MTFS function

Errors:

None

Description:

This function returns the elapsed time in milliseconds for the execution of the last MTFS function. This includes the time required to send the command packet and any required data packets to the display board, the time for the display board to execute the command, and the time for the display board to send any response or return data packets back to the host board.

The time values are obtained using the millis() function in the system runtime. If, for some reason, the millis() function is not working correctly, the values returned by this function will be incorrect.

##### MTFS::MountFs(fMount, fDelay)

Parameters:

fMount	- True to mount, false of unmount the file system

fDelay		- True to delay mounting, false to mount immediately

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function is used mount or unmount the file system on an SD card plugged in to the uSD connector on the display board. A file system must be mounted before any access can be made to the SD card. It is not strictly necessary, but it is good practice to unmount the file system before removing and SD card from the connector.

When this function returns true, it means that an SD card was detected, it was successfully initialized and that there is a valid file system on the card.

The fDelay parameter allows specifying to delay mounting the file system. If fDelay is false, the file system is mounted and validated immediately. If fDelay is true, the file system library code is initialized, but no check is made for the presence of an SD card or the validity of any file system. In this case, MountFs() will always return true. The actual mounting of the file system will occur on the first function call that requires access to the SD card.

##### MTFS::GetFreeSpace(pdwFree)

Parameters:

pdwFree	- Pointer to variable to receive the free space on the SD card

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function returns the number of free sectors on the SD card. The parameter pdwFree points to a unsigned 32-bit integer to receive the number of free sectors. The standard sector size of FAT file systems on SD cards uses 512 byte sectors, and this is the only sector size supported by the file system code in the MTDS system.

##### MTFS::SetCurrentDir(szDir)

Parameters:

szDir		- Pointer to string giving path for current directory

Return Value:

none

Errors:

Returns true if successful, false if not.

Description:

This function sets the path to the current directory on the SD card. This directory becomes the root of any relative paths specified in subsequent names passed to the file system functions.

##### MTFS::GetCurrentDir(cbMax, szDir)

Parameters:

cbMax		- Size of buffer specified by szDir

szDir		- Buffer to receive the string for current directory.

Return Value:

none

Errors:

Returns true if successful, false if an error occurs

Description:

This function returns a string giving the current directory for the SD card. This is the last value set by SetCurrenteDir(). The parameter cbMax specifies the size of the buffer to receive the string. The maximum number of characters returned will be cbMax-1.

##### MTFS::SetVolumeLabel(szVol)

Reserved for future use.

##### MTFS::GetVolumeLabel(cbMax, szVol)

Reserved for future use.

##### MTFS::CreateDir(szDir)

Parameters:

szDir		- String specifying the name of the directory to create.

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function creates a subdirectory on the SD card. The directory will be created in whatever directory is set as the current directory, or the root of the file system if no current directory has been set.

##### MTFS::OpenDir(szDir)

Parameters:

szDir		- String specifying the name of the directory to open

Return Value:

Returns a handle to the opened directory.

Errors:

Returns 0 if an error occurs.

Description:

This function will open the specified directory for read access. If the function succeeds, it will return a handle of type HDIR that can be passed to ReadDir() or CloseDir().

Typically, opening and reading from a directory is used to search through the file entries in the directory to discover what files or sub-directories are contained within the directory.

Once access to the directory is no longer needed, it should be closed using the CloseDir() function to release the handle and any system resources used.

##### MTFS::ReadDir(hdir, pfinf)

Parameters:

hdir		- Handle of a previously opened directory

pfinf		- Pointer to a FINF structure to receive the directory information

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function is used to read the entries contained within the previously opened directory specified by hdir. Each time that ReadDir() is called, it returns information about the next file in sequence in the directory. When the end of the directory is reached, ReadDir() will return false. To determine that it is actually the end of the directory rather than some other error, GetLastError() will return the value staFsFileNotFound to indicate that the end of the directory was reached.

The information about each directory entry is returned in the FINF structure specified by the parameter pfinf. See the section above describing the file system for more information about the FINF structure and searching directories.

##### MTFS::CloseDir(hdir)

Parameters:

hdir		- Handle to a previously opened directory

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function is used to close a directory previously opened by OpenDir() when access to the directory is not longer needed. A directory should always be closed after access is no longer needed to recover system resources being used to track the open directory.

##### MTFS::FileStat(szFile, pfinf)

Parameters:

szFile		- String giving the name of a file

pfinf		- Pointer to FINF structure to receive information about file

Return Value:

none

Errors:

Returns true if successful, false if an error occurs.

Description:

This function returns information about the file specified by the file name string in szFile. The file name string can specify a full path or can be a relative path from the current directory.

The information about the file is returned by filling in the FINF structure specified by pfinf. See the section above about the SD File System for more information about the fields defined in the FINF structure.

##### MTFS::OpenFile(szFile, md)

Parameters:

szFile		- String giving the file name

md		- File access mode

Return Value:

Returns a file handle (HFIL) for access to the file

Errors:

Returns 0 if an error occurs.

Description:

This function is used to create a new file or open an existing file for access. The file name is specified by szFile and can either be a full path to the file or a path relative to the current directory.

The type of access requested for the file is specified by md, which contains a bitwise of of the following options:

mdMtdsRead		- open the file for read access

mtMtdsWrite		- open the file for write access

mdMtdsOpenExisting	- open an existing file (i.e. do not create a new file)

mtMtdsOpenAlways	- open an existing file or create a new one if the file doesn't exist

mtMtdsCreateNew	- only create the file if it does't exist

mtMtdsCreateAlways	- create a new file whether it already exists or not

If the function succeeds, it returns a handle that can be use for further access to the file. Once access to the file is no longer required, it should be closed by calling CloseFile() to release system resources used by the open handle.

##### MTFS::CloseFile(fh)

Parameters:

fh		- File handle of the file to close

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function closes a file handle previously opened by OpenFile(). This releases system resources being used by the open handle. File handles should always be closed when access to the file is no longer required.

##### MTFS::ReadFile(fh, rgbBuf, cbRead, pcbRes)

Parameters:

fh		- File handle for the file to read

rgbBuf		- Pointer to buffer to receive the bytes read from the file

cbRead	- Number of bytes to read from the file

pcbRes		- Pointer to variable to receive number of bytes actually read

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will read the specified number of bytes from the open file referenced by fh. The file must have been opened for read access when it was opened. It starts reading at the current file pointer location within the file and will read until the specified number of bytes have been read or the end of file is reached. The data read is placed in the buffer specified by rgbBuf. The file pointer is updated to the next location in the file following the returned data.

The variable pointed to by pcbRes will be filled in with the number of bytes actually read. This may be less than the number of bytes requested if the end of file is reached or an error occurs while reading from the file. Failure to return the requested number of bytes because the end of the file was reached is not an error and the function will return true in this case.

##### MTFS::WriteFile(fh, rgbBuf, cbWrite, pcbRes)

Parameters:

fh		- File handle for file to write

rgbBuf		- Pointer to buffer containing data to write to the file

cbWrite	- Number of bytes to write to the file

pcbRes		- Pointer to variable to receive number of bytes written

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will write the specified number of bytes to the open file referenced by fh. The file must have been opened for write access when it was opened. It writes data from the buffer specified by rgbBuf to the file starting at the current file pointer location. The file pointer is updated to the next location in the file after the write is completed.

The variable pointed to by pcbRes will be filled in with the actual number of bytes written. This may be less than the number of bytes requested if an error occurs or the SD card becomes full.

##### MTFS::SyncFile(fh)

Parameters:

fh		- File handle for the file to synchronize

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will synchronize the file data on the SD card with what is currently in memory. This will flush any buffer currently held in memory to the disk and update the file directory information. This can be down following WriteFile() to ensure that the data written has actually been written to the SD card.

##### MTFS::TruncateFile(fh)

Parameters:

fh		- File handle of file to truncate

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function truncates the file referenced by fh to the current file pointer location. Any data previously in the file following the current file pointer location is no longer part of the file and any sectors after the current file pointer location previously allocated to the file are freed.

##### MTFS::SetFilePointer(fh, off)

Parameters:

fh		- File handle of open file

off		- File read/write pointer location to set

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function sets the read/write pointer location with the file. The read/write pointer location is the byte index from the beginning of the file where the next data will be read from or written to. When a file is first opened, the read/write pointer location is set to 0, It is automatically advanced as reads or writes are done. This function is used to reposition the read/write pointer within the file to allow random access to data within the file.

If the file was opened for write access, the file pointer location can be set to a position past the current end of the file. If this is done, the file size will be extended to the position set.

##### MTFS::GetFilePointer(fh, poff)

Parameters:

fh		- File handle of open file

poff		- Pointer to variable to receive the current file pointer position

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function returns the current position of the file read/write pointer. The file pointer position is an unsigned 32-bit integer giving the byte offset from the beginning of the file to the next location in the file to be read from or written to.

##### MTFS::DeleteFile(szFile)

Parameters:

szFile		- String giving file name of file to delete

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function deletes the named file from the SD card.The string in szName can be a full path to the file or a path relative to the current directory.

##### MTFS::RenameFile(szOld, szNew)

Parameters:

szOld		- String giving the current name of the file

szNew		- String giving the new name to give to the file

Return Value:

none

Errors:

Returns true if successful, or false if and error occurs.

Description:

This file will change the name of an existing file on the SD card. The name of the file specified by szOld will be changed to the name given by szNew. If szOld and szNew specify paths in addition to the file name, the path must be identical in each string. This function does not move a file from one directory to another, it simply changes the name of the file in the directory where it resides.

##### MTFS::SetFileAttrib(szFile, attrib, mask)

Parameters:

szFile		- String giving the file name

attrib		- New attribute values to set

mask		- Bit mask specifying which attribute values to modify

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will modify file attribute bits for the file specified by szFile. The parameter attrib contains the new attribute values. The value in mask is a bit mask specifying which attribute bits are to be modified. Only the attribute bits where there is a corresponding '1' bit in mask will be modified and will be set to the corresponding bit value specified by attrib.

The following file attribute bits are defined:

atMtdsReadOnly	- the file is read only

atMtdsHidden		- the file is hidden

atMtdsSystem		- the file is a system file

atMtdsVolume	- the file name is actually a volume label

atMtdsDirectory	- the file is actually a sub-directory

atMtdsArchive	- the file has been modified

Setting the atMtdsVolume or atMtdsDirectory bits may cause unpredictable and probably erroneous behavior.

##### MTFS::SetFileTime(szFile, pfinf)

Parameters:

szFile		- String giving the file name

pfinf		- Pointer to a FINF structure containing the date and time to set

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function will set the modification date and time for the specified file to the values contained in the FINF structure specified by pfinf.

##### MTFS::GetFileSize(fh, pcb)

Parameters:

fh		- File handle for an open file

pcb		- Pointer to an unsigned 32-bit variable to receive the file size

Return Value:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function returns the size, in bytes, of the open file referenced by the file handle in fh. The value if returned in the 32-bit unsigned integer variable specified by pcb.

##### MTFS::Feof(fh)

Parameters:

fh		- File handle of an open file

Return Value:

Returns true if the file pointer is at the end of the file, else false

Errors:

Returns true if an error occurs.

Description:

This function is used to check if the file read/write pointer is at the end of the file. This function will return true if the file pointer is at the end of the file, or false if not.

This function will return true, i.e. end of file, if the file handle specified by fh is invalid. It also returns true if any other error occurs. To distinguish between an error return and a correct return of the state of the file pointer, the return value from GetLastError() can be used to disambiguate. GetLastError() returns staCmdSuccess if the function completed without error.

##### MTFS::Ferr(fh)

Parameters:

fh		- File handle of an open file

Return Value:

Returns the last error state of the specified file handle

Errors:

Returns -1 if an error occurs in processing the function.

Description:

This function returns the error status of the last command to operate on the specified file handle. The error codes returned are a subset of the file system error status codes defined in ProtoDefs.h. See the above sections on the fSD Fle System and Error Reporting for more information.

## MTWS Class Interface Functions (Methods)

The MTWS object class provides a set of interface functions used to access the windowing system. See the section above on the Windowing System for more detailed information about the windowing system and how it is used.

##### MTWS::GetLastError(pcls, pcmd)

Parameters:

pcls	- Pointer to variable to receive command class

pcmd	- Pointer to variable to receive command code

Return Values:

Returns the error status value for the last MTWS operation.

Errors:

None

Description:

This function can be used to determine if the last MTWS operation attempted was successful or not. Generally, most functions return an indication of whether or not they succeeded. If a function fails more detailed information about the nature of the failure can be obtained using this function.

The return value of the function will be an error code, or the value staCmdSuccess if no error occurred. The variable pointed to by pcls will be set to the command class of the last command sent to the display board. The variable pointed to by pcmd will be set to the command code of the last command sent to the display board. Interpretation of the status code returned depends on the command class of the command that failed.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTWS::GetLastError()

Parameters:

none

Return Values:

Returns the error status value for the last MTWS operation.

Errors:

None

Description:

This variant of the GetLastError() function only returns the error status of the last MTWS operation without returning the class or command code of the last operation.

All of the symbols related to the error system and the communications protocol between the MTDS library and the display board are defined in the header file ProtoDefs.h

See the description of the Error Reporting System above for more detailed information.

##### MTWS::GetElapsedTime()

Parameters:

None

Return Value:

This function returns the execution time of the last MTWS function

Errors:

None

Description:

This function returns the elapsed time in milliseconds for the execution of the last MTWS function. This includes the time required to send the command packet and any required data packets to the display board, the time for the display board to execute the command, and the time for the display board to send any response or return data packets back to the host board.

##### MTWS::CreateWindow(hwinParent, wcls, wstl, xco, yco, dxco, dyco)

Parameters:

hwinParent		- Window handle of the parent window

wcls			- Window class

wstl			- Window style

xco, yco		- Position of the window rectangle

dxco, dyco		- Width and height of the window rectangle

Return Values:

Returns the window handle of the window created

Errors:

Returns 0 if an error occurs

Description:

This function is used to create a new window within the windowing system maintained by the display board. If it succeeds, a handle (HWIN) to the newly created window is returned.

The hwinParent parameter specifies the handle to the parent window for the newly created window. For a top level window, the symbol hwinDesktop is used for the parent window, otherwise the window handle of a previously created window is specified.

The wcls and wstl parameters specify the window class and window style. In version 1 of the MTDS system, the only window class supported is wclUser. The window class specifies the particular type of behavior for the window (e.g. a button class is envisioned in a future firmware version). The window style is an unsigned 32-bit integer containing 'style bits' which specify appearance options for the window. Window style bits specify such things as whether or not the window contains a title bar, border, and so on. The value passed in the wstl parameter is the bit-wise or of the various style bits desired for the window.

The parameters xco and yco specify the position of the window. The origin is the position of the upper left corner of the bounding rectangle for the window. The window position is relative to the client area of the parent window. The client area of the hwinDesktop window is the entire surface of the physical display.

The parameters dxco and dyco specify the width and height of the bounding rectangle of the window. If the specified width and height cause the window the exceed the bounding rectangle of the parent window, the window is clipped to the size of the parent window. The window is still defined at the larger size, but any drawing within the window is clipped to the extents of the parent window.

For more detailed information about the windows, see the section on the Windowing System above.

##### MTWS::DestroyWindow(hwin)

Parameters:

hwin		- Window handle of the window to destroy

Return Values:

none

Errors:

Returns true if successful, false if an error occurs

Description:

This function destroys the window specified by hwin and release all resources used by the window back to the system. If the window specified by hwin has child window, they are destroyed as well.

##### MTWS::CloseWindow(hwin)

Reserved for future use.

##### MTWS::ShowWindow(hwin, fShow)

Parameters:

hwin		- Window handle

fShow		- True to show the window, false to hide the window

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function is used to make the window specified by hwin either visible or hidden.

##### MTWS::SetPosition(hwin, xco, yco)

Parameters:

hwin		- Window handle of the window whose position to setbackground

xco, yco	- Position within the parent window

Return Values:

none

Errors:

Returns true if successful, false if an error occurs

Description:

This function sets the origin of the window specified by hwin to the position specified by xco and yco. The origin is the upper left corner of the window. The position specified by xco,yco is relative to the client area of the parent window of hwin.

##### MTWS::SetSize(hwin, dxco, dyco)

Parameters:

hwin		- Window handle of the window whose size is to be set

dxco		- Width of window to set

dyco		- Height of window to set

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function sets the width and height of the window specified by hwin to the values specified by dxco and dyco.

##### MTWS::GetWindowPos(hwin, pxco, pyco, pdxco, pdyco)

Parameters:

hwin		- Window handle of window to query

pxco		- Pointer to variable to receive window X position

pyco		- Pointer to variable to receive window Y position

pdxco		- Pointer to variable to receive window width

pdyco		- Pointer to variable to receive window height

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function returns the position and size of the window specified by hwin. The parameters pxco and pyco point to variables to receive the x and y coordinates of the upper left corner of the window. The values returned in *pxco and *pyco are logical coordinates relative to the client area of the parent window of hwin. The parameters pdxco and pdyco point to variables to receive the width and height of the window.

##### MTWS::MoveWindow(hwin, wmv, xco, yco, dxco, dyco)

Parameters:

hwin		- Window handle of window to move

wmv		- Window move options

xco, yco	- Window position to set

dxcy, dyco	- Window width and height to set

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to move and/or change the size of the window specified by hwin. The parameters xco and yco specify the new location of the upper left corner of the window relative to the client area of the parent window of hwin. The parameters dxco and dyco specify the new width and height of the window.

The parameter wmv specifies options for the function. The following options are defined and can be or'd together to form the value passed in wmv:

wmvSetPosition	- modify the position of the window according to xco,yco

wmvSetSize		- modify the size of the window according to dxco,dyco

wmvCopyBits		- copy the contents of the window

If the wmvCopyBits option is set in wmv, the existing contents of the window will be copied to the new location.

##### MTWS::GetWindowRect(hwin, prct)

Parameters:

hwin		- Window handle of the window to query

prct		- Pointer to an RCT structure to be filled in with the window rectangle

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will return the bounding rectangle of the window specified by hwin. The bounding rectangle of the window gives the upper left and lower right corners of the entire window in physical coordinates of the display bitmap.

##### MTWS::GetClientRect(hwin, prct)

Parameters:

hwin		- Window handle of the window to query

prct		- Pointer to an RCT structure to be filled in with the client rectangle

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will return the coordinates of the client rectangle of the window specified by hwin. The client rectangle gives the extents of the client area of the window in logical coordinates relative to the window client area. This means that the upper left corner of the rectangle returned will have x and y coordinates of 0,0 and the lower right corner rectangle returned will have the width and height of the client area.

##### MTWS::BeginUpdate(hwin)

Parameters:

hwin		- Window handle of window to update

Return Values:

Returns handle to the drawing state (HDS) for the update

Errors:

Returns 0 if an error occurs.

Description:

This function is used to obtain a handle to a DS that can be used to draw on the client area of the window specified by hwin.  The DS will have been initialized to the default state, but with the origin and clipping rectangle set to draw on the client area of the window.

Once any drawing to be performed on the window has been completed, EndUpdate() must be called to release the DS and other resources used by the system during window update.

##### MTWS::EndUpdate(hwin)

Parameters:

hwin		- Window handle of the window being updated

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function is called to end drawing on a window. It is called after calling BeginUpdate() and after all drawing onto the window has been done. This releases the HDS and other resources used by the system during window update.

##### MTWS::BeginNcUpdate(hwin)

Parameters:\

hwin		- Window handle of the window to update

Return Values:

Returns handle to the drawing state (HDS) to use for the update

Errors:

Returns 0 if an error occurs

Description:

This function is used to obtain a handle to a DS that can be used to draw on a window. The difference between this function and BeginUpdate() is that this function returns a DS that has been initialized with the origin and clipping rectangle set to draw on the entire surface of the window rather than just the client area.

The non-client area of a window is the area containing the title bar, window border and so on for windows of a style such that they have a border or other non-client elements.

Once all drawing has been performed on the window, EndNcUpdate() must be called to release the HDS and any other resources used by the system during widow drawing.

##### MTWS::EndNcUpdate(hwin)

Parameters:

hwin		- Window handle of window being updated

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is called to end drawing on a window. It is called after calling BeginNcUpdate() and after all drawing onto the window has been done. This releases the HDS and other system resources used for drawing on the window.

##### MTWS::DrawBorder(hwin)

Parameters:

hwin		- Window handle of the window

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function will draw the standard non-client elements of a window. This includes such things as the title bar, any menu bar, window border and so on specified by the window style in effect for the window. BeginNcUpdate() must have been called before calling this function.

##### MTWS::SetWindowId(hwin, id)

Parameters:

hwin		- Window handle of window to set

id		- Identifier to set in the window

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs

Description:

This function is used to set the window identifier for the window specified by hwin. The window identifier is an unsigned 32-bit value that can be used to uniquely identify a specific window in the windowing system.

##### MTWS::GetWindowId(hwin)

Parameters:

hwin		- Window handle of the window to query

Return Values:

Returns the window identifier previously assigned to the window

Errors:

Returns 0 if an error occurs.

Description:

This function returns the window identifier assigned to the window specified by hwin. If no identifier has been assigned to the window using SetWindowId(), it has the default value of 0.

##### MTWS::SetWindowTitle(hwin, szTitle)

Parameters:

hwin		- Window handle of the window to set

szTitle		- Pointer to the window title string to set

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to set the window title string in the window specified by hwin. The title string is an arbitrary character string assigned to the window. For windows that have a title bar, this is the title that will be drawn in the title bar. The symbol cchTitleMax specifies the maximum allowed length for a window title.

##### MTWS::GetWindowTitle(hwin, cchMax, szTitle)

Parameters:

hwin		- Window handle of the window to query

cchMax	- Maximum number of characters to return

szTitle		- Buffer to receive the window title characters returned

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to obtain the title string assigned to the window specified by hwin. The parameters cchMax specifies the maximum number of characters to return. The returned string will be placed in the buffer specified by szTitle, which must be at least cchMax+1 characters long. If no title string has been set in the window, a null string will be returned.

##### MTWS::SetWindowValue(hwin, val)

Parameters:

hwin		- Window handle of the window to set

val		- Value to assign to the window

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to set the window value in the window specified by hwin. The window value is an unsigned 32-bit integer value that is for the use of the application program. The window system never uses the window value.

##### MTWS::GetWindowValue(hwin, pval)

Parameters:

hwin		- Window handle of the window to query

pval		- Pointer to the variable to receive the window value

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function obtains the window value currently stored in the window specified by hwin. If no value has been set in the window by calling SetWindowValue() the default window value of 0 is returned.

##### MTWS::SetWindowString(hwin, cb, rgch)

Parameters:

hwin		- Window handle of the window to set

cch		- Number of bytes of window string data to set

rgch		- Pointer to buffer of window string data to set

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to set the window string stored in the window specified by hwin. The window string is an arbitrary array of bytes that the application can use in any way that it wants. The maximum allowed size of the window value array is defined by the symbol cchWindowStringMax. The windowing system never makes use of the window string.

##### MTWS::GetWindowString(hwin, cbBuf, pcbRet, pcbMax, rgch)

Parameters:

hwin		- Window handle of the window to query

cbBuf		- Maximum number of bytes of window string data to return

pcbRet		- Pointer to variable to receive number of bytes actually returned

pcbMax	- Pointer to variable to receive total window string length

rgch		- Pointer to buffer to receive window string data

Return Values:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to obtain the window string stored in the window specified by hwin. The window string data will be returned in the buffer specified by rgch. The maximum number of bytes to return is specified by cbBuf. The parameter pcbRet points to a variable that will be filled in with the actual number of bytes returned. The parameter pcbMax points to a variable that will be filled in with the total length of the window string stored in the window.

If no window string has been stored in the window by calling SetWindowString(), no data is returned and the value placed at *pcbMax will be 0.