# MyDisp Library Programmer's Reference Manual

Rev: 2016-09-23

## Introduction

This document describes the MyDisp graphics library. This library is part of the Digilent Multi-Touch Display System (MTDS). This system is a set of hardware display boards, device firmware running on the display boards, and libraries used to access the graphics functions of the display boards. This system is intended for use with Arduino and chipKIT compatible microcontroller boards.

## Overview

The MyDisp library provides a set of simple functions used to render graphics on an MTDS compatible display board and to create simple touch user interfaces for applications using the display boards.

This library is intended for use under the Arduino, or MPIDE development environments for Arduino compatible of chipKIT compatible microcontroller boards.

This library is accessed by including two header files into the user sketch, as follows:

#include <MyDisp.h>

#include <mtds.h>

The MyDisp library defines an object class used to access its functions and instantiates an interface object called mydisp. All library functions are accessed as method calls on the mydisp object:

For example: mydisp.begin() is used to initialize the library. A call to mydisp.drawLine(0, 0, 100, 100); will draw a line from the upper left corner of the display to a point 100 pixels down and to the right of the upper left corner.

## General Concepts Used by the Library

The following descriptions provide information about some general concepts used by the MyDisp library and provide information needed for a complete understanding of how the library works. Following this are descriptions of all of the library functions.

### Coordinate System

The library uses signed 16-bit integers to represent the X and Y coordinates used in the system. This allows the coordinate system to extend between +/- 32,767 in the X and Y directions. This represents a range of possible coordinate values that greatly exceeds the screen dimensions. Any drawing that occurs outside the bounding rectangle of the display will be clipped.

The graphics system uses a left handed coordinate system with the origin (0,0) at the upper left corner of the display. X values increase to the right, Y values increase going down.

### Color Values

Color values are 32-bit unsigned integer values. Colors are represented as RGB values with a red component, green component and blue component. Eight bits are used to store each color component, with the upper 8-bits of a color value being unused and stored as 0. This allows the expression of approximately 16 million distinct color values.

The underlying display hardware generally uses a smaller number of bits for each realizable color on the display. Typically, the hardware will represent colors as 16-bit values with 5-bits of red, 6-bits of green, and 5-bits of blue. This means that that it is possible in calls to library functions to specify more colors than can be represented by the hardware. When translating an external, 32-bit color value, to an internal, 16-bit, color value, the low order bits of the external color components are truncated, and the upper 8 bits are ignored. When translating an internal color value to an external color value, the low order bits are filled with zero.

The use of 32-bit RGB values for expressing color allows colors to be specified generically without regard to the specific hardware of the display device. The generic color is translated automatically to the nearest realizable color when used by the display.

The header files define symbols for some common colors, but any unsigned 32-bit integer value may be used. The following color constants are provided:

clrBlack		0x00000000			black

clrDkGray		0x00606060			dark gray

clrMedGray	0x00808080			medium gray

clrLtGray		0x00C0C0C0			light gray

clrWhite		0x00FFFFFF			white

clrRed		0x00FF0000			red

clrDkRed		0x00800000			dark red

clrGreen		0x0000FF00			green

clrDkGreen	0x00008000			dark green

clrBlue		0x000000FF			blue

clrDkBlue		0x00000080			dark blue

clrYellow		0x00FFFF00			yellow

clrDkYellow	0x00808000			dark yellow

clrCyan		0x0000FFFF			cyan

clrDkCyan		0x00008080			dark cyan

clrMagenta	0x00FF00FF			magenta

clrDkMagenta	0x00800080			dark magenta

clrLtBlueGray	0x00B8B8FF			light blue gray

clrMedBlueGray	0x007878FF			medium blue gray

### Foreground Color and Background Color

Foreground and background colors are used when drawing linear type graphical elements (lines, arcs, the perimeters of ellipses and rectangles) and text. When drawing one of these elements, pixels that are set (bits set to 1) in the pen or the character font correspond to the foreground, and are drawn with the foreground color. Pixels in the pen or font that are not set (bits set to 0) correspond to the background and are drawn using the background color.

The foreground and background colors are set using the setForeground() and setBackground() functions. The default foreground color is clrWhite and the default background color is clrBlack.

### Pen Pattern and Background Transparency

A pen pattern is an unsigned 16-bit integer specifying which pixels to draw when drawing a linear type graphics object. The pen is set using the setPen() function.

When drawing, '1' bits (bits that are set) in the pen correspond to foreground pixels and are drawn with the currently selected foreground color. Bits that are '0' (not set) correspond to background pixels. The drawing of background pixels depends on the current background transparency mode.

When background transparency is set off, background pixels are drawn using the currently selected background color. If background transparency is on, background pixels are not drawn. This allows the background pixels to “show through” what is being drawn. Background transparency is set using the setTransparency() function, specifying true or false to turn transparency on or off. Background transparency is off by default.

The pen pattern is advanced by one bit position for each pixel position in the graphical element being drawn. Once the complete pattern has been drawn, it repeats as more pixels are drawn. The pen pattern is reset to the beginning for each graphical element drawn. The pen pattern allows for various solid, dotted, dashed, and so on, patterns for drawing linear graphical elements.

Background transparency also applies when drawing text. When drawing text, the '1' pixels in the font are drawn with the foreground color, the '0' pixels are drawn with the background color or not drawn depending on the background transparency mode. Similarly to how linear object are drawn, if the background transparency on, the background will “show through” the text.

The header files define symbols for some common pen patterns, but any unsigned 16-bit value can be used for the pen:

penNull		0x0000

penSolid		0xFFFF

penDash		0xC3C3

penDot		0x9999

penDashDot	0xF18F

penDashDotDash	0xE667

penDashDotDot	0xC663

In addition to its use when drawing lines and text, background transparency is also used when drawing bitmap images using drawImage(). When transparency is enabled, pixels in the bitmap image being drawn whose color matches the current background transparency color are not drawn. Similarly to when drawing lines or text, this allows the background behind the bitmap image to “show through” the image. When transparency is turned off, all pixels in the bitmap image are drawn. The transparency color is set to clrBlack (0x00000000) by default.

### Drawing Intensity

An intensity can be specified that will be used when a bitmap image is drawn using drawImage(). The intensity is set using the setIntensity() function, and specifies a percentage value used to scale the brightness of each pixel drawn when rendering the bitmap image. The default value for intensity is 100, indicating that bitmap images are to be drawn at 100% intensity. The allowed range of values is 0 to 200. When rendering the bitmap image, the brightness of each pixel is scaled by the intensity value set. For example if intensity is set to 50 (meaning 50%), each pixel will be drawn at half the normal brightness.

### Touch Panel Operation

The display board provides a capacitive touch panel interface that supports two point multi-touch. The touch panel responds to and tracks finger touch and finger movement activity for up to two fingers simultaneously. The microcontroller on the display board monitors the touch activity and maintains a queue of touch events. The library function checkTouch() reads messages from this event queue and uses the information to maintain the current touch state.

Touch activity is obtained from the library by calling getFinger(), specifying the finger number about which to get touch information. The caller specifies the address of a MDFNG structure that will be filled in with the touch information. The MDFNG structure has the following definition:

struct MDFNG {

uint16_t	st;		// finger state

int16_t	xco;		// X coordinate of position

int16_t	yco;		// Y coordinate of position

};

Touch activity is tracked and reported by finger number. The first finger to touch the display is considered to be finger 0. Once finger 0 has started touching the display, it will be tracked and status about it will be recorded under finger 0. If, while finger 0 is on the display, a second touch is detected, this will be tracked as finger 1. Once a finger touch has been detected, it's identity (finger 0 or finger 1) is maintained as long as the touch is still active. Thus, if finger 0 is detected followed by finger 1, and then finger 0 leaves the display, touch activity for finger 1 is still tracked as finger 1. If finger 0 were to resume touching the display while finger 1 is still active, it will start being tracked as finger 0 again.

Touch activity reported via the st member of the MDFNG structure is assigned one of three states: FINGER_DOWN, FINGER_MOVE or FINGER_UP. The FINGER_DOWN state means that an initial touch has been detected. FINGER_MOVE means that a previously detected touch has moved to a different location without leaving the display. FINGER_UP means that the touch is no longer detected, i.e. that finger is no longer touching the display. For each of these states, the x,y pixel coordinate on the display is recorded and reported. For FINGER_DOWN and FINGER_MOVE the position of the finger is recorded. For FINGER_UP, the last known position on the display is recorded.

Information about the touch state on the display board is queried and updated by the checkTouch() function. This function needs to be called sufficiently often to “keep up” with the touch activity. If it isn't called often enough, touch activity will be lost and finger tracking will become erratic. Generally, this means that checkTouch() should be called at least every few milliseconds or tens of milliseconds. If the interval between successive calls to checkTouch() is on the order of 100ms or more, touch activity will almost certainly be lost.

### Buttons

A button is a user interface element provided by the MyDisp library. A button has one of two states: touched or not-touched. The appearance of a button is specified in one of two ways when it is created. The first method is to provide the names of two bitmap image files. The first image is used to draw the button in the not-touched, (up) state. The second image is used to draw the button in the touched, (down) state. The second method for specifying button appearance is to provide the name of a single bitmap image file. In this case, the specified image is used to draw the button in the not-touched (up) state. The same image is drawn at a reduced intensity to draw the button in the touched (down) state.

A button has a position and an active area on the display. The active area is a rectangle whose position is determined when it is created by specifying the x,y coordinate of the upper left corner. The size of the active area is determined by the size of the image bitmap specified for the not-touched state when the button is created. The function createButton() is used to create a button. If a button is no longer needed, it should be deleted by calling deleteButton() to release the resources it was using and to speed up processing by eliminating the need to track its activity.

The maximum number of buttons that the system supports at one time is specified by the symbol NUM_BUTTONS, defined in the library header file. The value of NUM_BUTTONS depends on the build environment and the target board. The library uses a static array to track buttons and to avoid using too much memory on systems with a small amount of RAM (e.g. Arduino Uno boards), the number of buttons supported is less on these systems than on systems having more RAM. Buttons are identified by a number, or ID, in the range 0 – NUM_BUTTONS-1. When referring to a button in any of the library functions, the button number, or ID is used.

A button can be “touched” by the user and the library will track the state of buttons being touched and record a button press event for the last button touched. A button press event occurs when a FINGER_DOWN is detected within the active area of the button and a FINGER_UP then occurs without the finger moving outside the button's active area. If a FINGER_DOWN is detected, but the finger subsequently moves outside the active area, the button stops being tracked for a potential press event.

Buttons can be enabled or disabled. When a button is disabled, it does not respond to touch activity and no press events will be recorded for it. An enabled button responds to touch activity and records a press event if it is pressed. Additionally, the drawButton() function will not draw disabled buttons. Buttons are disabled, by default, when they are created and must be enabled before they become active. The enableButton() function is used to enable or disable a button.

By having some buttons enabled and some buttons disabled, it is possible to build user interfaces that change dynamically by selectively enabling or disabling buttons in response to button press events or other user activity. This can also be accomplished by creating and deleting the buttons, but enabling and disabling is faster and the user interface can be more responsive.

The library provides functions for checking the current status of buttons. The function isEnabled() will return true if the specified button is enabled. The function isTouched() returns true if the button is enabled and finger 0 is currently within its active area. The function getButton() returns the button identifier for the last button pressed.

Buttons are not drawn automatically by the library. The function drawButton() is called by the user sketch to update the appearance of a button on the display. Only enabled buttons are drawn by drawButton(). If drawButton() is called to draw a button that is not enabled, nothing is drawn. When calling the drawButton() function, the sketch writer specifies to draw the button in the BUTTON_UP state or the BUTTON_DOWN state. The BUTTON_UP state is drawn using the first or “up” bitmap specified when the button was created. The BUTTON_DOWN state is drawn using the second or “down” bitmap. Alternatively, if only a single bitmap image was specified when creating the button, the “up” state is drawn using the specified bitmap image. The “down” state is drawn by drawing the same image bitmap at a reduced intensity.

The button image can be erased by specifying BUTTON_ERASE when calling drawButton(). When a button is enabled, the current content of the display in the active area of the button is saved into an off-screen memory area. When BUTTON_ERASE is specified, this off-screen data is used to redraw the active area of the button on the display. Note, that for this to work correctly, the background of the screen needs to have not changed since the button was enabled.

For BUTTON_ERASE to work correctly, it is necessary for the screen background to have not changed since the button was enabled. If the screen is going to be redrawn, the correct procedure is to disable all buttons, redraw the screen, and then to re-enable the buttons that need to be enabled. This will ensure that the buttons have the correct background information cached to allow BUTTON_ERASE to function correctly.

When a button is drawn using either BUTTON_UP or BUTTON_DOWN, the last drawn state is remembered, and subsequent calls to drawButton() with the same state are ignored. Normally, this will simplify and speed up the button drawing logic in a sketch. However, in some cases this will result in the button not being drawn when the sketch writer wants it to be. The enableButton() function clears the last drawn state information when the button is enabled, so enableButton() can be called to re-enable an already enabled button to cause the library to forget the last drawn state and force drawButton() to draw it again. The off-screen button background cache is not updated when enabling an already enabled button. The only effect of calling enableButton() on a button that is already enabled is to reset the last drawn state.

## Interface Function Definitions

The following section contains descriptions for all of the library functions.

### Setup and Control Functions

The following functions are used to initialize the library and display board and to control basic operation of the system.

##### MYDISP::begin()

Parameters:

None

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function initializes the library and establishes communications with the display board. It must be called before any other library functions can be used.

The display board can take several seconds to initialize after reset, and if the begin() function is called before the display is ready, begin() may fail. If begin() fails, it is generally a good idea to wait a few seconds and then try again. If begin() repeatedly fails, it is probably an indication that the display board is not connected or is malfunctioning.

The begin() function initializes the various drawing parameters as follows:

Foreground Color		- clrWhite

Background Color		- clrBlack

Pen Pattern			- penSolid

Background Transparency	- On

Font				- hfntConsole

Drawing Intensity		- 100%

This function will return true if it able to successfully communicate with the display board. It returns false if it is unable to communicate with the display board.

##### MYDISP::end()

Parameters:

None

Return Value:

None

Errors:

None

Description:

This function is used to terminate communications with the display board and use of the MyDisp library. After this function has been called, it is necessary to call begin() once again before again using any MyDisp library functions.

##### MYDISP::checkTouch()

Parameters:

None

Return Value:

None

Errors:

None

Description:

This function manages all of the background activity for monitoring and processing touch activity on the display board and processing button functions. This function must be called regularly for touch activity and button activity to be handled correctly.

A call to this function should generally be placed at the top of the loop() function in the sketch. This will ensure that it is called each time the loop function is entered. If there are any places in the sketch code where lengthy delays occur, calls to this function should be placed inside those areas.

Ideally, this function should be called every few milliseconds. If the interval between calls to checkTouch() is too long, touch events may be lost and button behavior may become erratic. See the description of Touch Panel Operation above for more information about the touch system and use of checkTouch().

##### MYDISP::displayOn(fEn)

Parameters:

fEn		- Enable (true), or disable (false) the display

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function turns the display on or off. When the display is off, the backlight power supply is turned off and the backlight powered down.

This function will only fail if there is a communications error with the display board.

##### MYDISP::clearDisplay(clr)

Parameters:

clr		- Color to use to clear the display

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will clear the display by filling it with the specified color.

This function will only fail if there is a communications error with the display board.

##### MYDISP::setBacklight(val)

Parameters:

val		- Display backlight brightness to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function sets the display backlight brightness to the specified percentage value. The allowed range of values is 0-100. When the brightness is set to 0, the display backlight power supply is turned off.

This function will fail if the specified brightness value is out of range or a communications error occurs with the display board.

##### MYDISP::displayHds()

Parameters:

None

Return Value:

Returns the handle to the drawing state object being used by the library

Errors:

Returns 0 if it fails.

Description:

This function is normally not used. It is provided for the cases where the sketch writer wants to call functions in the underlying MTDS library and needs access to the drawing state handle being used by the MyDisp library for drawing on the display.

##### MYDISP::setPen(pen)

Parameters:

pen		- Pen pattern to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This sets the pen pattern to the specified value. The pen pattern is used to draw linear elements. This includes, lines, the borders of rectangles and the borders of ellipses. See the description above for more information about pens.

This function will only fail if there is a communications error with the display board.

##### MYDISP::setForeground(clr)

Parameters:

clr		- Foreground color to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

Sets the current foreground color to use. The foreground color is used when drawing linear type elements or text. See the sections on Color Values and Foreground and Background Color above for more information about colors.

This function will only fail if there is a communications error with the display board.

##### MYDISP::setBackground(clr)

Parameters:

clr		- Background color to set.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

Sets the current background color to use. The background color is used when drawing linear type elements or text. See the sections on Color Values and Foreground and Background Colors above for more information about colors.

This function will only fail if there is a communications error with the display board.

##### MYDISP::setTransparency(fTrans)

Parameters:

fTrans		- Enable (true) or disable (false) background transparency mode.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function enables or disables background transparency mode. Background transparency is used when drawing objects that use a pen or when drawing text. When background transparency is enabled, pixels that correspond to the background in the pen or current text font are not drawn. This allows the background that is being drawn over to “show through” the element being drawn.

Background transparency is also used by drawBitmap() when drawing a bitmap image. In this case, when transparency is enabled, pixels in the bitmap image being drawn whose color matches the background transparency color are not drawn.

This function will only fail if there is a communications error with the display board.

##### MYDISP::setTransparency(fTrans, clr)

Parameters:

fTrans		- Enable (true) or disable (false) background transparency mode.

clr		- Transparency color to set

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This variant of the setTransparency() function allows for turning transparency on or off and additionally for setting the transparency color. The transparency color is the color treated by drawImage() as transparent. Pixels that match the transparency color are not drawn when transparency is enabled. The transparency color is clrBlack by default.

##### MYDISP::setIntensity(ity)

Parameters:

ity		- Bitmap drawing intensity to set.

Return value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function sets the intensity value to be used when drawing bitmap images using the drawImage() function. The intensity value is specified as an integer between 0 and 200, giving a percentage from 0% to 200%. When drawing an image, the intensity value is used to scale the brightness of each pixel of the image. For example, setting intensity to 50, will cause the pixels of the image to be drawn at half the normal brightness.

##### MYDSIP::setFont(idFont)

Parameters:

idFont		- The identifier for the font to be selected

Return Value:

None

Errors:

Return true if successful, or false if it fails.

Description:

This sets the specified font as the current font to be used when drawing text. See the section above on Fonts for more information about fonts. In version 1 of the MTDS display board firmware, the only font supported is hfntConsole.

This function will fail if an invalid font identifier if specified or a communication error occurs with the display board.

### Graphics Rendering Functions

The following functions draw graphical elements on the display.

##### MYDISP::setPixel(x, y, clr)

Parameters:

x		- X coordinate of pixel to set.

y		- Y coordinate of pixel to set.

clr		- Color value for the pixel.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

Set the pixel at the specified display coordinate to the specified color value. All drawing is clipped to the bounds of the display. Any coordinate values can be specified for x and y, but any coordinate outside the bounds of the display will result in nothing being drawn.

This function will only fail if there is a communications error with the display board.

##### MYDISP::getPixel(x, y)

Parameters:

x		- X coordinate of the pixel

y		- Y coordinate of the pixel.

Return Value:

Returns the color of the pixel at the specified location.

Errors:

Returns 0 if an error occurs.

Description:

Return the color of the pixel at the specified display coordinate. All drawing is clipped to the bounds of the display. Any coordinate values can be used for x and y. The color value clrBlack is returned for any pixel outside the bounds of the display.

This function will only fail if there is a communications error with the display board. In this case, the color clrBlack is returned.

##### MYDISP::drawLine(x1, y1, x1, y2)

Parameters:

x1, y1		- Coordinate of first endpoint of the line

x2, y2		- Coordinate of the second endpoint of the line

Return Value:

None

Errors:

Return true if successful, or false if it fails.

Description:

This function draws a line from the first specified endpoint to the second endpoint. The line is drawn using the current pen pattern, foreground color, background color and background transparency mode.

This function will only fail if there is a communications error with the display board.

##### MYDISP::drawArc(x1, y1, x2, y2, xr1, yr1, xr2, yr2)

Parameters:

x1, y1		- Upper left corner of bounding rectangle.

x2, y2		- Lower right corner of bounding rectangle.

xr1, yr1	- Endpoint of starting radial line

xr2, yr2	- Endpoint of ending radial line

Return Value:

None

Errors:

Return true if successful, or false if it fails.

Description:

This function draws an elliptical arc. The arc will be a segment of the ellipse specified by x1,y1 and x2,y2. These two points specify the upper left corner and lower right corner of a rectangle, the ellipse being tangent to this rectangle. The arc segment is drawn from the beginning angle clockwise to the ending angle. The beginning angle is specified by xr1,yr1 which is the endpoint of a radial line from the center of the ellipse. The ending angle is specified by xr2,yr2 which is the endpoint of a second radial line from the center of the ellipse. The arc is drawn clockwise starting at the intersection of the ellipse and radial line 1 and ending at the intersection of the ellipse and radial line 2.

The arc is drawn using the current pen, foreground color, background color and background transparency mode. All drawing is clipped to the bounds of the display. If the set of parameters specified results in all pixels being outside the bounds of the display, nothing will be drawn.

This function will only fail if there is a communications error with the display board.

##### MYDISP::drawRectangle(fFill, x1, y1, x2, y2)

Parameters:

fFill		- True to fill the rectangle, false to only draw the perimeter.

x1, y1		- Coordinate of upper left corner.

x2,y2		- Coordinate of lower right corner.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw the perimeter of and optionally fill the interior of a rectangle. The rectangle is specified by its upper left and lower right corners. The perimeter is drawn clockwise starting at the upper left corner. The perimeter is drawn using the current pen, foreground color, background color and background transparency mode.

If fFill is true, the interior area of the rectangle will be filled with the current foreground color. If it is desired to suppress drawing the perimeter of the rectangle, set the pen to penNull before calling this function.

This function will only fail if there is a communications error with the display board.

##### MYDISP::drawEllipse(fFill, x1, y1, x2, y2)

Parameters:

fFill		- True to fill the ellipse, false to only draw the perimeter.

x1,y1		- Coordinate of upper left corner of bounding rectangle.

x2,y2		- Coordinate of lower right corner of bounding rectangle.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw the perimeter of and optionally fill the interior of an ellipse. The ellipse is specified by the upper left and lower right corners of a bounding rectangle. The ellipse is drawn tangent to this rectangle. The perimeter is drawn clockwise starting at the upper center of the ellipse and is drawn using the current pen, foreground color, background color and background transparency mode.

If fFill is true, the interior of the ellipse will be filled with the current foreground color. If it is desired to suppress drawing the perimeter of the ellipse, set the pen to penNull before calling this function.

This function will only fail if there is a communications error with the display board.

##### MYDISP::drawImage(name, x, y)

Parameters:

name		- Character string giving the file name of the bitmap image file

x,y		- Coordinate for where to draw the bitmap.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will read a bitmap image from a file on the SD card and draw it on the display at the specified location. The location given in x,y indicates the location for the upper left corner of the bitmap.

This function will fail if the file doesn't exist, or if the bitmap is in an unrecognized format or too large to load into memory. It can also fail if there is a communications error with the display board.

##### MYDISP::drawImage(hbmp, x, y)

Parameters:

hbmp		- handle to a previously loaded bitmap image

x,y		- Coordinate for where to draw the bitmap.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw a bitmap image stored in memory onto the display at the specified location. The location given in x,y indicates the location for the upper left corner of where the bitmap is to be drawn.

The hbmp parameter specifies the handle of a memory bitmap. This will normally be obtained as the return value of the loadBitmap() function.

This function will fail if the specified hbmp is invalid, or if there is a communications error with the display board.

##### MYDISP::drawText(txt, x, y)

Parameters:

txt		- The text string to print.

x,y		- Coordinate for the origin of the text.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function will draw the given text string at the specified location. The location specifies the upper left corner of the text to be drawn. The string is a standard “C”, '\0' terminated array of characters. The text is drawn using the current font, foreground color, background color and background transparency mode.

This function can fail if the specified string is too long or if a communications error occurs with the display board.

##### MYDISP::loadImage(name)

Parameters:

name		- file name string specifying a bitmap image file

Return Value:

Returns the handle to the in-memory bitmap containing the contents of the loaded bitmap file.

Errors:

Returns 0 if the function fails.

Description:

This function will load the contents of the specified bitmap image file into memory. It returns the resulting memory bitmap handle, which can then be used to refer to the bitmap at a later time. In particular, this bitmap handle can be used as the parameter to a call to drawBitmap() to cause the bitmap to be drawn on the display.

If a given bitmap image is going to be drawn multiple times, the drawing performance can be improved by loading it into memory first and then drawing it as needed.

When the bitmap is no longer needed, the memory it occupies should be reclaimed by calling deleteImage().

##### MYDISP::deleteImage(hbmp)

Parameters:

hbmp		- the handle to the memory bitmap to delete.

Return value:

none

Errors:

Returns true if successful, or false if an error occurs.

Description:

This function is used to delete from memory a bitmap previously loaded using loadImage(). This releases the memory on the display device being used to hold the in-memory copy of the bitmap image.

### User Interface Functions

The following function can be used to build touch based user interfaces on the display.

##### MYDISP::createButton(id, img1, img2, x, y)

Parameters:

id		- Identifier to assign to the button

img1		- Name of the bitmap file for the “up” image for the button.

img2		- Name of the bitmap file for the “down” image for the button.

x,y		- Coordinate for the upper left corner of the button.

Return Value:

Returns true if successful, or false if it fails.

Description:

This function creates a button and initializes the internal data used to track its behavior. The identifier is a number in the range 0 – NUM_BUTTONS. The two image file names specify the names of files containing the bitmaps used for drawing the button. The “up” image is used to draw the button in the “up” (i.e. not touched) state and the “down” image is used to draw the button in the “down” (i.e. touched) state. The coordinate x,y specifies the upper left corner for the button rectangle. The size of the button rectangle is taken from the dimensions of the first bitmap image. The second bitmap image doesn't have to be the same size as the first one, but drawing may look strange if they aren't the same size.

The two image files are loaded into memory on the display board for later use by drawButton().

A button is disabled when it is first created. It must be enabled using enableButton() before it can be used.

Nothing is drawn as a result of calling createButton(). Button drawing is done using the function drawButton().

Once the button is no longer needed, it should be deleted using deleteButton() to release the resources used by the button.

See the section above on Buttons for more detailed information about the behavior and use of buttons.

This function can fail if the specified id is already in use, an error occurs loading either of the image bitmaps, or a communications error occurs with the display board.

##### MYDISP::createButton(id, img, x, y)

Parameters:

id		- Identifier to assign to the button

img		- Name of the bitmap file for drawing the button.

x,y		- Coordinate for the upper left corner of the button.

Return Value:

Returns true if successful, or false if it fails.

Description:

This function creates a button and initializes the internal data used to track its behavior. It is identical to the createButton() function described above, except that only a single bitmap image file name is specified. When this variant of createBitmap() is used, button drawing behavior is different. In this case, the specified image file is drawn at full intensity to draw the button in the “up” or un-touched state. The same image is used to draw the button in the “down” or touched state, except that it is drawn at reduced intensity.

See the previous description of createButton() or the section above on Buttons for more detailed information about the behavior and use of buttons.

This function can fail if the specified id is already in use, an error occurs loading either of the image bitmaps, or a communications error occurs with the display board.

##### MYDISP::deleteButton(id)

Parameters:

id		- Identifier of the button to delete.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function deletes the specified button and frees all resources used. After calling this function, the identifier can be used when creating another button.

This function can fail if the specified button identifier is invalid or isn't in use. It can also fail if there is a communications error with the display board.

##### MYDISP::getButton()

Parameters:

None

Return Value:

Returns button identifier of last button pressed or -1 if no button pressed.

Errors:

None

Description:

This function returns the button identifier of the last button pressed since the last time it was called. There is no queue of button press events, so if more than one button press occurred since the last time this function was called, only the last one is returned.

A button press is defined as the first finger going down and then up within the bounding rectangle of a button without moving outside the bounding rectangle between the down event and the up event.

##### MYDISP::enableButton(id, fEn)

Parameters:

id		- Button identifier.

fEn		- True to enable or false to disable the button.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function is used enable or disable a button. A button that is disabled doesn't respond to touch activity and will not be drawn by drawButton(). In order for a button to recognize touch activity or be drawn by drawButton(), it must be enabled. See the section on Buttons above for more detailed information about buttons and their behavior and use.

This function will fail if the identifier isn't valid or the button hasn't been created.

##### MYDISP::drawButton(id, st)

Parameters:

id		- Button identifier

st		- Button state to draw

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function draws the specified button on the display. The drawing state can be BUTTON_UP, BUTTON_DOWN, or BUTTON_ERASE. When BUTTON_UP is specified, the first, or “up”, bitmap specified when the button was created is drawn at the button location. When BUTTON_DOWN is specified the second, or “down” bitmap is drawn. If the button was created using the variant of the createButton() function where only a single bitmap file name is specified, the button will be drawn using the specified bitmap image when BUTTON_UP is specified. The same bitmap image, but at reduced intensity will be drawn when BUTTON_DOWN is specified.

When a button is enabled, the area of the screen corresponding to the button rectangle is saved to an off-screen memory location. When BUTTON_ERASE is specified, the button rectangle is erased by drawing this saved screen region back to the screen. Note, that if the background of the screen has been changed since the button was enabled, erasing the button won't look correct because it will draw an out-of-date background to the screen.

The button remembers when it has been drawn in the BUTTON_UP or BUTTON_DOWN states and subsequent calls to drawButton() to draw the same state will be ignored.

This function will fail if the button identifier is invalid or the button hasn't been created, if the button drawing state specified is invalid, or a communications error occurs with the display board.

##### MYDISP::isEnabled(id)

Parameters:

id		- Button identifier

Return Value:

Returns true if the button is enabled.

Errors:

Returns false if an error occurs.

Description:

This function returns the enable state of the specified button. It returns true if the button is enabled. A return value of false means either that the button is disabled, or the identifier specified is invalid or hasn't been created.

##### MYDISP::isTouched(id)

Parameters:

id		- Button identifier

Returns Value:

Returns true if the specified button is being touched.

Errors:

Returns false if an error occurs.

Description:

This function returns true if the specified button is being touched. A button being touched means that a finger went down within the bounding rectangle of the button and is still on the button without having moved outside the bounding rectangle of the button.

This function returns false if the specified button is not being touched, or if the button identifier is invalid or the button hasn't been created.

##### MYDISP::getFinger(num, fng)

Parameters:

num		- Finger number (0 or 1)

fng		- Pointer to structure to receive touch data for the finger.

Return Value:

None

Errors:

Returns true if successful, or false if it fails.

Description:

This function provides the last finger activity and the x,y coordinates of the last finger position for the specified finger. See the section on Touch Panel Operation above for more detailed information about the processing touch activity.

This function will fail if the specified finger number is invalid.