Analysis of plane frames with arbitrary cross-sections

Joint coordinates - xJ; yJ

xJ=0 m 0 m 8 m 16 m 16 m , yJ=0 m 8 m 10 m 8 m 0 m

nJ=lenxJ=5

Elements - [J1; J2]

transpeJ=12342345 ,  nE=nrowseJ=4

Element endpoint coordinates

x1e=xJ.eJe,1 , y1e=yJ.eJe,1 ,  x2e=xJ.eJe,2 , y2e=yJ.eJe,2

Element length -  le=x2e-x1e2+y2e-y1e2

Element direction

ce=x2e-x1ele , se=y2e-y1ele

Transformation matrix

Diagonal 3x3 block - te=ce;se;0 | -se;ce;0 | 0;0;1

Generation of the full transformation matrix

Te=addte;addte;matrix6;6;1;1;4;4

Supports - [Joint; cx; cy; cr]

c=11020 kN/ m1020 kN/ m0 kNm51020 kN/ m1020 kN/ m1020 kNm

nc=nrowsc=2

Loads - [Element, qx, qy]

q=110 kN/ m0 kN/ m20 kN/ m-20 kN/ m30 kN/ m-10 kN/ m

Load values on elements

qx=10 kN/ m 0 kN/ m 0 kN/ m 0 kN/ m

qy=0 kN/ m -20 kN/ m -10 kN/ m 0 kN/ m

Scheme of the structure

Materials

Modules of elasticity - E=45;35· GPa=45 GPa 35 GPa

Poisson coefficients - ν=0.2;0.2=0.2 0.2

Shear modules - G=E2·1+ν=18.75 GPa 14.58 GPa

Assignments on elements - eM=1;2;2;1=1 2 2 1

Cross-sections

Section S1 - h1=500 mm - circular -  bCz=2·h122-z-h122

Section S2 - b2=250 mm , h2=700 mm - rectangular - bRz=b2

General representation - bz;s=takes;bCz;bRz

Cross section properties

Equations

Area - As=0 mmhsbz;s dz

First moment of area - Ss=0 mmhsbz;s·z dz

Centroid - zcs=SsAs

Second moment of area - Is=0 mmhsbz;s·z-zcs2 dz

First moment of area below z - S1z;s=0 mmzbζ;s·zcs-ζ dζ

Shear area - Ass=Is20 mmhsS1z;s2bz;s dz

Calculated results

Centroids - zc=zc1;zc2=250 mm 350 mm

Areas - A=A1;A2=196350 mm2 175000 mm2

Shear areas - As=As1;As2=176715 mm2 145833 mm2

Second moments of area - I=I1;I2=3067961576 mm4 7145833333 mm4

Assignments on elements - eS=1;2;2;1=1 2 2 1

Element stiffness matrix

Elastic properties for element "e"

EAe=EeM.e·AeS.e ,  EIe=EeM.e·IeS.e ,  GAse=GeM.e·As.eS.e

kse=12·EIeGAse·le2 ,  αe=EAele , βe=EIele3·1+kse

Stiffness matrix coefficients for element "e"

k11e=αe·mkN , k22e=12·βe·mkN , k23e=6·βe·le·1kN

k33e=4+kse·βe·le2·1kNm , k36e=2-kse·βe·le2·1kNm

Assembling the 3x3 stiffness matrix blocks for element "e"

kiie=k11e | 0;k22e;k23e | 0;k23e;k33e

kije=-k11e | 0;-k22e;k23e | 0;-k23e;k36e

kjie=transpkije

kjje=k11e | 0;k22e;-k23e | 0;-k23e;k33e

Full element stiffness matrix

kEe=stackaugmentkiie;kije;augmentkjie;kjje

Stiffness matrices obtained in local coordinates

kE1=110446600-11044660003210.6612842.60-3210.6612842.6012842.668627.80-12842.634113.2-1104466001104466000-3210.66-12842.603210.66-12842.6012842.634113.20-12842.668627.8

kE2=74276500-7427650005243.4621619.30-5243.4621619.3021619.31194680-21619.358809.3-74276500742765000-5243.46-21619.305243.46-21619.3021619.358809.30-21619.3119468

Stiffness matrices obtained in global coordinates

transpT1·kE1·T1=3210.660-12842.6-3210.660-12842.60110446600-11044660-12842.6068627.812842.6034113.2-3210.66012842.63210.66012842.60-11044660011044660-12842.6034113.212842.6068627.8

transpT2·kE2·T2=699382173535-5243.46-699382-173535-5243.4617353548627.120973.8-173535-48627.120973.8-5243.4620973.81194685243.46-20973.858809.3-699382-1735355243.466993821735355243.46-173535-48627.1-20973.817353548627.1-20973.8-5243.4620973.858809.35243.46-20973.8119468

Global stiffness matrix

K=10200-12842.6-3210.660-12842.60000000000102000-11044660000000000-12842.6068627.812842.6034113.2000000000-3210.66012842.67025921735357599.17-699382-173535-5243.460000000-11044660173535115309320973.8-173535-48627.120973.8000000-12842.6034113.27599.1720973.81880965243.46-20973.858809.3000000000-699382-1735355243.461398763010486.9-6993821735355243.46000000-173535-48627.1-20973.8097254.20173535-48627.120973.8000000-5243.4620973.858809.310486.90238937-5243.46-20973.858809.3000000000-699382173535-5243.46702592-1735357599.17-3210.66012842.6000000173535-48627.1-20973.8-1735351153093-20973.80-110446600000005243.4620973.858809.37599.17-20973.8188096-12842.6034113.2000000000-3210.660-12842.610200-12842.60000000000-1104466001020000000000012842.6034113.2-12842.601020

Element load vector

Lateral load in local CS - qEe=-qx.e·se+qy.e·ce

Axial load in local CS - nEe=qx.e·ce+qy.e·se

Equivalent loads at element endpoints

FExe=qx.e·le2·1kN , FEye=qy.e·le2·1kN  , MEe=qEe·le212·1kNm

Load vector - FEe=FExe;FEye;MEe;FExe;FEye;-MEe

Results for elements

Element Е 1  - FE1=40 0 -53.33 40 0 53.33

Element Е 2  - FE2=0 -82.46 -109.95 0 -82.46 109.95

Element Е 3  - FE3=0 -41.23 -54.97 0 -41.23 54.97

Element Е 4  - FE4=0 0 0 0 0 0

Global load vector

F=40 0 -53.33 40 -82.46 -56.62 0 -123.69 54.97 0 -41.23 54.97 0 0 0

Results

Solution of the system of equations by Cholesky decomposition

Z=clsolveK;F=0 0 -0.000928 0.00809 -0.000126 -0.00274 0.0119 -0.0157 0.000699 0.0157 -9.84×10-5 0.000846 0 0 0

Joint displacements

zJj=sliceZ;3·j-2;3·j

zj=roundzJjδz·δz·1000·mm;mm;1

z1=0 mm 0 mm -0.928

z2=8.09 mm -0.126 mm -2.74

z3=11.88 mm -15.67 mm 0.699

z4=15.67 mm -0.0984 mm 0.846

z5=0 mm 0 mm 0

Support reactions

ri=rowc;i , ji=take1;ri

Ri=-zJji·m;m;1·lastri;3

Joint J1 - R1=-18.84 kN 138.69 kN 0 kNm

Joint J5 - R2=-61.16 kN 108.7 kN 230.05 kNm

Element end forces

zEe=zJeJe,1;zJeJe,2

REe=colkEe·Te·zEe-Te·FEe;1·1;1;m;1;1;m·kN

RE1=138.69 kN 18.84 kN -1.42×10-14 kNm -138.69 kN 61.16 kN -169.29 kNm

RE2=92.97 kN 119.71 kN 169.29 kNm -52.97 kN 40.29 kN 158.18 kNm

RE3=65.7 kN -10.62 kN -158.18 kNm -85.7 kN 90.62 kN -259.24 kNm

RE4=108.7 kN 61.16 kN 259.24 kNm -108.7 kN -61.16 kN 230.05 kNm

Element internal forces

Ne;x=-take1;REe-nEe·x ,  Qe;x=take2;REe+qEe·x

Me;x=-take3;REe+take2;REe·x+qEe·x22

Axial forces diagram, kN

Shear forces diagram, kN

Bending moments diagram, kNm

Deformed shape

Shape function in relative coordinates ξ = x/l (with account to shear deflections)

Φ1e;ξ=11+kse·1+kse-kse·ξ-3·ξ2+2·ξ3

Φ2e;ξ=ξ·le·m-11+kse·1+kse2-2+kse2·ξ+ξ2

Φ3e;ξ=ξ1+kse·kse+3·ξ-2·ξ2

Φ4e;ξ=ξ·le·m-11+kse·-kse2-1-kse2·ξ+ξ2

Element endpoint displacements and rotations

zE,loce=Te·zEe

u1e=take1;zE,loce , v1e=take2;zE,loce , φ1e=take3;zE,loce

u2e=take4;zE,loce , v2e=take5;zE,loce , φ2e=take6;zE,loce

Displacement functions

ue;ξ=u1e·1-ξ+u2e·ξ+nEe· mEAe·ξ·1-ξ

ve;ξ=v1e·Φ1e;ξ+φ1e·Φ2e;ξ+v2e·Φ3e;ξ+φ2e·Φ4e;ξ+qEe·le424·EIe·ξ2·1-ξ2 m+qEe·le22·GAse·ξ·1-ξ m

Deformed shape, mm